***Engineer***

1. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above.

2. The ratio of tangential and normal components of a stress on an inclined plane through θ° to the direction of the force, is :

a. cos θ

b. tan θ

c. cos θ

d. sec θ

3. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

4. If a steel rod of 20 mm diameter and 5 metres long elongates by 2.275 mm when subjected to an axial pull of 3000 kg, the stress developed, is

a. 9.5541 kg/cm2

b. 95.541 kg/cm2

c. 955.41 kg/cm2

d. 9554.1 kg/cm2.

5. If a constant section beam is subjected to a uniform bending moment throughout, its length bends to

a. a circular arc

b. a parabolic arc

c. a catenary

d. none of these.

6. Pick up the correct statement from the following :

a. the lines of sight while observing back sight and fore  sight lie in the same horizontal plane

b. the   staff readings are measurements made vertically downwards from a horizontal plane

c. the horizontal plane with reference to which staff readings are taken, coincides with the level surface through the telescope axis

d. all the above.

7. The radius of curvature of the arc of the bubble tube is generally kept

a. 10 m

b. 25 m

c. 50 m

d. 100 m

8. The sag of 50 m tape weighing 4 kg under 5 kg tension is roughly

a. 0.053 m

b. 0.063 m

c. 0.073 m

d. 0.083 m

9. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

10. While working on a plane table, the correct rule is :

a. Draw continuous lines from all instrument stations

b. Draw short rays sufficient to contain the points sought

c. Intersection should be obtained by actually drawing second rays

d. Take maximum number of sights as possible from each station to distant objects.

11. The construction of optical square is based, on the principle of optical

a. reflection

b. refraction

c. double refraction

d. double reflection.

12. Magnetic declination at any place

a. remains constant

b. does not remain constant

c. fluctuates

d. changes abruptly.

13. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

14. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

15. Expanded metal is

a. manufactured from steel sheets

b. used for reinforced concrete in road pavements

c. measured in term of SWM (shortway mesh) and LWM (long way mesh)

d. all the above.

16. Commonly used thinner in

a. cellulose paints is ethyle acetate

b. oil paints, is naptha

c. distemper, is water

d. all the above.

17. Minimum of 40% of iron, is available in

a. Red haemetite

b. Limonite

c. Siderite

d. Black band.

18. The hardest rock is

a. marble

b. diamond

c. talc

d. quartz.

19. Pick up the correct statement from the following:

a. Insufficient quantity of water makes the concrete unworkable

b. Excess quantity of water makes the concrete segregated

c. Excess quantity of water causes bleeding in concrete

d. All the above.

20. The main object of compaction of concrete, is:

a. to eliminate air holes

b. to achieve maximum density

c. to provide initimate contact between the concrete and embedded materials

d. all the above.

21. Pick up the correct statement from the following:

a. The concrete cured at a temperature below 23°C, gains strength up to 28 days

b. The concrete does not set at freezing point

c. The strength of concrete increases with its age

d. All the above.

22. Workability of concrete for a given water content is good if the aggregates, are

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregates.

23. The type of aggregates not suitable for high strength concrete and for pavements subjected to tension, is

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregate

24. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

25. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the void ratio, is

a. 0.858

b. 0.860

c. 0.862

d. 0.864

26. A failure wedge develops if a retaining wall

a. moves away from the backfill

b. moves towards the backfill

c. sinks downwards

d. stresses equally by vertical and horizontal forces.

27. Number of piles required to support a column, is

a. 1

b. 2

c. 3

d. 4

28. The seepage exit gradient in a soil is the ratio of

a. flow line to slope

b. head upstream to that at downstream

c. head loss to the length of the seepage

d. none of these.

29. For a base failure of a slope, depth factor

a. Df = 1

b. Df < 1

c. Df > 1

d. none of these.

30. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

31. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

32. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

33. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

34. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

35. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

36. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

37. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

38. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

39. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

40. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

41. The angle between isometric axes is

a. 30°

b. 60°

c. 90°

d. 120°

42. Topographic map is used to represent the

a. artificial detail

b. natural detail

c. construction detail

d. all of the above

43. The best drawing paper used for water colour is :

a. hand made paper (hand made drawing sheet)

b. tracing drawing sheet

c. ordinary drawing sheet

d. ammonia paper

44. The set of technical drawings used during the manufacturing phase of product including cvili construction, are known as

a. structural drawing

b. as built drawing

c. structural drawing

d. working drawing

45. Pick up the element of the cost from the following:

a. direct material

b. direct labour

c. Over head

d. All of these

46. Present worth Annuity (PWA) is generally known as

a. Premium annuities

b. Income annuities

c. Future annuities

d. All of these

47. Keeping in view, the feasibility order of magnitude, the preliminary, conceptual or budget estimates, are prepared by :

a. construction manager

b. owner himself/herself

c. construction manager

d. None of these

48. Professional code of conduct issued by Nepal Engineering Council has

a. 6 articles

b. 7 articles

c. 8 articles

d. 9 articles

49. The number of countries in which Nepal Engineering Council has recognized the academic institutions are (September, 2013).

a. 20

b. 30

c. 40

d. 50

50. The Engineering Council members consist of the following members now more

a. 12

b. 14

c. 16

d. 18