***Engineer***

1. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7

2. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

3. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

4. A solid cube is subjected to equal normal forces on all its faces. The volumetric strain will be x-times the linear strain in any of the three axes when

a. x = 1

b. x = 2

c. x = 3

d. x = 4

5. The shape of the bending moment diagram over the length of a beam, carrying a uniformly increasing load, is always

a. linear

b. parabolic

c. cubical

d. circular.

6. The minimum range for sliding the focusing lens in the internal focusing telescope for focusing at all distances beyond 4 m is

a. 5 mm

b. 10 mm

c. 15 mm

d. 20 mm.

7. Flint glass

a. has slightly the greater refracting power than crown glass

b. has roughly double refracting power than that of crown

c. and crown glass proportions yield the required focal length and neutralise the dispersion produced by the convex lens at the emergence from the concave

d. all the above.

8. The 10 mm markings on a levelling staff placed at 20 m are separated by

a.  radian

b.  radian

c.  radian

d.  radian.

9. The area of any irregular figure of the plotted map is measured with

a. sextant

b. clinometer

c. planimeter

d. optical square

10. Pick up the correct statement from the following :

a. the handles are on swivel joints to prevent twisting of the chain

b. the fifth tag from either end of the chain is numbered 5

c. the length of the Gunter's chain is 66 ft.

d. all the above.

11. The staff reading at a distance of 80 m from a level with the bubble at its centre is 1.31 m. When the bubble is moved by 5 divisions out of the centre, the reading is 1.39 m. The angular value of the one division of the bubble, is

a. 41.25 secs

b. 14.52 secs

c. 25.05

d. none of these

12. During secular variation of magnetic meridian at different places

a. range of oscillations is constant

b. period of oscillation is constant

c. range and period of oscillation both vary

d. period of oscillation only varies.

13. The compound of Portland cement which contributes to the strength after two to three years is

a. Tricalcium silicate

b. Di-calcium silicate

c. Tricalcium aluminate

d. Tetracalcium alumino ferrite.

14. Pick up the correct statement from the following:

a. The bulking of fine sand is more than that of coarse sand

b. If the percentage content of moisture exceeds 10%, increase in bulk of sand starts increasing

c. The volume of fully saturated sand equals that of dry sand

d. All the above.

15. For melting one tonne of cast iron

a. 700 m3 air is required

b. 20 kg limestone is required

c. one quintal coke is required

d. all the above.

16. Chlorite, a green colour mineral is mainly derived from the decomposition of

a. augite

b. biotite

c. horn blende

d. All of these.

17. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

18. Sand stone is

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. volcanic rock.

19. To hydrate 500kg of cement full water needed, is

a. 110 kg

b. 120 kg

c. 130 kg

d. 140 kg

20. The light weight aggregates are obtained from

a. sedimentary rocks

b. metamorphic rocks

c. igneous rocks

d. volcanic source.

21. Joints in concrete structures, are provided

a. to reduce the tensile stresses likely to be developed due to evaporation of water

b. to minimise the change in the dimensions of the slab

c. to minimise the necessary cracking

d. all the above.

22. Shrinkage in concrete can be reduced by using

a. less cement in the concrete

b. proper concrete mix

c. presaturated aggregates

d. all the above.

23. If the effective working time is 7 hours and per batch time of concrete is 3 minutes, the output of a concrete mixer of 150 litre capacity, is

a. 15, 900 litres

b. 16, 900 litres

c. 17, 900 litres

d. 18, 900 litres

24. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

25. If drainage is permitted throughout the test, during the application of both normal, and shear stresses so that full consolidation occurs and no excess pore pressure is set up at any stage of the test, is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

26. The critical exist gradient of seepage water in soils, increases with

a. a decrease in specific gravity

b. a decrease in void ratio

c. both (a) and (c)

d. none of these.

27. The active earth pressure of a soil is proportional to (where φ is the angle of friction of the soil)

a. tan (45° - φ)

b. tan2 (45° + φ/2)

c. tan2 (45° - φ/2)

d. tan (45° + φ)

28. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

29. A soil not fully consolidated under the existing over-burden pressure, is called

a. pre-consolidated

b. normally consolidated

c. under-consolidated

d. none of these.

30. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

31. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

32. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

33. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

34. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

35. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

36. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

37. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

38. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

39. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

40. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

41. The drawing for municipality is done at scale

a. 1" = 8"

b. 1:50

c. 1:100

d. both (a) and (c)

42. Ground plane is

a. horizontal plane

b. vertical plane

c. inclined plane

d. none of the above

43. ”Title Block” is placed at bottom right hand corner of the drawing which gives information about

a. title of drawing

b. scale of the drawing

c. serial no of drawing

d. all of the above

44. The most important material for drawing is

a. pencil and eraser

b. eraser & paper

c. paper & pencil

d. none of the above

45. Each financial ratio is generally compared by

a. a past ratio calculated from the past financial standard of the firm.

b. a ratio developed by using the projected financial statement of the firm.

c. a ratio of some selected firms most progressive and successful at the point of consideration.

d. All of these

46. The wages of supervisors and material handlers are charged as :

a. Over head

b. direct labour cost

c. indirect labour cost

d. None of these

47. Pick up the correct method adopted for developing the approximate or conceptual estimates from the following :

a. Cost per function method

b. Cost per square metre

c. Cost per linear unit

d. All of these

48. Which of the following is not concerned with the authority of Engineers?

a. good behaviour with client .

b. conscious with his profession

c. divulge the confidential matter

d. express an opinion on engincering matter

49. For the registration of general engineer tha amount of fee required is:

a. NRS 2000/-

b. NRS 3000/-

c. NRS 3500/-

d. NRS 4000/-

50. Provision for professional code of conduct of engineers in Nepal is made in detail in:

a. NEC Act

b. NEC Rules

c. NEC bye-laws

d. NEC ordience