***Engineer***

1. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

2. The strain energy due to volumetric strain

a. is directly proportional to the volume

b. is directly proportional to the square of exerted pressure

c. is inversely proportional to Bulk modulus

d. all the above.

3. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

4. The minimum number of rivets for the connection of a gusset plate, is

a. 1

b. 2

c. 3

d. 4

5. The range within which a load can be applied on a rectangular column, to avoid any tensile stress, is

a. one-third of the base

b. one-fourth of the base

c. one-fifth of the base

d. one sixth of the base on either side of centroid.

6. Pick up the correct statement from the following :

a. the principal axes of both the lenses coincide the optical axis of the telescope

b. the lens nearer the object to be viewed is convex and is called objective

c. the lens nearer the eye is called eyepiece

d. all the above.

7. The ratio of the length of long chord and the tangent length of a circular curve of radius R deflecting through angle Δ, is

a. cos

b. tan

c. 2 sin

d. 2 cos .

8. The Random errors tend to accumulate proportionally to

a. numbers of operations involved

b. reciprocal of operations involved

c. square root of the number of operation involved

d. cube root of the number of operation involved.

9. If the radius of a simple curve is 600 m, the maximum length of the chord for calculating offsets, is taken

a. 15 m

b. 20 m

c. 25 m

d. 30 m.

10. Pick up the correct statement from the following

a. 1 degree of arc corresponds to a displacement ratio of 1:57

b. the angular errors tend to propagate themselves along a traverse as the square root of the number of stations

c. the errors arising from the linear measures tend to be roughly proportional to the lengths of the lines

d. all the above.

11. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

12. An angle of deflection right, may be directly obtained by setting the instrument to read

a. zero on back station

b. 180° on back station

c. 90°

d. 270° on back station.

13. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

14. Good quality cement contains higher percentage of

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite

15. In   the   method   of   condensation polymerization,

a. low-molecular substances are removed from the high molecular substance

b. the reaction proceeds with an evolution of ammonia

c. the reaction proceeds with an evolution of hydrogen chloride

d. all the above.

16. Plastics are compounds of carbon with element

a. hydrogen

b. nitrogen

c. oxygen

d. All of these.

17. Stones used for ornamental work must be

a. soft

b. hard

c. light

d. heavy.

18. Pick up the correct statement from the following:

a. Catalysts are added to assist and accelerate the hardening of resin,

b. The fillers are inert materials and they impart strength and hardness

c. Fibrous fillers increase thermal resistance

d. All the above.

19. Pick up the correct statement from the following i.

a. Higher workability indicates deficiency of sand

b. If the concrete mix is dry, the slump is zero

c. Concrete mix having zero slump, is unsuitable for high strength.

d. All the above.

20. The surface where two successive placements of concrete meet, is known as

a. Expansion joint

b. Construction joint

c. both (a) and (b)

d. both (6) and (c).

21. Pick up the incorrect statement from the following:

a. In properly graded aggregates, bulk density is more

b. In single size aggregates, bulk density is least

c. In single size aggregates, bulk density is maximum

d. None of these.

22. Cement used for normal concrete construction, is obtained by burning a mixture of

a. argilaceous and calcareous materials

b. silicious and catcareous materials

c. silicious, argillaceous and calcareous materials

d. none of these.

23. The strength and quality of concrete, depend upon:

a. surface area of aggregates

b. shape of aggregates

c. surface texture of aggregates

d. all the above.

24. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

25. According to the Indian Standards the specific gravity is the ratio of the unit weight of soil solids to that of water at a temperature of

a. 17°C

b. 23°C

c. 27°C

d. 30°

26. Coulomb's wedge theory assumes that

a. slip surface is the plane which passes through the heel of the wall

b. sliding wedge itself acts as a rigid body and the value of earth pressure is obtained by considering the limiting equilibrium of the wedge

c. position and direction of the resultant earth pressure, are known

d. all the above.

27. Fundamental relationship between dry density (γd), bulk density (γ) and water content (ω), is :

a. γ =

b. γd =

c. ω =

d. ω =

28. When the seepage pressure becomes equal to the pressure due to submerged weight of a soil, the effective pressure is reduced to zero and the soil particles have a tendency to move up in the direction of flow. This phenomenon is generally known

a. quick condition

b. boiling condition

c. quick sand

d. all the above.

29. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

30. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

31. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

32. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

33. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

34. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

35. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

36. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

37. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

38. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

39. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

40. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

41. The area occupied by Ao (drawing sheet) is ........... times the area occupied by A4 (drawing sheet)

a. 2

b. 4

c. 8

d. 16

42. Uses of the T-square, set squares, scale and protractor are combined in the

a. set-square

b. drafting machine

c. compass

d. none of the above

43. The maps which are on sufficient large scale to enable the individual features shown on the map to be identified on the ground by their shape and position are called :

a. topographical maps

b. geographical maps

c. geological maps

d. site plan

44. ....... is used for setting of short equal distances.

a. compass

b. bow divider

c. scale

d. lengthening bar

45. If P is principal amount, i is the rate of interest per annum and n is the number of periods in years, the compound amount factor (CAF) is :

a. (1 + i)n

b. (1 + i)(1/2n)

c. (n + i)

d. None of these

46. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

47. Pick up the correct reason for making conceptual (or preliminary) estimate from the following:

a. To have a check on a definitive cost estimate.

b. To check qoutations from contractors and/or sub contractors.

c. To compute target estimate for the owner while drawings and specifications are in initial stage.

d. All of these.

48. During construction price variation clause in a contract applies to

a. increase in cost of important materials only

b. variation in the cost of material labour and fuel

c. variation in total cost of the project estimated as lump sum figure

d. the rate of inflation

49. Collusive practice" means

a. offering anything to influence the action of public official

b. arrangement between two or more parties to achieve an improper purpose including influencing improperly the action of another party

c. mis representation of facts in order to influence a procurement process

d. all of the above

50. The Nepal Engineers' Association was established in

a. 2014 BS

b. 2024 BS

c. 2034 BS

d. 2044 BS