***Engineer***

1. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

2. Pick up the correct statement from the following:

a. The shear stress is zero at the outer fibres and maximum at the neutral axis

b. The bending stress at the outer fibres, is known as principal stress

c. The planes of principal stresses are inclined at 45° to the neutral plane

d. All the above.

3. In case of principal axes of a section

a. sum of moment of inertia is zero

b. difference of moment inertia is zero

c. product of moment of inertia is zero

d. none of these.

4. The shape of the bending moment diagram over the length of a beam, carrying a uniformly distributed load is always

a. linear

b. parabolic

c. cubical

d. circular.

5. For a given material Young's modulus is 200 GN/m2 and modulus of rigidity is 80 GN/m2. The value of Poisson's ratio is

a. 0.20

b. 0.25

c. 0.30

d. 0.40

6. Measuring with a 30 m chain, 0.01 m too short, introduces

a. positive compensating error

b. negative compensating error

c. positive cumulative error

d. negative cumulative error.

7. The smaller horizontal angle between the true meridian and a survey line, is known

a. declination

b. bearing

c. azimuth

d. dip.

8. The minimum range for sliding the focusing lens in the internal focusing telescope for focusing at all distances beyond 4 m is

a. 5 mm

b. 10 mm

c. 15 mm

d. 20 mm.

9. An angles of 45° with a chain line may be set out with

a. optical square

b. open cross staff

c. Fench cross staff

d. prismatic square.

10. Pick up the correct statement from the following :

a. with both handles in his left hand, the chain man throws out the chain with his right hand and the second chain man assists him to free it from knots

b. the follower of the chaining operation should be more experienced than the leader

c. at the end of the tenth chain length, the two chain men meet and the ten arrows are handed over to the leader

d. all the above.

11. Perpendicularity of an offset may be judged by eye, if the length of the offset is

a. 5 m

b. 10 m

c. 15 m

d. 20 m.

12. Resolving power of a telescope depends on

a. the diameter of the aperture

b. the pupil aperture of the eye

c. the diameter of the object glass

d. all the above.

13. Which one of the following is an air binding material ?

a. Gypsum

b. Acid-resistant cement

c. Quick lime

d. All of these.

14. Laterite is a/an

a. volcanic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

15. The silica is used for preparing

a. silica bricks

b. coke oven

c. lining for glass furnaces

d. all of these.

16. The commonly used lime in white washing, is

a. fat lime

b. hydraulic lime

c. lime

d. quick lime.

17. Smith's test of stones is performed to find out

a. the presence of soluble matter of stone

b. the compressive strength of the stone

c. the hardness of the stone

d. the toughness of the stone.

18. The commonly used raw material in the manufacture of cement, is

a. slate

b. sand stone

c. lime stone

d. basalt.

19. M 150 grade of concrete approximates

a. 1 : 1 :2 mix

b. 1 : 2 : 4 mix

c. 1 : 1.5 : 3 mix

d. none of these.

20. High temperature

a. increases the strength of concrete

b. decreases the strength of concrete

c. has no effect on the strength of concrete

d. none of these.

21. If the effective plan area of a warehouse is 54 sq. m, and maximum height of piles permitted is 270 cm, the number of cement bags to be stored, is

a. 2200 bags

b. 2400 bags

c. 2700 bags

d. 3000 bags

22. Water cement ratio is generally expressed in volume of water required per

a. 20 kg

b. 30 kg

c. 40 kg

d. 50 kg

23. Vicat's apparatus is used for

a. consistency test

b. setting time test

c. soundness test

d. compressive strength test.

24. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

25. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

26. The ultimate bearing capacity of a soil, is

a. total load on the bearing area

b. safe load on the bearing area

c. load at which soil fails

d. load at which soil consolidates.

27. The maximum net pressure intensity causing shear failure of soil, is known

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

28. Pick up the incorrect definition from the following:

a. The rotation of soil particles into stable state while remoulding, is known as the thiostropy of soil

b. The water content at which a soil changes from the liquid state to solid state, is known liquid limit of the soil

c. The water content at which a soil flows, is known plastic limit of the soil

d. None of these.

29. The ratio of volume of air voids to the volume of total voids, is known as

a. air content

b. percentage air voids

c. percentage voids

d. porosity.

30. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

31. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

32. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

33. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

34. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

35. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

36. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

37. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

38. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

39. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

40. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

41. To draw very light line which pencil is used

a. B

b. HB

c. H

d. 2H

42. Angles in multiples of 15 degree are constructed by the combined use of

a. T-square and set-square

b. set-square and scale

c. scale and french curve

d. french curve and erasing shield

43. The plane on which perspective is formed is called:

a. horizontal plane

b. picture plane

c. ground plane

d. central plane

44. Free hand sketching can be done only when, one has

a. proper proficiency

b. good practice

c. proper patience et

d. all of the above

45. Pick up the correct statement regarding financial statement analysis from the following.

a. Final analysis always involves the use of various financial statements i.e., balance sheet and income statement.

b. The balance sheet is the summary of assests, liabilities and owner's equity of business at a point in time.

c. The income statement is the summary of revenues and expenses of a firm over a particular period of time.

d. All the above

46. Renu Bala deposits Rs 1200 now, Rs 800 two years from now and Rs 1000 five years from now. If the savings bank's rate of interest in 5%, she will receive an amount of Rs X 10 years from now, where X is

a. Rs 3415

b. Rs 4225

c. Rs 4413

d. Rs 4826

47. Pick up the correct statement from the following:

a. The gross profit percentage is the average profit margin obtained on goods sold.

b. The relationship of contribution to sales is known as contribution ratio

c. The difference between sales and variable cost of sales, is called contribution.

d. All of these

48. Integrity is

a. state or quality of being complete, undivided or unbroken moral soundness, honesty & uprightness

b. the wholeness of deriving quality

c. both a) and b) of above

d. none of the above

49. "Corrupt practice" means

a. offering/receiving anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above

50. For the registiration of professional engineer amount fee required is:

a. NRS 5000/-

b. NRS 10000/-

c. NRS 15000/-

d. NRS 20000/-