***Engineer***

1. A shaft subjected to a bending moment M and a torque T, experiences

a. maximum bending stress =

b. maximum shear stress =

c. both (a) and (b)

d. neither (a) nor (b)

2. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

3. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

4. For structural analysis, Maxwell's reciprocal theorem can be applied to :

a. plastic structures

b. elastic structures

c. symmetrical structures

d. all the above.

5. For a cantilever with a uniformly distributed load W over its entire length L, the maximum bending moment is

a.  WL

b.  WL

c.  W2L

d.  WL2

6. The angle of intersection of a curve is the angle between

a. back tangent and forward tangent

b. prolongation of back tangent and forward tangent

c. forward tangent and long chord

d. back tangent and long chord.

7. If i is the stadia distance, f is the focal length and d is the distance between the objective and vertical axis of the techeometer, the multiplying constant, is

a. f/i

b. i/f

c. (f + d)

d. f/d.

8. The chaining on sloping ground is

a. easier along the falling gradient

b. easier along the up gradient

c. equally convenient along falling as well as up gradient

d. all the above.

9. Probable accidental error in precise levelling as recommended by International Geodetic Association, should not exceed (where k is in kilometers.)

a. ± 0.5 k mm

b. ± 1 k mm

c. ± 2 k mm

d. ± 5 k mm.

10. A theodolite is said to be in perfect adjustment if

a. rotation axis is vertical to the transit axis

b. transit axis is perpendicular to line of collimation

c. line of collimation sweeps out a vertical plane while the telescope is elevated or depressed

d. all the above.

11. The Random errors tend to accumulate proportionally to

a. numbers of operations involved

b. reciprocal of operations involved

c. square root of the number of operation involved

d. cube root of the number of operation involved.

12. Accuracy of elevation of various points obtained from contour map is limited to

a.  of the contour interval

b.  th of the contour interval

c.  rd of the contour interval

d.  th of the contour interval.

13. Good quality cement contains higher percentage of

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite

14. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

15. Pick up the correct statement from the following:

a. In stone arches, the stones are placed with their natural beds radial

b. In cornices, the stones are placed with their natural beds as vertical

c. In stone walls, the stones are placed with their natural beds as horizontal

d. All the above.

16. Durability of building stone is affected by its

a. texture

b. resistance to atmosphere

c. location in structure

d. all the above.

17. The variety of pig iron used for the manufacture of steel by Bessemer process, is

a. Bessemer pig

b. Grey pig

c. White forge pig

d. Mottled pig.

18. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

19. Water cement ratio is

a. weight of water to that of cement

b. weight of concrete to that of water

c. volume of concrete to that of water

d. both (a) and (b) of the above.

20. 'Ware house pack' of cement means

a. pressure exertion of the bags of upper layers

b. pressure compaction of the bags on lower layers

c. packing the ware house

d. none of these.

21. Concrete containing

a. silicious aggregates, has higher co-efficient of expansion

b. igneous aggregates, has intermediate coefficient of expansion

c. lime stones, has lowest co-efficient of expansion

d. All the above.

22. Strength of concrete with passage of time

a. increases

b. decreases

c. fluctuates

d. remains constant.

23. Specified compressive strengh of concrete is obtained from cube tests at the end of

a. 7 days

b. 14 days

c. 21 days

d. 28 days.

24. Depending upon the properties of a material, the failure envelope may

a. be either straight or curved

b. pass through the origin of stress

c. intersect the shear stress axis

d. all the above.

25. The ultimate Settlement of a soil is directly proportional to:

a. compressive index

b. void ratio

c. both (a) and (b)

d. none of these.

26. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

27. The compression resulting from a long term static load and consequent escape of pore water, is known as

a. compaction

b. consolidation

c. swelling

d. none of these.

28. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

29. A structure is erected on an impervious clay whose thickness is 12 m. Drainage is possible both at upper and lower surfaces. Coefficient of consolidation is 0.015 cm2 per minute. For attaining 50% consolidation with a time factor 0.20, the number of days required

a. 3233

b. 3123

c. 33331

d. 3313

30. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

31. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

32. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

33. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

34. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

35. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

36. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

37. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

38. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

39. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

40. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

41. The unseen/inner edge of an object is represented by

a. solid line

b. dotted line

c. hatch line

d. none of the above

42. Axonometric drawing is a drawing like

a. parallel to the plane

b. perpendicular to the plane

c. inclined to the plane

d. all of the above

43. The most of the texture can be drawn with the help of drawing pen with line thickness

a. 0.1mm

b. 0.5mm

c. 0.8mm

d. 1.0mm

44. To remove unnecessary lines ....... is used

a. duster

b. chalk

c. sand box

d. eraser

45. Pick up the correct statement from the following:

a. A NPV profile graph shows the curvilinear relationship between the net present value of the project and discount rate employed.

b. In a NPV profile, if discount rate is zero, then net present value is simply total cash inflows less the total cash outflows of the project.

c. As the discount rate increases, the net present value profile slopes downward to the right.

d. All of these

46. Pick up the correct statement from the following:

a. The ratio of current assests, loans and advances, and the current liquidity is called current ratio.

b. Larger the current ratio, larger is the margin of safety.

c. The operating profit is the difference between gross profit and operating expenses.

d. All of these

47. Renu Bala deposits Rs 1200 now, Rs 800 two years from now and Rs 1000 five years from now. If the savings bank's rate of interest in 5%, she will receive an amount of Rs X 10 years from now, where X is

a. Rs 3415

b. Rs 4225

c. Rs 4413

d. Rs 4826

48. Code of conduct is not related with

a. making the advice of the enginerr impartial

b. building code

c. ensuring the quality of advice

d. making the advice untained by any personal consideration

49. Which of the following an engineer should not follow:

a. act as faithful agent for client

b. accept compensation from more than one party for same service without the consent of all

c. indicate client the adverse consequences if judgment is over ruled

d. undertake only these assignments for which he is qualified

50. Engineering shall act in such a manner as to uphold and enhance the honor, and dignity of the engineering profession and shall act with ..... for bribery, fraud, and corruption.

a. a tolerance

b. a penchant

c. zero tolerance

d. a predilection