***Engineer***

1. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

2. A spring of mean radius 40 mm contains 8 action coils of steel (N = 80000 N/mm2), 4 mm in diameter. The clearance between the coils being 1 mm when unloaded, the minimum compressive load to remove the clearance, is

a. 25 N

b. 30 N

c. 35 N

d. 40 N

3. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

4. Pick up the correct statement from the following :

a. In the standard rolled channels, the shear centre is on the horizontal line passing through and away from the C.G. beyond web

b. In equal angles, the shear centre is on the horizontal plane and away from the C.G., outside of the leg projection

c. In T-sections, the shear centre is at the C.G. of the section

d. All the above.

5. A rectangular beam 20 cm wide is subjected to a maximum shearing force of 10, 000 kg, the corresponding maximum shearing stress being 30 kg/cm2. The depth of the beam is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm.

6. The ratio of the linear displacement at the end of a line, subtended by an arc of one second to the length of the line, is

a. 1:206 300

b. 1:3440

c. 1:57

d. 1:100.

7. The slope correction for a length of 30 m along a gradient of 1 in 20, is

a. 3.75 cm

b. 0.375 cm

c. 37.5 cm

d. 2.75 cm.

8. The operation of making the algebraic sum of latitudes and departures of a closed traverse, each equal to zero, is known

a. balancing the sights

b. balancing the departures

c. balancing the latitudes

d. balancing the traverse.

9. The 'fix' of a plane table station with three known points, is bad if the plane table station lies

a. in the great triangle

b. outside the great triangle

c. on the circumference of the circumscribing circle

d. none of these.

10. If deflection angles are measured in a closed traverse, the difference between the sum of the right-hand and that of the left hand angles should be equal to

a. 0°

b. 90°

c. 180°

d. 360°

11. The branch of surveying in which both horizontal and vertical positions of a point, are determined by making instrumental observations, is known

a. tacheometry

b. tachemetry

c. telemetry

d. all the above.

12. If a linear traverse follows a sharp curve round a large lake where it is difficult to have long legs, the accuracy of the traverse may be improved by

a. taking short legs

b. making repeated observations of angular and linear measurements

c. making a subsidiary traverse to determine the length of a long leg

d. all the above.

13. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

14. Calcination of iron ores is done

a. to remove carbonic acid

b. by roasting in heaps

c. after dressing

d. all the above.

15. If water required for 1 bag of cement is 30 litres, the water cement ratio is :

a. 0.40

b. 0.50

c. 0.60

d. None of these.

16. The cast iron when heated to red heat with powdered red haemetite in an oven for increasing its toughness, is converted to

a. grey cast iron

b. white cast iron

c. mottled cast iron

d. toughed cast iron.

17. Plastic asphalt is

a. used as a water proofing layer over roof

b. a mixture of cement and asphalt

c. a natural asphalt

d. a refinery product.

18. The cracks which extend from bark towards the sap wood in the cross section of a tree, are called

a. radial shakes

b. star shakes

c. heart shakes

d. cup shakes.

19. Pick up the incorrect statement from the following:

a. Admixtures accelerate hydration

b. Admixtures make concrete water proof

c. Admixtures make concrete acid proof

d. Admixtures give high strength

20. Ordinary concrete is not used for concrete grade

a. M 150

b. M 200

c. M 250

d. M 400

21. In the method of voids for determination of the quantity of cement paste, it is assumed that

a. Voids in fine aggregates are filled by the cement paste

b. Volume of fine aggregates is equal to total voids in coarse aggregates plus 10% extra

c. Volume of cement paste required is equal to total volume of voids in fine aggregates plus 15% extra

d. All the above.

22. Pick up the correct statement from the following:

a. The free water is the amount of water added while mixing and the amount of water held on the surface of the aggregates prior to mixing

b. The total water is the free water and the amount actually absorbed by the aggregates

c. Neither (a) nor (b)

d. Both (a) and (b).

23. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

24. The ratio of settlement at any time 't' to the final settlement, is known as

a. co-efficient of consolidation

b. degree of consolidation

c. consolidation index

d. consolidation of undisturbed soil.

25. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

26. If G is specific gravity of sand particles, e is porosity, the critically hydraulic gradient

a. ic =

b. ic =

c. ic =

d. ic =

27. 'Drift' is the material picked up, mixed, disintegrated, transported and redeposited by

a. wind

b. gravitational force

c. glaciated water

d. all the above.

28. The ratio of the difference between the void ratio of the soil in its loosest state and its natural void ratio (e) to the difference between the void ratios in the loosest and fully dense state, is generally termed as

a. degree of density

b. relativity

c. density index

d. all the above.

29. The maximum net pressure intensity causing shear failure of soil, is known

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

30. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

31. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

32. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

33. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

34. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

35. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

36. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

37. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

38. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

39. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

40. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

41. The purpose of making the drawing board with seasoned soft wood

a. easy to carry

b. for fixing of drawing paper with the help of drawing pin

c. to match with T-square

d. all of the above

42. Bisecting a line means, divide a line into..... equal part.

a. two

b. three

c. four

d. none of the above

43. Free hand sketch is normally used for

a. concept

b. final

c. discussion

d. none of the above

44. Circles of small radii are drawn by means of a

a. straight bar

b. bow compass

c. lengthening bar

d. drop compass

45. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

46. The project contractor relies on the cost of the estimate :

a. for submission of a competitive bid for a lumpsum contract

b. for a unit price contract

c. for preparation of a definitive estimate to help negotiate contract.

d. All of these

47. Liquidity ratios are used :

a. to measure a firms ability to meet short-cut obligations.

b. to compare short term obligations to short-term resources available to meet these obligations.

c. to obtain much insight into the present cash solvency of the firm and the firm's ability to remain solvent in the event of adversity.

d. All of these

48. According to the Public Procurement Regulation, 2064, Who approves the prequalification criteria ?

a. concerned project chief

b. concerned department chief

c. concerned secretary

d. concerned minister

49. NEC act and its regulation........

a. 55 and 57

b. 54 and 55

c. 55 and 56

d. 57 and 58

50. Workers are not allowed to work contiunously more than

a. 3hr

b. 5hr

c. 7hr

d. 4hr