***Engineer***

1. A load of 1960 N is raised at the end of a steel wire. The minimum diameter of the wire so that stress in the wire does not exceed 100 N/mm2 is :

a. 4.5 mm

b. 5.0 mm

c. 5.5 mm

d. 6.0 mm

2.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

3. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

4. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

5. The moment diagram for a cantilever carrying linearly varying load from zero at its free end and to maximum at the fixed end will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

6. If h is the difference in level between end points separated by l, then the slope correction is  . The second term may be neglected if the value of h in a 20 m distance is less than

a.  m

b. 1 m

c. 2 m

d. 3 m

7. Volume of the earth work may be calculated by

a. end areas

b. Prismoidal formula

c. Trapezoidal

d. all the above.

8. Pick up the correct statement from the following :

a. The directions of plumb lines suspended at different points in a survey are not strictly parallel

b. In surveys of small extent, the effect of curvature may be ignored and the level surface of the earth is assumed as horizontal

c. In surveys of large extent, the effect of curvature of the earth must be considered

d. All the above.

9. If a tacheometer is fitted with an anal-latic lens

a. additive constant is 100, multiplying constant is zero

b. multiplying constant is 100, additive constant is zero

c. both multiplying and additive constants are 100

d. both multiplying and additive constants are 50.

10. The boundary of water of a still lake, represents

a. level surface

b. horizontal surface

c. contour line

d. a concave surface.

11. A clinometer is used for

a. measuring angle of slope

b. correcting line of collimation

c. setting out right angles

d. defining natural features.

12. The formula for the horizontal distances for inclined sights, on staff held normalis  s cos θ + (f + d) cos θ ± h sin θ

a. minus sign is used for angle of depression

b. plus sign is used for angle of depression

c. minus sign is used for angle of elevation

d. non of these.

13. Quick lime

a. reacts with carbon dioxide

b. may be used for white-washing

c. when mixed with water forms slaked lime

d. all the above.

14. Vanadium steel is generally used for

a. railway switches and crossing

b. bearing balls

c. magnets

d. axles and springs.

15. Pick up the correct statement from the following:

a. Attrition test is done to find out the rate of wear of stones which are used in road construction

b. Crushing test is done to find out the compressive strength of the stone

c. Impact test is done to determine toughness of a stone

d. All the above.

16. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

17. The usual percentages of clay and metal in cermet are :

a. 50%, 50%

b. 60%, 40%

c. 70%, 30%

d. 80%, 20%.

18. Pegmatite is a/an

a. intrusive igneous rock

b. extrusive igneous rock

c. sedimentary rock

d. metamorphic rock.

19. The concrete mix which causes difficulty in obtaining a smooth finish, possess

a. internal friction

b. harshness

c. bleeding

d. none of these.

20. Pick up the correct statement from the following:

a. High percentage of C3S and low percentage of C2S make the cement less resistive to chemical attack

b. Low percentage of C3S and high percentage of C2S contribute to slow hardening

c. Low percentage of C3S and high percentage of C2S provide greater resistance to chemical attack

d. All the above.

21. The 28 days cube strength of mass concrete using aggregates of maximum size 5 cm for gravity dams should be

a. between 350 to 600 kg/cm2

b. between 150 to 500 kg/cm2

c. below 200 kg/cm2

d. none of these.

22. After casting, an ordinary cement concrete on drying

a. expands

b. mix

c. shrinks

d. none of these.

23. The cement becomes useless if its absorbed moisture content exceeds

a. 2%

b. 3%

c. 4%

d. 5%

24. The earth pressure of a soil at rest, is proportional to (where φ is the angle of internal friction of the soil)

a. tan (45° + φ)

b. tan2 (45° - φ)

c. tan2 (45° + φ)

d. none of these

25. Cohesive soils are generally

a. plastic and also compressible

b. elastic and also compressible

c. plastic but incompressible

d. none of these.

26. Soils containing organic matters

a. are of spongy nature

b. swell with decrease of moisture

c. shrink with increase of moisture content

d. none of these.

27. Pick up the correct statement from the following:

a. The porosity of a soil is defined as the ratio of the volume of voids to the gross volume of the soil

b. The bulk density of a soil is defined as the unit weight of the soil

c. The dry density of a soil is defined as weight of solids to the total volume of the soil

d. All the above.

28. A stratum of clay 2 m thick will get consolidated 80% in 10 years. For the 80% consolidation of 8 m thick stratum of the same clay, the time required is

a. 120 years

b. 140 years

c. 160 years

d. 180 years.

29. Which one of the following statements is true ?

a. Clays are more porous than sands

b. Pressure of organic matter in a soil decreases the bearing capacity of the soil

c. Aluminous cement is used for foundations in soils with chemical deposits

d. All the above.

30. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

31. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

32. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

33. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

34. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

35. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

36. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

37. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

38. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

39. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

40. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

41. The locus of centre of curvature is known as

a. space

b. evolute

c. point

d. centre

42. The most of the texture can be drawn with the help of drawing pen with line thickness

a. 0.1mm

b. 0.5mm

c. 0.8mm

d. 1.0mm

43. Drawing is the .................of engineers.

a. language

b. tools

c. machine

d. all of the above

44. Free hand sketching can be done only when, one has

a. proper proficiency

b. good practice

c. proper patience et

d. all of the above

45. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

46. Pick up the correct statement from the following:

a. The capital required to get a project started is the first cost.

b. The first cost is a single cash flow or a series of cash flows that are made in the beginning of the activity's life span

c. The first cost of purchasing a car is the sum of the down payment, taxes and dealers charges.

d. All of these

47. If P is principal amount, i is the rate of interest and n is the number of periods in years, then the interest factor is :

a. (1 + ni)

b. (ni - 1)

c. ni

d. None of these

48. According to labour act considering health condition is

a. first aid after accident

b. safety on inst. and cloth

c. education& training

d. none of the above

49. For procurement of works up to 2 million, buy which of the following method is to be considered?

a. by sealed tender

b. by direct negotiation

c. by bidding

d. by direct labour or direct negotiation

50. Provision for professional code of conduct of engineers in Nepal is made in detail in:

a. NEC Act

b. NEC Rules

c. NEC bye-laws

d. NEC ordience