***Engineer***

1. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

2. A truss containing j joints and m members, will be a simple truss if

a. m = 2j - 3

b. j = 2m - 3

c. m = 3j - 2

d. j = 3m - 2

3. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

4. In a tension test, the yield stress is 300 kg/cm2, in the octa hedral shear stress at the point is:

a. 100 2 kg/cm2

b. 150 2 kg/cm2

c. 200 2 kg/cm

d. 250 2 kg/cm2.

5. For a stable frame structure, number of members required, is

a. twice the number of joints minus three

b. twice the number of joints minus two

c. twice the number of joints minus one

d. none of these.

6. A standard steel tape of length 30 m and cross-section 15 x 1.0 mm was standardised at 25°C and at 30 kg pull. While measuring a base line at the same temperature, the pull applied was 40 kg. If the modulus of elasticity of steel tape is 2.2 x 106 kg/cm2, the correction to be applied is

a. - 0.000909 m

b. + 0.0909 m

c. 0.000909 m

d. none of these

7. ABCD is a regular parallelogram plot of land whose angle BAD is 60°. If the bearing of the line AB is 30°, the bearing of CD, is

a. 90°

b. 120°

c. 210°

d. 270°

8. Back bearing of a line is equal to

a. Fore bearing ± 90°

b. Fore bearing ± 180°

c. Fore bearing ± 360°

d. Fore bearing ± 270°

9. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum perpendicular offset for the transition curve, is

a. 1.70 m

b. 2.70 m

c. 3.70 m

d. 4.70 m

10. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

11. Two concave lenses of 60 cm focal length are cemented on either side of a convex lens of 15 cm focal length. The focal length of the combination is

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

12. If the rate of gain of radial acceleration is 0.3 m per sec3 and full centrifugal ratio is developed. On the curve the ratio of the length of the transition curve of same radius on road and railway, is

a. 2.828

b. 3.828

c. 1.828

d. 0.828.

13. Sand stone is

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. volcanic rock.

14. If the furnace is provided with insufficient fuel at low temperatures, the type of pig iron produced, is called

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

15. For a 50 kg cement bag water required, is

a. 18.5 litres

b. 20.5 litres

c. 22.5 litres

d. 25 litres.

16. Wrought iron contains carbon upto

a. 0.25%

b. 1.0%

c. 1.5%

d. 2%.

17. If water required for 1 bag of cement is 30 litres, the water cement ratio is :

a. 0.40

b. 0.50

c. 0.60

d. None of these.

18. The portion of a brick cut to form angles other than right angles in plan, is known as

a. queen closer

b. king closer

c. closer

d. squint brick.

19. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

20. Pick up the incorrect statement applicable to the field test of good cement.

a. The colour of the cement is bluish

b. A handful of cement thrown into a bucket of water should sink immediately

c. By rubbing cement in between fingers, one should feed rough

d. All the above.

21. The mixture of different ingredients of cement, is burnt at

a. 1200°C

b. 1400°C

c. 1600°C

d. 1800°C

22. Bulking of sand is

a. mixing of different sizes of sand particles

b. mixing of lime with sand

c. maximum water with sand

d. swelling of sand when wetted.

23. While compacting the concrete by a mechanical vibrator, the slump should not exceed

a. 5.0 cm

b. 7.5 cm

c. 10 cm

d. 15 cm

24. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the degree of saturation, is

a. 95.4%

b. 95.5%

c. 95.6%

d. 95.7%

25. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

26. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the void ratio, is

a. 0.858

b. 0.860

c. 0.862

d. 0.864

27. Pick up the correct statement from the following:

a. If the ratio of depth to width is more than 2, it is deep foundation

b. If the ratio of the length to width is between 1 and 2, it is spread foundation

c. If the length is large as compared to width, it is a strip foundation

d. All the above.

28. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

29. The specific gravity of sands, is approximately

a. 2.0

b. 2.2

c. 2.4

d. 2.6

30. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

31. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

32. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

33. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

34. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

35. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

36. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

37. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

38. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

39. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

40. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

41. In full sectional view, the object is imagined to be cut off

a. one third

b. one half

c. one fourth

d. one fifth

42. Which of the following theories of mathematics is used by a CAD (computer Aided Design) software in are in creating a perspective design?

a. descriptive geometry

b. linear algebra

c. Bernoulli's theorem

d. vector analysis

43. The scale should never be used as a ........ for drawing straight lines.

a. set-square

b. working edge

c. straight edge

d. none of the above

44. In the first angle projection the object is assumed to be placed in

a. fourth quadrant

b. third quadrant

c. second quadrant

d. first quadrant

45. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

46. If a seller recovers his capital along with accumulated compensating interest not in one single lumpsum payment but in periodical equal payments, over time :

a. Capital Recovery Annuity fs availed

b. Present work Annuity is availed

c. Sinking Fund Annuity is availed

d. Sinking Fund Annuity is availed

47. Renu Bala deposits Rs 1200 now, Rs 800 two years from now and Rs 1000 five years from now. If the savings bank's rate of interest in 5%, she will receive an amount of Rs X 10 years from now, where X is

a. Rs 3415

b. Rs 4225

c. Rs 4413

d. Rs 4826

48. NEC act and its regulation........

a. 55 and 57

b. 54 and 55

c. 55 and 56

d. 57 and 58

49. To work for survey, design estimate & supervision of a road project, the consultant has to submit

a. technical proposal

b. financial proposal

c. technical and financial proposal

d. none of the above

50. The purpose of bid evaluation is to determine

a. lowest bid

b. responsive bid

c. substantially responsive bid

d. lowest evaluated substantially responsive bid