***Engineer***

1. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

2. A rolled steel joist is simply supported at its ends and carries a uniformly distributed load which causes a maximum deflection of 10 mm and slope at the ends of 0.002 radian. The length of the joist will be,

a. 12 m

b. 14 m

c. 16 m

d. 18 m

3. A simply supported beam which carries a uniformly distributed load has two equal overhangs. To have maximum B.M. produced in the beam least possible, the ratio of the length of the overhang to the total length of the beam, is

a. 0.207

b. 0.307

c. 0.407

d. 0.508

4. A cantilever carrying a uniformly distributed load W over its full length is propped at its free end such that it is at the level of the fixed end. The bending moment will be zero at its free end also at

a. fixed point of the cantilever

b. 1/4th length from free end

c. 3/4th length from free end

d. half length from free end.

5. The maximum twisting moment a shaft can resist, is the product of the permissible shear stress and

a. moment of inertia

b. polar moment of inertia

c. polar modulus

d. modulus of rigidly.

6. For high sensivity of the bubble tube

a. a liquid of low surface tension is used

b. the bubble space should be long

c. the bubble tube should not be too narrow

d. all the above.

7. The 'point of curve' of a simple circular curve, is

a. point of tangency

b. point of commencement

c. point of intersection

d. mid-point of the curve

8. Back bearing of a line is equal to

a. Fore bearing ± 90°

b. Fore bearing ± 180°

c. Fore bearing ± 360°

d. Fore bearing ± 270°

9. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

10. The magnetic bearing of a line is 32° and the magnetic declination is 10° 15' W. The true bearing is

a. 21° 45'

b. 42° 15'

c. 42° 15' W

d. 21° 45' W

11. Grid lines are parallel to

a. magnetic meridian of the central point of the grid

b. line representing the central true meridian of the grid

c. geographical equator

d. none of these.

12. The ratio of the linear displacement at the end of a line, subtended by an arc of one second to the length of the line, is

a. 1:206 300

b. 1:3440

c. 1:57

d. 1:100.

13. Apiece of sawn timber whose cross-sectional dimensions exceed 5 cm, in one direction and 20 cm in the other direction, is called a

a. cant

b. deal

c. baulk

d. strip.

14. The most commonly used base for timber painting, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

15. If the furnace is provided with insufficient fuel at low temperatures, the type of pig iron produced, is called

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

16. Mastic asphalt is

a. water proof

b. fire proof

c. elastic

d. all the above.

17. Pick up the correct statement from the following:

a. Lime is available by dissolving calcium carbonate in water

b. Lime is available by calcining calcium carbonate at 900°C

c. Lime is nothing but calcium chloride

d. None of these.

18. Quick lime (or caustic lime)

a. is obtained by the calcination of pure lime stone

b. has great affinity to moisture

c. is amorphous

d. All the above.

19. Strength of concrete with passage of time

a. increases

b. decreases

c. fluctuates

d. remains constant.

20. Placing of concrete should preferably be done at a temperature of

a. 10°C

b. 20°C

c. 23°C

d. 27 ± 2°C

21. If 1500 g of water is required to have a cement paste 1875 g of normal consistency, the percentage of water is,

a. 25%

b. 30%

c. 35%

d. 40%

22. On a grading curve, the gap grading is represented by

a. a horizontal line

b. a vertical line

c. N.W. inclined line

d. N.E. inclined line

23. Slump test is done for

a. clay

b. sand

c. lime

d. concrete.

24. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

25. The maximum load carried by a pile, when it continues to sink without further increase of load, is known as

a. ultimate load carrying capacity

b. ultimate bearing capacity

c. ultimate bearing resistant

d. all the above.

26. Pick up the correct statement from the following:

a. Failure plane carries maximum shear stress

b. Failure plane does not carry maximum shear stress

c. Failure plane carries shear stress equal to maximum shear stress

d. None of these.

27. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

28. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

29. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

30. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

31. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

32. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

33. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

34. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

35. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

36. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

37. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

38. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

39. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

40. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

41. Number of equal pieces of A4 size paper from A0 is equal to

a. 2

b. 4

c. 8

d. 16

42. The locus of centre of curvature is known as

a. space

b. evolute

c. point

d. centre

43. The art of representation of an object by systemic lines on a paper is known as

a. drawing

b. art

c. dimensioning

d. profession

44. The projection of any feature on the horizontal plane is called

a. plan

b. elevation

c. map

d. all of the above

45. The more critical (or severe) test of the firm's liquidity can be judged by :

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

47. The owner of the construction company makes use of the estimate :

a. to assist in financial arrangements

b. to determine economic feasibility of the project.

c. to determine the tax, insurance and evaluation purpose.

d. All of these

48. According to the Public Procurement Regulation, 2064, Who approves the prequalification criteria ?

a. concerned project chief

b. concerned department chief

c. concerned secretary

d. concerned minister

49. As per the public Procurement Regulation, 2064 of Nepal, what is the maximum age for a technical proposal for consulting services ?

a. 90%

b. 80%

c. 70%

d. 60%

50. Labour act was introduced /founded in

a. 2038

b. 2048

c. 2043

d. 2053