***Engineer***

1. The strain energy due to volumetric strain

a. is directly proportional to the volume

b. is directly proportional to the square of exerted pressure

c. is inversely proportional to Bulk modulus

d. all the above.

2. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

3. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

4. The section modulus of a rectangular light beam 25 metres long is 12.500 cm3. The beam is simply supported at its ends and carries a longitudinal axial tensile load of 10 tonnes in addition to a point load of 4 tonnes at the centre. The maximum stress in the bottom most fibre at the mid span section, is

a. 13.33 kg/cm2 compressive

b. 26.67 kg/cm2 tensile

c. 26.67 kg/cm2 compressive

d. none of these.

5. The maximum compressive stress at the top of a beam is 1600 kg/cm2 and the corresponding tensile stress at its bottom is 400 kg/cm2 . If the depth of the beam is 10 cm, the neutral axis from the top, is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

6. Reduced bearing of a line is an angle between

a. north line and given line measured clockwise

b. north line and given line measured anticlockwise

c. east or west and the given line

d. given line and the part of the meridian whether N end or S end, lying adjacent to it.

7. Magnetic declination at any place

a. remains constant

b. does not remain constant

c. fluctuates

d. changes abruptly.

8. The smaller horizontal angle between the true meridian and a survey line, is known

a. declination

b. bearing

c. azimuth

d. dip.

9. While viewing through a level telescope and moving the eye slightly, a relative movement occurs between the image of the levelling staff and the cross hairs. The instrument is

a. correctly focussed

b. not correctly focussed

c. said to have parallax

d. free from parallax.

10. If θ is the slope of the ground and l is the measured distance, the correction is

a. 2l sin2 θ/2

b. 2l cos2 θ/2

c. 2l tan2 θ/2

d. 2l cot2 θ/2.

11. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

12. In a telescope the object glass of focal length 14 cm, is located at 20 cm from the diaphragm. The focussing lens is midway between them when a staff 16.50 m away is focussed. The focal length of the focussing lens, is

a. 5.24 cm

b. 6.24 cm

c. 7.24 cm

d. 8.24 cm

13. Black marble is generally found in the district of

a. Jaipur

b. Jabalpur

c. Jaisalmer

d. Pune.

14. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

15. Softer variety of steel may be obtained by

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

16. A well seasoned timber may contain moisture up to

a. 4 to 6%

b. 6 to 8%

c. 8 to 10%

d. 10 to 12%.

17. The most durable varnish is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

18. The most important constituent of varnish, is

a. drier

b. solvent

c. resin

d. all the above.

19. A concrete using an air entrained cement

a. has more resistance to weathering

b. is more plastic and workable

c. is free from segregation and bleeding

d. all the above.

20. Transport of concrete by pumps, is done for a distance of

a. 200 m

b. 300 m

c. 400 m

d. none of these.

21. Construction joints are provided

a. where the member is supported by other member

b. at 18 m apart in huge structures

c. in concrete wall at sill level of windows

d. all the above.

22. The increased cohesiveness of concrete, makes it

a. less liable to segregation

b. more liable to segregation

c. more liable to bleeding

d. more liable for surface scaling in frosty weather

23. If 50 kg of fine aggregates and 100 kg of coarse agregates are mixed in a concrete whose water cement ratio is 0.6, the weight of water required for harsh mix, is

a. 10 kg

b. 12 kg

c. 14 kg

d. 15 kg.

24. If G is specific gravity of sand particles, e is porosity, the critically hydraulic gradient

a. ic =

b. ic =

c. ic =

d. ic =

25. The expression  ρω is used for

a. dry density

b. bulk density

c. degree of saturation

d. optimum water content.

26. The property of a soil which allows it to be deformed rapidly without rupture, elastic rebound and also a volume change, is known

a. porosity

b. plasticity

c. permeability

d. ductility.

27. The direct shear test suffers from the following disadvantage:

a. Pore water pressure cannot be measured

b. Shear stress on the failure plane is not uniform.

c. The area under the shear and vertical loads does not remain constant throughout the test

d. All the above.

28. A triaxial shear test is prefered to direct shear test, because

a. it can be performed under all three driange conditions with complete control

b. precise measurement of pore pressure and change in volume during test, is not posible

c. stress distribution on the failure plane, is non uniform

d. none of these.

29. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

30. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

31. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

32. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

33. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

34. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

35. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

36. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

37. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

38. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

39. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

40. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

41. For drawing of small objects, the scale used is

a. enlarged

b. full size

c. reducing

d. all of the above

42. Mechanical lines are types of

a. mechanic drawing

b. line of drawn with free hand

c. line drawn with drawing equipment

d. none of the above

43. Bisecting a line means, divide a line into..... equal part.

a. two

b. three

c. four

d. none of the above

44. The other name of axonometric projection is

a. one plane projection

b. two plane projection

c. three plane projection on

d. all of the above

45. The construction estimate of a project is used by :

a. the owner of the facility

b. the consulting architect/engineer

c. the contractor of the project

d. All of these

46. The project contractor relies on the cost of the estimate :

a. for submission of a competitive bid for a lumpsum contract

b. for a unit price contract

c. for preparation of a definitive estimate to help negotiate contract.

d. All of these

47. Pick up the correct statement from the following:

a. An annuity is a series of equal payments occurring at equal period of time.

b. Annuity is called an equal payment or uniform payment series.

c. An annuity may have periods of time of any length but should always be of equal length.

d. All the above.

48. Fundamental principle of Professional Engineering Ethics is:

a. uploading and advancing Engineering Profession

b. maintain high level of ethical conduct

c. both of the above

d. none of the above

49. Which one type of membership is not categorized by NEC

a. general engineer

b. foreign engineer

c. professional engineer

d. supreintendent engineer

50. Who is the Chairperson of Project Co-ordination Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer