***Engineer***

1. Total strain energy theory for the failure of a material at elastic limit, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

2. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

3. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

4. The slenderness ratio of a vertical column of square cross- section of 10 cm side and 500 cm long, is

a. 17.3

b. 173.2

c. 137.2

d. 13.72

5. If two forces acting at a joint are not along the straight line, then for the equilibrium of the joint

a. one of the forces must be zero

b. each force must be zero

c. forces must be equal and of the same sign

d. forces must be equal in magnitude but opposite in sign.

6. The theodolites used for making tacheometric observations by optical wedge system, are

a. provided with stadia hairs in front of eye piece

b. not provided with stadia hairs at all

c. fitted with a glass wedge inside the telescope

d. fitted with a glass wedge in front of telescope.

7. A well conditioned triangle has no angle less than

a. 20°

b. 30°

c. 45°

d. 60°.

8. Whole circle bearing of a line is preferred to a quadrantal bearing merely because

a. bearing is not completely specified by an angle

b. bearing is completely specified by an angle

c. Sign of the correction of magnetic declination is different in different quadrants

d. its trigonometrical values may be extracted from ordinary tables easily.

9. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

10. If S is the length of a subchord and R is the radius of simple curve, the angle of deflection between its tangent and sub-chord, in minutes, is equal to

a. 573 R/S

b. 171.9 S/R

c. 1718.9 R/S

d. 1718.9 S/R.

11. While working on a plane table, the correct rule is :

a. Draw continuous lines from all instrument stations

b. Draw short rays sufficient to contain the points sought

c. Intersection should be obtained by actually drawing second rays

d. Take maximum number of sights as possible from each station to distant objects.

12. The representation of general topography of a very flat terrain is possible only

a. by drawing contours at large interval

b. by drawing contours at small interval

c. by giving spot levels at large interval

d. by giving spot levels to salient features at close interval.

13. The size of modular bricks, is

a. 10 x 10 x 9 cm

b. 19 x 9 x 9 cm

c. 22.5 x 10 x 8.5 cm

d. 22.5 x 8.0 x 9 cm.

14. Name the type of cement from the following for canal linings :

a. sulphate resisting cement

b. rapid hardening cement

c. quick setting cement

d. pozzuolana cement.

15. Plaster of Paris is obtained by calcining

a. gypsum

b. kankar

c. lime stone

d. none of these.

16. The initial setting time of hydraulic lime, is

a. 60 minutes

b. 90 minutes

c. 120 minutes

d. 150 minutes.

17. The plastics prepared from Vinyl resin are

a. non-toxic

b. transparent

c. colourless

d. all of these.

18. The steel used for rails under heavy traffic and on sharp curves, is

a. Nickel steel

b. Chrome steel

c. Magnese steel

d. Vanadium steel.

19. Pozzolana cement is used with confidence for construction of

a. massive foundations

b. abutments

c. R.C.C. structures

d. all the above.

20. Pick up the correct statement from the following:

a. Angular and rough aggregates reduce the workability of the concrete

b. Large size aggregates increase the workability due to lesser surface area

c. The slump of the concrete mix decreases due to an increase in temperature

d. All the above.

21. The lower water cement ratio in concrete, introduces

a. greater density and smaller permeability

b. improved frost resistance

c. greater wear resistance and improved bond strength

d. all the above.

22. The void ratio of

a. single size coarse aggregate is roughly 0.45.

b. graded coarse aggregate is roughly 0.040

c. fine aggregate is roughly 0.45

d. all the above.

23. The internal dimensions of a ware house are 15 m x 5.6 m, and the maximum height of piles is 2.70 m, the maximum number of bags to be stored in two piles, are

a. 2000 bags

b. 2500 bags

c. 3000 bags

d. 4000 bags

24. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

25. If dry density, water density and specific gravity of solids of a given soil sample are 1.6 g/cc, 1.84 g/cc and 2.56 respectively, the porosity of the soil sample, is

a. 0.375

b. 0.370

c. 0.380

d. 0.390

26. The minimum depth of building foundations on

a. sandy soils is 80 cm to 100 cm

b. clay soils is 90 cm to 160 cm

c. rocky soils is 5 cm to 50 cm

d. all the above.

27. Maximum size of clay particles, is :

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm

d. 0.08 mm

28. The fundamental equation of specific gravity (G), dry density (γd), unit weight of water (γω) and void ratio (e), is

a. e =

b. G =

c. γd =

d. γω =

29. The intensity of active earth pressure at a depth of 10 metres in dry cohesionless sand with an angle of internal friction of 30° and with a weight of 1.8 t/m3, is

a. 5 t/m2

b. 6 t/m2

c. 7 t/m2

d. 8 t/m2

30. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

31. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

32. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

33. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

34. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

35. Optimistic time, most likely time and pessimistic times for the activities of a network in the given figure are written above their arrows. If the contractual obligation time for the project is 75, the latest occurrence time for the event 2, is

a. 20

b. 25

c. 35

d. 15

36. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

37. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

38. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

39. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

40. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

41. The edge of drawing board where T-square is made to slide is called

a. sliding edge

b. fixing edge

c. working edge

d. all of the above

42. Which pencil is used for thinner line?

a. H

b. B

c. 2B

d. 2H

43. Third angle projection is the traditional ............. system.

a. American

b. British

c. Indian

d. Nepali

44. The projection of any feature on the horizontal plane is called

a. plan

b. elevation

c. map

d. all of the above

45. The more critical (or severe) test of the firm's liquidity can be judged by :

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. In the cash-flow diagram shown in the given figure

a. Equal deposits of Rs 3000 per year (A) are made, starting now.

b. The rate of interest is 10% per yearaccount

c. The amount accumulated after the seventh deposit is to be computed

d. All of these

47. The construction manager uses the estimate of the project

a. to advise the architect/engineer regarding design cost parameter especially in value engineering analysis.

b. to develop bids on the project.

c. to control the project during its construction.

d. All of these

48. The minimum qulification & experience required for the registration of foreigner engineers is

a. bacheclor in engg. and 5 years experience

b. bachelor in engg. and 10yrs experience

c. master in engg. and 5 yrs experience

d. master in engg. and 10yrs experience

49. Ethics as a normative science needs code of conduct and guidelines to maintaining high level of standard of good behavior in the society. This is applicable to :

a. Lawers

b. doctors

c. Engineers

d. all of the above

50. Which is the professional body of architectural and engineering firms is Nepal?

a. Society of Nepalese Architects (SONA)

b. Society of Public Health Engineers (SOPHEN)

c. Sociality of Consulting Engineering and Architectural firms

d. None of above