***Engineer***

1. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

2. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

3. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

4. A beam is said to be of uniform strength, if

a. B.M. is same throughout the beam

b. shear stress is same throughout the beam

c. deflection is same throughout the beam

d. bending stress is same at every section along its longitudinal axis.

5. If the width b and depth d of a beam simply supported with a central load are interchanged, the deflection at the centre of the beam will be changed in the ratio of

a. d/b

b. (d/b)2

c. (b/d)2

d. (b/d)3.

6. The included angles of a theodolite traverse, are generally measured

a. clockwise from the forward station

b. anti-clockwise from the back station

c. anti-clockwise from the forward station

d. clockwise from the back station.

7. In a theodolite

a. the telescope axis is perpendicular to transit axis

b. the axis of rotation is perpendicular to transit axis

c. the telescope axis, the transit axis and the rotation axis pass through the centre of theodolite

d. all the above.

8. In case of a direct vernier scale

a. graduations increase in opposite direction in which graduations of the main scale increase

b. smallest division is longer than smallest division of the main scale

c. graduations increase in the same direction in which graduations of the main scale increase

d. none of these.

9. Number of subdivisions per metre length of a levelling staff is

a. 100

b. 200

c. 500

d. 1000

10. Magnetic bearing of a survey line at any place

a. remains constant

b. changes systematically

c. varies differently in different months of the year

d. is always greater than true bearing.

11. In a perfect prismatic compass

a. ends of the needle and pivot are in same vertical and horizontal planes

b. pivot is vertically over the centre of the graduated circle

c. needle is always kept sensitive

d. all the above.

12. If a linear traverse follows a sharp curve round a large lake where it is difficult to have long legs, the accuracy of the traverse may be improved by

a. taking short legs

b. making repeated observations of angular and linear measurements

c. making a subsidiary traverse to determine the length of a long leg

d. all the above.

13. Asphalt is obtained from

a. petroleum distillation

b. bitumen distillation

c. plastic distillation

d. none of these.

14. The rocks which are formed due to cooling of magma at a relatively shallow depth from the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Ignoeous rocks.

15. The cement becomes unsound by the presence of excess

a. sulphur

b. magnesia

c. lime

d. All of these.

16. Porcelain is used as :

a. electric insulators

b. storage vessels

c. reactor chambers

d. All of the above.

17. The low voltage porcelain is mainly used for

a. switch block

b. insulating tubes

c. lamp sockets

d. All of these.

18. Slump test for concrete is carried out, to determine

a. strength

b. durability

c. workability

d. water content.

19. Pick up the correct statement from the following:

a. Concrete using sand stones, cracks due to excessive shrinkage

b. Very hard and close grained crystallined lime stones are suitable aggregates but provide low strength

c. Broken bricks produce a concrete having good fire resisting qualities

d. All the above.

20. C.R.R.I. charts are used to obtain a relatioship between strength of concrete and

a. water cement ratio

b. workability

c. grading of aggregate

d. fineness modulus

21. Pick up the incorrect statement from the following:

a. Tricalcium silicate (C3S) generates more heat of hydration

b. Tricalcium silicate (C3S) develops early strength

c. Tricalcium silicate (C3S) has more resistance to sulphate attack

d. None of these.

22. After casting, an ordinary cement concrete on drying

a. expands

b. mix

c. shrinks

d. none of these.

23. Saw dust can be rendered chemically inert by boiling it in water containing

a. ferrous sulphate

b. potassium chloride

c. ammonia

d. nitric acid

24. A coarse-grained soil has a voids ratio 0.75, and specific gravity as 2.75. The critical gradient at which quick sand condition occurs, is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

25. The zero atmospheric pressure is at

a. mean sea level

b. water table

c. phreatic surface

d. both (c) and (d) of the above.

26. In a liquid limit test, the moisture content at 10 blows was 70% and that at 100 blows was 20%. The liquid limit of the soil, is

a. 35%

b. 50%

c. 65%

d. none of these.

27. 'Talus' is the soil transported by

a. wind

b. water

c. glacier

d. gravitational force.

28. The general relationship between specific gravity (G), weight of water (γω), degree of saturation (Sr), void ratio (e) and bulk density (γ), is

a. γ =

b. γ =

c. γ =

d. γ =

29. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

30. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

31. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

32. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

33. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

34. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

35. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

36. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

37. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

38. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

39. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

40. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

41. The isometric length of the edge of the cube is

a. more than actual length

b. qual to actual length

c. less than actual length

d. all of the above

42. Measurements from scale to the drawing are transferred with the aid of 19

a. scale

b. compass

c. divider

d. plumbing fork

43. In Engineering Drawing, French Curves mean :

a. the shape of an hourglass

b. drafting tools used to draw non-linear figures

c. drafting tools used to draw oblong figures

d. helical designs patented in France

44. Vertical lines are drawn

a. top to bottom

b. bottom to top

c. both (a) and (b) above

d. none of the above

45. The person desires to pay off the amount in 10 equal annual instalments. The amount of each instalment is :

a. Rs 5638

b. Rs 6638

c. Rs 7738

d. None of these

46. If a is the base amount expenditure, b is the increase in the operation cost each year over a period of n years, the total cost of maintenance is :

a. a + (n + 1) b

b. a + (n - 1) b

c. a x (n - 1) b

d. a - (n - 1) b

47. If P is principal amount, i is the rate of interest and n is the number of periods in years, then the interest factor is :

a. (1 + ni)

b. (ni - 1)

c. ni

d. None of these

48. Profession is defined as

a. systematic knowledge through specialized training

b. synonym to job or occupation

c. types of service to needy people

d. all of above

49. Who is the Chairperson of Project Co-ordination Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

50. "Fraudulent practice” means

a. offering anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above