***Engineer***

1. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

2. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

3. In the truss, the force in the member AC is

a. 6.25 t compressive

b. 8.75 t tentile

c. t tensile

d. t compressive.

4. During a tensile test on a ductile material

a. nominal stress at fracture is higher than the ultimate stress

b. true stress at fracture is higher than the ultimate stress

c. true stress at fracture is the same as the ultimate stress

d. none of these.

5. When a rectangular beam is loaded longitudinally, shear develops on

a. bottom fibre

b. top fibre

c. middle fibre

d. every-horizontal plane.

6. Stadia techeometry was discovered by James Watt in the year.

a. 1670

b. 1770

c. 1870

d. 1900.

7. The whole circle bearing of a line is 290°. Its reduced bearing is

a. N 20° E

b. N 20° W

c. N 70° W

d. S 70° E

8. In a closed traverse, sum of south latitudes exceeds the sum of north latitudes and the sum of east departures exceeds the sum of west departures, then, the closing line will lie in

a. north-west quadrant

b. north east quadrant

c. south-east quadrant

d. south-west quadrant.

9. While measuring the distance between two points along upgrade with the help of a 20 m chain, the forward end of the chain is shifted forward through a distance

a. 20 (sin θ - 1)

b. 20 (cos θ - 1)

c. 20 (sec θ - 1)

d. 20 (cosec θ - 1).

10. The area of any irregular figure of the plotted map is measured with

a. sextant

b. clinometer

c. planimeter

d. optical square

11. The line of collimation method of reduction of levels, does not provide a check on

a. intermediate sights

b. fore sights

c. back sights

d. reduced levels.

12. If the whole circle bearing of a line is 270°, its reduced bearing is

a. N 90° W

b. S 90° W

c. W 90°

d. 90° W.

13. The clay to be used for manufacturing bricks for a large project, is dugout and allowed to weather throughout

a. the monsoon

b. the winter

c. the summer

d. none of these.

14. Pick up the correct statement regarding low heat cement from the following:

a. Its initial setting time is about one hour

b. Its final setting time is about 10 hours

c. Its mainly used for mass concrete work

d. All the above.

15. Chemically, marble is known as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

16. Pick up the hypabyssal rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

17. Veneering means

a. carving out designs on timber planks

b. chemically treating timber planks

c. thick   layer   of superior wood glued to inferior wood

d. thin   layer   of superior   wood glued to inferior wood.

18. Pozzolana (or surkhi) is used in lime

a. to impart hydraulicity

b. to prevent shrinkage

c. to decrease the cost of construction

d. to decrease the setting time.

19. Concrete mainly consists of

a. aggregates

b. admixture

c. water

d. all the above.

20. The risk of segregation is more for

a. wetter mix

b. larger proportion of maximum size aggregate

c. coarser grading

d. all the above.

21. The preliminary test is repeated if the difference of compressive strength of three test specimens, exceeds

a. 8 kg/cm2

b. 10 kg/cm2

c. 12 kg/cm2

d. 15 kg/cm2

22. To obtain a very high strength concrete, use very fine grained

a. Granite

b. Magnetite

c. Barite

d. Volcanic scoria.

23. For the construction of cement con-certe dams, the maximum permissible size of the aggregates, is

a. 40 mm

b. 50 mm

c. 60 mm

d. 70 mm

24. Pick up the correct statement applicable to plate load test

a. At the centre of the test pit, a hole is dug out whose size is kept equal to the size of the test plate

b. Bottom level of the hole dug at the centre of the test pit, is kept at the level of the actual formation

c. Ratio of the depth of the hole to the width of the test plate is the same as the ratio of the depth of the actual formation to the width of the formation

d. All the above.

25. The maximum shear stress occurs on the filament which makes an angle with the horizontal plane equal to

a. 30°

b. 45°

c. 60°

d. 90°

26. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

27. Pick up the correct statement from the following:

a. At critical hydraulic gradient, the saturated sand becomes quick

b. The critical gradient depends on the void ratio and the specific gravity

c. The quick sand occurs more in fine sand and silt than coarse material

d. All the above.

28. The maximum load carried by a pile, when it continues to sink without further increase of load, is known as

a. ultimate load carrying capacity

b. ultimate bearing capacity

c. ultimate bearing resistant

d. all the above.

29. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

30. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

31. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

32. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

33. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

34. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

35. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

36. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

37. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

38. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

39. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

40. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

41. A 100 m long line making 30° with the vertical, show its projection on horizontal plane

a. 100 m

b. 50 m

c. 86.6 m

d. 70.71 m

42. Concept drawing is normally provided for the work of

a. tender

b. construction

c. revise estimate

d. all of the above

43. Vertical lines are drawn

a. top to bottom

b. bottom to top

c. both (a) and (b) above

d. none of the above

44. Mechanical lines are types of

a. mechanic drawing

b. line of drawn with free hand

c. line drawn with drawing equipment

d. none of the above

45. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

46. The ratio of current assests to current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

47. Annuities involve:

a. all payments of equal amount

b. payment at equal time intervals

c. payments at the end of periods.

d. All of these

48. Engineering shall act in such a manner as to uphold and enhance the honor, and dignity of the engineering profession and shall act with ..... for bribery, fraud, and corruption.

a. a tolerance

b. a penchant

c. zero tolerance

d. a predilection

49. Fundamental principle of Professional Engineering Ethics is:

a. uploading and advancing Engineering Profession

b. maintain high level of ethical conduct

c. both of the above

d. none of the above

50. Prequilification is not required for any types of firm if the costing to buy an equipment is

a. less than 20 million

b. more than 20 million

c. any amount

d. all of the above