***Engineer***

1. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

2. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

3. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

4. The ratio of the tensile stress developed in the wall of a boiler in the circumferential direction to the tensile stress in the axial direction, is

a. 4

b. 3

c. 2

d. 1

5. A shaft turning 150 r.p.m. is subjected to a torque of 150 kgm. Horse power transmitted by the shaft is

a. π

b. 10 π

c. π2

d. 1/π

6. A transit is oriented by setting its vernier A to read the back azimuth of the preceding line. A back sight on the preceding transit station taken and transit is rotated about its vertical axis. The vernier A reads

a. azimuth of the forward line

b. bearing of the. forward line

c. back bearing of the forward line

d. equal to 360°-azimuth of the forward line.

7. The 'fix' of a plane table from three known points, is good, if

a. middle station is nearest

b. middle station is farthest

c. either the right or left station is nearest

d. none of these.

8. Imaginary line passing through points having equal magnetic declination is termed as

a. isogon

b. agonic line

c. isoclinic line

d. none of these.

9. In chain surveying field work is limited to

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. all the above.

10. The diaphragm of a stadia theodolite is fitted with two additional

a. horizontal hairs

b. vertical hairs

c. horizontal and two vertical hairs

d. none of these.

11. One of the Lehmann's rules of plane tabling, is

a. location of the instrument station is always distant from each of the three rays from the known points in proportion to their distances

b. when looking in the direction of each of the given points, the instrument station will be on the right side of one and left side of the other ray

c. when the instrument station is outside the circumscribing circle its location is always on the opposite side of the ray to the most distant point as the inter-section of the other two rays

d. none of these.

12. Offsets are measured with an accuracy of 1 in 40. If the point on the paper from both sources of error (due to angular and measurement errors) is not to exceed 0.05 cm on a scale of 1 cm = 20 m, the maximum length of offset should be limited to

a. 14.14

b. 28.28 m

c. 200 m

d. none of these.

13. If the ore impurities is

a. clay, lime stone is used as flux

b. lime stone, clay is used as flux

c. quartz, lime stone and argallaceous iron ores are used as flux

d. All the above.

14. The softest rock is

a. marble

b. diamond

c. talc

d. quartz.

15. The rocks in which argil (or clay) predominates, are called

a. sillicious rocks

b. argillaceous rocks

c. calcareous rocks

d. igneous rocks.

16. The clay to be used for manufacturing bricks for a large project, is dugout and allowed to weather throughout

a. the monsoon

b. the winter

c. the summer

d. none of these.

17. Inhaling of fly-ash over a long period causes

a. fibrosis of lungs

b. bronchitis

c. pneumonitis

d. All of these.

18. Index number expressing the relative sizes of both coarse and fine aggregates, is called

a. proportioning of aggregates

b. fineness modulus

c. grading of aggregates

d. none of these.

19. According to the recommendations of IS : 456-1978, the expansion joints

a. are provided to ensure minimum resistance

b. are supported on separate columns

c. do not carry reinforcement across them

d. all the above.

20. The compaction of concrete, improves

a. density

b. strength

c. durability

d. all the above.

21. If 50 kg of fine aggregates and 100 kg of coarse agregates are mixed in a concrete whose water cement ratio is 0.6, the weight of water required for harsh mix, is

a. 10 kg

b. 12 kg

c. 14 kg

d. 15 kg.

22. For a good concrete

a. cement should be sufficient to produce the required strength

b. water should be free from organic materials

c. mixing of ingredients should be done thoroughly so as to produce homogeneity

d. All the above.

23. Pick up the incorrect statement applicable to the field test of good cement.

a. The colour of the cement is bluish

b. A handful of cement thrown into a bucket of water should sink immediately

c. By rubbing cement in between fingers, one should feed rough

d. All the above.

24. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

25. A soil has bulk density 2.30 g/cm3 and water content 15 per cent, the dry density of the sample, is

a. 1.0 g/cm2

b. 1.5 g/cm3

c. 2.0 g/cm3

d. 2.5 g/cm3

26. If G is specific gravity of sand particles, e is porosity, the critically hydraulic gradient

a. ic =

b. ic =

c. ic =

d. ic =

27. The critical exist gradient of seepage water in soils, is

a. inversely proportional to the specific gravity

b. directly proportional to the specific gravity

c. inversely proportional to the voids ratio

d. none of these.

28. Fundamental relationship between dry density (γd), specific gravity (G), water content (ω) and percentage of air voids (na) is :

a. γd =

b. γd =

c. γd =

d. γd =

29. On wetting, cohesive soils,

a. loose permeability

b. gain shear strength

c. loose elasticity

d. decrease their shear strength.

30. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

31. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

32. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

33. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

34. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

35. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

36. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

37. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

38. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

39. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

40. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

41. In a topographic drawing the symbols representing natural features are drawn by free hand while artificial works are drawn by

a. free hand

b. mechanically

c. both (a) and (b) of above

d. none of the above

42. In orthographic projection, the line of sight and plane of projection are

a. perpendicular

b. parallel

c. horizontal

d. vertical

43. The edge of drawing board where T-square is made to slide is called

a. sliding edge

b. fixing edge

c. working edge

d. all of the above

44. During drawing stock of T-square is placed

a. adjoining the working edge of the board

b. adjoining the sliding edge of the board

c. surface of the board,

d. none of the above

45. Pick up the correct statement from the following:

a. The ratio of current assests, loans and advances, and the current liquidity is called current ratio.

b. Larger the current ratio, larger is the margin of safety.

c. The operating profit is the difference between gross profit and operating expenses.

d. All of these

46. Pick up the method used for project evaluation and selection in capital budgetting from the following:

a. Internal ratio of return

b. Net present worth

c. Profitability index

d. All the above

47. The annuity which refers to a debt payment for recovering the initial amount or capital in equal periodical payments, is known as;

a. Present Worth Annuity

b. Sinking fund annuity

c. Compound annuity

d. Capital recovery annuity

48. Which Ministry of Government of Nepal deals with Civil Engineering group of Nepal Engineering service?

a. Ministry of General Administration

b. Ministry of Energy

c. Ministry of Irrigation

d. Ministry of Physical Infrastructure and Transport

49. What percent of interest is taken for the advance amount paid to the contractor by employer?

a. 1% of advance amount

b. 2% of advance amount

c. 3% of advance amount

d. none of the above

50. Which action does not fall under the principle of public procurement

a. procurement should be economic and efficient

b. procurement should be always be lowest cost

c. information and equal opportunities to all

d. procurement process should be transparent