***Engineer***

1. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

2. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

3. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

4. For a cantilever with a uniformly distributed load W over its entire length L, the maximum bending moment is

a.  WL

b.  WL

c.  W2L

d.  WL2

5. A simply supported wooden beam 150 cm long and having a cross section 16 cm x 24 cm carries a concentrated load, at the centre. If the permissible stress ft = 75 kg/cm2 and fs = 10 kg/cm2 the safe load is

a. 3025 kg

b. 3050 kg

c. 3075 kg

d. 3100 kg.

6. The ratio of the angles subtended at the eye, by the virtual image and the object, is known as telescope's

a. resolving power

b. brightness

c. field of view

d. magnification.

7. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum deflection angle to locate its junction point, is

a. 1°43' 08"

b. 1°43' 18"

c. 1°43' 28"

d. 1°43' 38"

8. For high sensivity of the bubble tube

a. a liquid of low surface tension is used

b. the bubble space should be long

c. the bubble tube should not be too narrow

d. all the above.

9. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

10. The tangent to the liquid surface in a level tube, is parallel to the axis of the level tube at

a. every point of the bubble

b. either end of the bubble

c. the mid-point of the bubble

d. no where.

11. Ramsden eye-piece consists of

a. two convex lenses short distance apart

b. two concave lenses short distance apart

c. one convex lens and one concave lens short distance apart

d. two plano-convex lenses short distance apart, with the convex surfaces facing each other.

12. The best method of interpolation of contours, is by

a. estimation

b. graphical means

c. computation

d. all of these.

13. Snowcrete is one of the patent forms of

a. distempers

b. water proof cement paints

c. enamel paints

d. cellulose paints.

14. Age of a tree may be ascertained by

a. radius of its stem

b. circumference of its stem

c. number of branches

d. number of annual rings.

15. Asbestos

a. is a natural fibrous mineral substance

b. is composed of hydrous silicates of calcium and magnesium (CaSiO3, 3MgSiO3)

c. contains iron oxide and alumina

d. all the above.

16. Based on flow quality, the sequence of pipes is

a. A.C. pipes, G.I. pipes, C.I. pipes, PVC pipes

b. C.I. pipes, G.I. pipes, A.C. pipes, PVC pipes

c. C.I. pipes, G.I. pipes, PVC pipes, A.C. pipes

d. None of these.

17. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

18. The commonly used raw material in the manufacture of cement, is

a. slate

b. sand stone

c. lime stone

d. basalt.

19. The process of mixing, transporting, placing and compacting concrete using Ordinary Port land Cement should not take more than

a. 30 minutes

b. 40 minutes

c. 60 minutes

d. 75 minutes

20. Pick up the correct statement from the following:

a. Water and aggregates are measured in litres

b. The finished concrete is measured in cubic metres

c. 20 bags of cement make one tonne

d. All the above.

21. An aggregate is said to be flaky if its least dimension is less than

a. 2/5th of mean dimension

b. 3/5th of mean dimension

c. 4/5th of mean dimension

d. none of these.

22. Water required per bag of cement, is

a. 14 kg

b. 21 kg

c. 28 kg

d. 35 kg

23. The standard sand now a days used in India, is obtained from

a. Jullundur (Punjab)

b. Hyderabad (Andhra Pradesh)

c. Ennore (Madras)

d. Cuttuck (Orissa).

24. The Mohr's straight theory is based on the following fact :

a. Material fails essentially by shear

b. Ultimate strength of the material is determined by the stress in the plane of slip

c. Failure criterion is independent of the intermediate principal stress

d. All the above.

25. A decrease in water content results in a reduction of the volume of a soil in

a. liquid state

b. plastic state

c. semi solid state

d. all of these.

26. The liquid limit and plastic limit exist in

a. sandy soils

b. silty soils

c. gravel soils

d. clay soils.

27. The consistency index of a soil is defined as the ratio of

a. liquid limit plus the natural water content to the plasticity index of the soil

b. liquid limit minus the natural water content to the plasticity index of the soil

c. natural water content of a soil minus plastic limit to the plasticity index of the soil

d. natural water content of a soil plus its plastic limit to the plasticity index of the soil.

28. The neutral stress in a soil mass is

a. force per neutral area

b. force per effective area

c. stess taken up by the pore water

d. stress taken up by solid particles.

29. The pressure that builds up in pore water due to load increment on the soil, is termed

a. excess pore pressure

b. excess hydrostatic pressure

c. hydrodynamic pressure

d. all the above.

30. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

31. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

32. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

33. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

34. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

35. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

36. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

37. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

38. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

39. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

40. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

41. In one plane drawing, which one does not falls under

a. orthographic

b. axonometric

c. oblique

d. dimetric

42. The other name of axonometric projection is

a. one plane projection

b. two plane projection

c. three plane projection on

d. all of the above

43. Which is not pictorial projection?

a. oblique projection

b. orthographic projection

c. trignometric

d. axonometric projection

44. The size of the title block of all sizes of drawing sheet is :

a. 420 mm, 65 mm

b. 185 mm, 65 mm

c. 297 mm, 65 mm

d. 185 mm, 50 mm

45. Pick up the method used for project evaluation and selection in capital budgetting from the following:

a. Internal ratio of return

b. Net present worth

c. Profitability index

d. All the above

46. The annuity which refers to a debt payment for recovering the initial amount or capital in equal periodical payments, is known as;

a. Present Worth Annuity

b. Sinking fund annuity

c. Compound annuity

d. Capital recovery annuity

47. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

48. For the registiration of professional engineer amount fee required is:

a. NRS 5000/-

b. NRS 10000/-

c. NRS 15000/-

d. NRS 20000/-

49. Which of the following an engineer should not follow:

a. act as faithful agent for client

b. accept compensation from more than one party for same service without the consent of all

c. indicate client the adverse consequences if judgment is over ruled

d. undertake only these assignments for which he is qualified

50. The Nepal Engineers' Association was established in

a. 2014 BS

b. 2024 BS

c. 2034 BS

d. 2044 BS