***Engineer***

1. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4

2. A masonry dam (density = 20, 000 N/m3) 6 m high, one metre wide at the top and 4 m wide at the base, has vertical water face. The minimum stress at the base of the dam when the reservoir is full, will be

a. 75 N/m2

b. 750 N/m2

c. 7500 N/m2

d. 75000 N/m2

3. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

4. The equivalent length of a column fixed at one end and free at the other end, is

a. 0.7 l

b. l

c. 2 l

d. 1.5 l.

5. In a solid arch, shear force acts

a. along the axis of the arch

b. perpendicular to the axis of arch

c. tangentially to the arch

d. none of these.

6. There are two stations A and B. Which of the following statements is correct :

a. the fore bearing of AB is AB

b. the back bearing of AB is BA

c. the fore and back bearings of AB differ by 180°

d. all the above.

7. The additional lines which are measured to show the correctness of the chain surveying are called:

a. check clines

b. proof lines

c. tie lines

d. all of these.

8. The line of collimation method of reduction of levels, does not provide a check on

a. intermediate sights

b. fore sights

c. back sights

d. reduced levels.

9. ABCD is a rectangular plot of land. If the bearing of the side AB is 75°, the bearing of DC is

a. 75°

b. 255°

c. 105°

d. 285°

10. The bearing of lines OA and OB are 16° 10' and 332° 18', the value of the included angle BOA is

a. 316° 10'

b. 158° 28'

c. 348° 08'

d. 43°  52'

11. If i is the stadia distance, f is the focal length and d is the distance between the objective and vertical axis of the techeometer, the multiplying constant, is

a. f/i

b. i/f

c. (f + d)

d. f/d.

12. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

13. A volatile substance added to a paint to make its application easy and smooth, is known as

a. base

b. solvent

c. vehicle

d. none to these.

14. Lacquer paints

a. are less durable as compared to enamel paints

b. consist of resin and nitro-cellulose

c. contain alcohol as thinner

d. all the above.

15. In a rock calcium carbonate predominates. State whether it is :

a. Sillicious rock

b. Argillaceous rock

c. Calcareous rock

d. None of these.

16. The proportions of charcoal, saltpetre and sulphur in gun powder by weight, are respectively:

a. 15, 75, 10

b. 75, 10, 15

c. 10, 15, 75

d. 10, 75, 15.

17. The property by virtue of which lime sets under water, is known as

a. slacking

b. setting

c. hydraulicity

d. calcining.

18. Linseed oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

19. Pick up the correct statement from the following:

a. Insufficient quantity of water makes the concrete unworkable

b. Excess quantity of water makes the concrete segregated

c. Excess quantity of water causes bleeding in concrete

d. All the above.

20. Construction joints are generally provided in concrete

a. retaining walls

b. lining of canals

c. lining of tunnels

d. all the above.

21. Workability of concrete for a given water content is good if the aggregates, are

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregates.

22. If 20 kg of coarse aggregate is sieved through 80 mm, 40 mm, 20 mm, 10 mm, 4.75 mm, 2.36 mm, 1.18 mm, 600 micron, 300 micron and 150 micron standard sieves and the weights retained are 0 kg, 2 kg, 8 kg, 6 kg, 4 kg respectively, the fineness modulus of the aggregate, is

a. 7.35

b. 7.40

c. 7.45

d. none of these.

23. M10 grade of concrete approximates

a. 1 : 3 : 6 mix

b. 1 : 1 : 2 mix

c. 1 : 2 : 4 mix

d. 1 : 1.5 : 3 mix

24. Through a point in a loaded soil mass, there exists n typical planes mutually orthogonal on which the stress is wholly normal and no shear stress acts, if n is

a. 1

b. 2

c. 3

d. 4

25. Plasticity index is defined as the range of water content between

a. liquid and plastic limit

b. plastic limit and semi solid limit

c. semi-solid limit and liquid limit

d. liquid limit and solid limit.

26. Depending upon the properties of a material, the failure envelope may

a. be either straight or curved

b. pass through the origin of stress

c. intersect the shear stress axis

d. all the above.

27. Pick up the correct statement from the following:

a. Illite bond is stronger than montmorillonite bond

b. Illites do not swell when wet

c. Illites are composed of two silica tetrahedralsheets with a central octahedral sheet

d. All the above.

28. Water formed transported soil is

a. alluvial

b. marine

c. lacustrine

d. loess.

29. A clay subjected to pressure in excess to its present over-burden, is said to be

a. pre-compressed

b. pre-consolidated

c. over-consolidated

d. all the above.

30. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

31. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

32. Optimistic time, most likely time and pessimistic times for the activities of a network in the given figure are written above their arrows. If the contractual obligation time for the project is 75, the latest occurrence time for the event 2, is

a. 20

b. 25

c. 35

d. 15

33. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

34. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

35. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

36. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

37. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

38. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

39. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

40. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

41. In engineering drawing, auxiliary views are used to show details of

a. incline surface

b. horizontal surface

c. vertical surface

d. none of the above

42. Circles of very small radii are drawn by means of a

a. straight bar

b. bow compass

c. lengthening bar

d. drop compass

43. Engineers prefer to make ......... sketches than to pictorial sketches to clear the shape of an object

a. orthographic

b. perspective

c. auto CAD

d. all of the above

44. Axonometric projection is a special types of

a. orthographic projection

b. perspective projection

c. isometric projection

d. multi view projective

45. A project construction cost estimate includes:

a. the labour and material cost

b. the equipment and over head cost

c. the profit of the contractor

d. All of these

46. Pick up the correct statement from the following:

a. Both the numerator and denominator of financial ratios come directly from the balance sheet.

b. Income statement ratios compare one 'flow' item from the income statement to another flow item from the income statement.

c. Income statement ratios compare a flow item from the income statement to another flow item form the income statement

d. All of these

47. Which one of the following definitions, is correct ?

a. The ratio of total debt to share holder's equity is called 'debt ratio'.

b. The ratio debt-to-total assests is called Debt-to-total assest ratio.

c. The ratio of earnings before interest and taxes for a particular reporting period to the amount of interest charges for the period, is called interest coverage ratio.

d. All of these

48. Which Ministry of Government of Nepal deals with Civil Engineering group of Nepal Engineering service?

a. Ministry of General Administration

b. Ministry of Energy

c. Ministry of Irrigation

d. Ministry of Physical Infrastructure and Transport

49. Prequalification is done

a. after the invitation of bids

b. before the invitation of bids

c. during the invitation of bids

d. none of the above

50. In Nepal, which of the following Act contains the provisions o health and safety of the workers ?

a. contract act

b. Industry act

c. labour act

d. all of the above