***Engineer***

1. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

2. The yield moment of a cross section is defined as the moment that will just produce the yield stress in

a. the outer most fibre of the section

b. the inner most fibre of the section

c. the neutral fibre of the section

d. the fibre everywhere

3. The ratio of shear stress and shear strain of an elastic material, is

a. Shear Modulus

b. Young's Modulus

c. Modulus of Elasticity

d. both (a) and (b).

4. The law which states, "within elastic limits strain produced is proportional to the stress producing it", is known as

a. Stress law

b. Hooke's law

c. Poisson's law

d. none of these.

5. The maximum twisting moment a shaft can resist, is the product of the permissible shear stress and

a. moment of inertia

b. polar moment of inertia

c. polar modulus

d. modulus of rigidly.

6. The radius of a simple circular curve is 300 m and length of its specified chord is 30 m. The degree of the curve is

a. 5.73°

b. 5.37°

c. 3.57°

d. 3.75°.

7. The distance between steps for measuring down hill to obtain better accuracy

a. decreases with decrease of slope

b. increases with increase of slope

c. decreases with increase of slope

d. decreases with decrease of weight of the chain.

8. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

9. Total latitude of a point is positive if it lies

a. north of the reference parallel

b. south of the reference parallel

c. east of the reference parallel

d. west of the reference parallel.

10. The zero of the graduated circle of a prismatic compass is located at

a. north end

b. east end

c. south end

d. west end.

11. The longitudinal section of the surface of bubble tube is

a. straight

b. circular

c. parabolic

d. elliptic.

12. The angle of intersection of a contour and a ridge line, is

a. 30°

b. 45°

c. 60°

d. 90°.

13. Vanadium steel is generally used for

a. railway switches and crossing

b. bearing balls

c. magnets

d. axles and springs.

14. Pick up the metal refractory from the following:

a. Molyblendum

b. Tungsten

c. Zicronium

d. All of these.

15. Bullet proof glass is made of thick glass sheet sandwiched by a layer of

a. steel

b. stainless steel

c. high test plastic

d. chromium plate.

16. Asbestos

a. is a natural fibrous mineral substance

b. is composed of hydrous silicates of calcium and magnesium (CaSiO3, 3MgSiO3)

c. contains iron oxide and alumina

d. all the above.

17. The size of mould for bricks, is generally kept

a. a little large to specified size

b. a little small to specified size

c. equal to specified size

d. 10% larger than specified size

18. The coefficient of hardness of stones used in road work should be greater than

a. 10

b. 12

c. 15

d. 17.

19. Slump test of concrete is a measure ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

20. For road pavements, the cement generally used, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. none of these.

21. For a concrete mix 1:3:6 and water cement ratio 0.6 both by weight, the quantity of water required per bag, is

a. 12 kg

b. 14 kg

c. 16 kg

d. 20 kg

22. For the construction of thin R.C.C. structures, the type of cement to be avoided, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. sulphate resisting cement.

23. Slump test of concrete is a meausre ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

24. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

25. Pick up the correct statement from the following:

a. The porosity of a soil is defined as the ratio of the volume of voids to the gross volume of the soil

b. The bulk density of a soil is defined as the unit weight of the soil

c. The dry density of a soil is defined as weight of solids to the total volume of the soil

d. All the above.

26. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

27. The clay soil mainly consists of

a. Montomorillonite

b. Illites

c. Vermiculite

d. All the above.

28. The ratio of the difference between the void ratio of the soil in its loosest state and its natural void ratio (e) to the difference between the void ratios in the loosest and fully dense state, is generally termed as

a. degree of density

b. relativity

c. density index

d. all the above.

29. Pick up the correct statement from the following:

a. The ratio of the liquid limit minus the natural water content to the plasticity index of soils, is called consistency index

b. The ratio of natural water content minus its plastic limit to its plasticity index is called liquidity index

c. The ratio between plasticity index and flow index (i.e. slope of flow curve in case of liquid limit), is called toughness index

d. All the above.

30. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

31. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

32. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

33. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

34. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

35. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

36. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

37. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

38. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

39. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

40. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

41. The letter 'H' lebelled in a Drawing Pencil indicates -

a. relative hardness

b. thickness of line

c. both of above

d. none of the above

42. The solid which is generated by the revolution of a right angle triangle about one of its perpendicular sides which remains fixed is called ....

a. a prism

b. a pyramid

c. a cylinder

d. a cone

43. The shades and shadows are :

a. the same

b. the different

c. the result of object on the ground

d. the result of light on the ground

44. Angles in multiples of 15 degree are constructed by the combined use of

a. T-square and set-square

b. set-square and scale

c. scale and french curve

d. french curve and erasing shield

45. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

46. In a cash flow series :

a. uniform gradient signifies that an income or disbursement changes by the same amount in each interest period.

b. Either an increase or a decrease in the amount of a cash flow is called the gradient.

c. The gradient in the cash flow may be positive or negative.

d. All of these

47. The annuity which refers to a debt payment for recovering the initial amount or capital in equal periodical payments, is known as;

a. Present Worth Annuity

b. Sinking fund annuity

c. Compound annuity

d. Capital recovery annuity

48. The purpose of bid evaluation is to determine

a. lowest bid

b. responsive bid

c. substantially responsive bid

d. lowest evaluated substantially responsive bid

49. During construction price variation clause in a contract applies to

a. increase in cost of important materials only

b. variation in the cost of material labour and fuel

c. variation in total cost of the project estimated as lump sum figure

d. the rate of inflation

50. The Nepal Engineers' Association was established in

a. 2014 BS

b. 2024 BS

c. 2034 BS

d. 2044 BS