***Engineer***

1. In plastic analysis, the shape factor for a triangular section, is

a. 1.5

b. 1.34

c. 2.34

d. 2.5

2. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

3. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

4. A solid cube is subjected to equal normal forces on all its faces. The volumetric strain will be x-times the linear strain in any of the three axes when

a. x = 1

b. x = 2

c. x = 3

d. x = 4

5. The neutral axis of a beam cross-section must

a. pass through the centroid of the section

b. be equidistant from the top of bottom films

c. be an axis of symmetry of the section

d. none of these.

6. The staff intercept will be

a. greater farther off the staff is held

b. smaller, farther off the staff is held

c. smaller, nearer the staff is held

d. same, wherever the staff is held.

7. The zero of the graduated circle of a prismatic compass is located at

a. north end

b. east end

c. south end

d. west end.

8. Pick up the correct statement from the following :

a. mistakes arise from inattention, inexperience or carelessness

b. systematic errors persist and have regular effects in the survey performances

c. accidental errors occur inspite of every precaution is taken

d. all the above.

9. For a curve of radius 100 m and normal chord 10 m, the Rankine's deflection angle, is

a. 0°35'.95

b. 1°25'.53

c. 1°35'.95

d. 2°51'.53.

10. In tacheometrical observations, vertical staff holding is generally preferred to normal staffing, due to

a. ease of reduction of observations

b. facility of holding

c. minimum effect of careless holding on the result

d. none of these.

11. Cross-staff is used for

a. setting out right angles

b. measuring contour gradient

c. taking levels

d. measuring distances

12. Mistakes which may produce a very serious effect upon the final results arise due to

a. in attention

b. in experience

c. carelessness

d. all of these.

13. If the iron ore contains clay as an impurity, the flux added during calcination, is

a. clay

b. lime stone

c. argillaceous iron ore

d. all the above.

14. Kaolin is chemically classified as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

15. The cement becomes unsound by the presence of excess

a. sulphur

b. magnesia

c. lime

d. All of these.

16. Bitumen felt is used for

a. water proofing

b. damp proofing

c. both (a) and (b)

d. neither (a) nor (b).

17. The commonly used base for iron and steel work, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

18. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

19. The grade of concrete M 150 means that compressive strength of a 15 cm cube after 28 days, is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

20. Placing of concrete should preferably be done at a temperature of

a. 10°C

b. 20°C

c. 23°C

d. 27 ± 2°C

21. Pozzolana cement is used with confidence for construction of

a. massive foundations

b. abutments

c. R.C.C. structures

d. all the above.

22. The concrete mix which causes difficulty in obtaining a smooth finish, possess

a. internal friction

b. harshness

c. bleeding

d. none of these.

23. For ensuring quality of concrete, use

a. single sized aggegates

b. two sized aggregate

c. graded aggregates

d. coarse aggregates.

24. Pick up the correct statement from the following:

a. The ratio of the liquid limit minus the natural water content to the plasticity index of soils, is called consistency index

b. The ratio of natural water content minus its plastic limit to its plasticity index is called liquidity index

c. The ratio between plasticity index and flow index (i.e. slope of flow curve in case of liquid limit), is called toughness index

d. All the above.

25. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

26. A soil mass coated with a thin layer of paraffin weighs 460 g. When immersed, it displaces 299 cc of water. The weight of paraffin is 10 g. If specific gravity of solids is 2.5 and that of paraffin 0.9, the void ratio of soil, is

a. 0.55

b. 0.60

c. 0.65

d. 0.70

27. Pick up the correct definition from the following:

a. The lateral pressure exerted by the soil when the retaining wall moves away from the back fill, is generally known as active earth pressure of the soil

b. The lateral pressure exerted by the soil when the retaining wail moves towards the soil, is generally known as 'Passive earth pressure of the soil'

c. The lateral pressure exerted by the soil when the retaining wall has no movement relative to the back fill, is known as 'earth pressure at rest of the soil'

d. All the above.

28. The method of the slices is applicable to

a. homogenous soils

b. stratified soils

c. saturated soils

d. non-uniform slopes

29. The earth pressure of a soil at rest, is proportional to (where φ is the angle of internal friction of the soil)

a. tan (45° + φ)

b. tan2 (45° - φ)

c. tan2 (45° + φ)

d. none of these

30. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

31. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

32. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

33. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

34. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

35. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

36. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

37. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

38. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

39. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

40. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

41. In third angle projection top view is put

a. above front view

b. below font view

c. side view

d. none of the above

42. Dashed (dotted) line is used for indication

a. edge of the object

b. hidden edge of the object

c. centre line of the object

d. all of the above

43. The best drawing paper used for water colour is :

a. hand made paper (hand made drawing sheet)

b. tracing drawing sheet

c. ordinary drawing sheet

d. ammonia paper

44. Pencil of ...... grade sharpened in the form of ......... is used for sketching

a. soft & conical point

b. low and conical point

c. hard & conical point

d. chisel & conical point

45. The key to profitable operation for project cost control, is :

a. To keep the project cost equal to original cost estimate.

b. To keep the project cost equal to subsequent construction budget.

c. To keep the project cost within the cost budget and knowing when and where job costs are deviating.

d. None of these

46. In a cash flow series :

a. uniform gradient signifies that an income or disbursement changes by the same amount in each interest period.

b. Either an increase or a decrease in the amount of a cash flow is called the gradient.

c. The gradient in the cash flow may be positive or negative.

d. All of these

47. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. None of these

48. Engineering ethics is the field of applied ethics and system of moral principles that applt to be

a. practice engineering

b. responsibility engineering

c. management engineering

d. accountability engineering

49. As per the NEC Act, 2055 the name of engineers

a. general, professional, and foreigner

b. special, local and international wngineer

c. civil, electrical and mechanical engineer.

d. computer, electronics and agriculture engineer

50. For the registration of foregineer engineers the amount of fee required is:

a. NRS 3000/-

b. NRS 5000/-

c. NRS 10000/-

d. NRS 15000/-