***Engineer***

1. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

2. Pick up the correct statement from the following:

a. For a uniformly distributed load, B.M. curve is a parabola

b. For a load varying linearly, the shear force curve is a parabola

c. For a load varying linearly, the B.M. curve is a cubic parabola

d. All the above.

3. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

4. Pick up the incorrect statement

a. A welded joint develops strength of its parent metal

b. Welded joints provide rigidity

c. Welded joints have better finish

d. Welding takes more time than riveting.

5. A rectangular beam 20 cm wide is subjected to a maximum shearing force of 10, 000 kg, the corresponding maximum shearing stress being 30 kg/cm2. The depth of the beam is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm.

6. Rankine's deflection angle in minutes is obtained by multiplying the length of the chord by

a. degree of the curve

b. square of the degree of the curve

c. inverse of the degree of the curve

d. none of these.

7. Pick up the correct statement from the following :

a. to locate a gross error in bearing that may exist in controlled theodolite traverse, we may plot the traverse from each end. The traverse station having the same coordinates by each route is the one where the error lies

b. to locate a gross error in bearing, in a controlled traverse, we plot the traverse and the station through which perpendicular to sector of the closing line passes is the station at which the error was made

c. to locate a gross error due to taping in a controlled traverse, we plot the traverse to a convenient scale. The bearing of the closing error will be approximately the same as that of the leg in which the gross error consists

d. all the above.

8. Tilt of the staff in stadia tacheometry increases the intercept if it is

a. away from the telescope pointing down hill

b. towards the telescope pointing up-hill

c. away from the telescope pointing up-hill

d. none of these.

9. The imaginary line passing through the intersection of cross hairs and the optical centre of the objective, is known as

a. line of sight

b. line of collimation

c. axis of the telescope

d. none of these.

10. Two concave lenses of 60 cm focal length are cemented on either side of a convex lens of 15 cm focal length. The focal length of the combination is

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

11. Pick up the correct statement from the following :

a. spherical aberration may be minimised by replacing the single lens by a combination of the lenses.

b. in telescope objectives, a combination of convex lens and concave lens is used.

c. in eyepieces, two plano-convex lenses placed at a certain distance apart are used

d. all the above.

12. Pick up the correct statement from the following :

a. the handles are on swivel joints to prevent twisting of the chain

b. the fifth tag from either end of the chain is numbered 5

c. the length of the Gunter's chain is 66 ft.

d. all the above.

13. The most important constituent of an oil paint, is

a. vehicle

b. pigment

c. base

d. all the above.

14. The most commonly used base for timber painting, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

15. Oil varnish generally consists of

a. synthetic resin and spirit

b. oil, wax and resin

c. resin, oil and turpentine

d. spirit, oil and wax.

16. The wedging is adopted for quarrying costly stratified rock such as

a. marble

b. limestone

c. sandstone

d. All the above.

17. The most commonly used synthetic abrasive is

a. aluminium carbide

b. boric acid

c. silicon

d. All of these.

18. Rapid hardening cement contains

a. Tri-calcium silicate

b. Tri-calcium aluminate

c. Tetra-calcium alumino-ferrite

d. Dicalcium silicate.

19. If the average compressive strength is 4000 kg/cm2 and standard deviation is 500, the co-efficient of variation is

a. 12.5%

b. 15%

c. 18.5 %

d. 20%

20. A construction joint is provided where

a. bending moment is small

b. shear force is small

c. the member is supported by other member

d. all the above.

21. You are asked to construct a massive dam, the type of cement you will use, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. white cement.

22. Pick up the incorrect statement from the following:

a. Cement bags should preferably be piled on wooden planks

b. Cement bags should be placed such that bags of one layer does not touch the bags of the adjacent layer

c. Width and height of the pile should not exceed 3 m and 2.70 m respectively

d. None of these.

23. Slump test of concrete is a measure ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

24. A phreatic line is defined as the line within a dam section below which there are

a. positive hydrostatic pressure

b. negative hydrostatic pressure

c. negative equipotential lines

d. none of these.

25. The liquidity index is defined as a ratio expressed as percentage of

a. plastic limit minus the natural water content, to its plasticity index

b. natural water content minus its plastic limit to its plasticity index

c. natural water content plus its plastic limit to its plasticity index

d. liquid limit minus the natural water content to the plasticity index.

26. The shear strength in plastic undrained clay, is due to

a. inter-granular friction

b. internal friction

c. cohesion

d. none of these.

27. Terzaghi's analysis assumes :

a. soil is homogeneous and isotropic

b. elastic zone has straight boundaries inclined at ψ = φ to the horizontal and plastic zones fully developed

c. failure zones do not extend above the horizontal plane through the base of the footing

d. all the above.

28. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

29. The maximum shear stress occurs on the filament which makes an angle with the horizontal plane equal to

a. 30°

b. 45°

c. 60°

d. 90°

30. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

31. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

32. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

33. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

34. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

35. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

36. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

37. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

38. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

39. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

40. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

41. The other name of axonometric projection is

a. one plane projection

b. two plane projection

c. three plane projection on

d. all of the above

42. Hatching line is a line which makes an angle of 45° with

a. horizontal

b. vertical

c. main line of the section

d. all of the above

43. In orthographic projection, projector lines are

a. vertical lines

b. inclined lines

c. horizontal lines

d. none of above

44. Working edge of drawing board is kept ........... of the user.

a. left

b. right

c. ahead

d. all of the above

45. Which one of the following statements is correct?

a. The number of years required to recover the initial cash investment in a project, is called Pay Back period (PBP).

b. The discount rate that equates the present value of the expected Net Cash Flows (CFs) with the Initial Cash Outflow (ICO) is known as internal rate of return.

c. The present value of the proposal's net cash flows, less the proposal's initial cash outflow is known as the Net Present Value (NPV)

d. All of these

46. If P is principal amount, i is the rate of interest per annum and n is the number of periods in years, the compound amount factor (CAF) is :

a. (1 + i)n

b. (1 + i)(1/2n)

c. (n + i)

d. None of these

47. Earning per share is the most important ratio for

a. share holders

b. banks

c. company's management

d. All of these

48. For the formation of users committee, members are selected

a. among the people of whole district

b. among the beneficiaries of the project

c. by nomination by the chief District officer of the district

d. none of the above

49. The general experience required for nomination of chairperson of NEC is:

a. 10 yrs

b. 15 yrs

c. 20 yrs

d. 25 yrs

50. NEC act and its regulation........

a. 55 and 57

b. 54 and 55

c. 55 and 56

d. 57 and 58