***Engineer***

1. Pick up the correct statement from the following:

a. For a uniformly distributed load, B.M. curve is a parabola

b. For a load varying linearly, the shear force curve is a parabola

c. For a load varying linearly, the B.M. curve is a cubic parabola

d. All the above.

2. The normal and tangential components of stress on an inclined plane through θ° to the direction of the force, will be equal if θ is

a. 45°

b. 30°

c. 60°

d. 90°

3. To determine the force in BD of the truss shown in the given figure a section is passed through BD, CD and CE, and the moments are taken about

a. A joint

b. B joint

c. C joint

d. D joint.

4. A member which does not regain its original shape after removed of load producing deformation is said

a. plastic

b. elastic

c. rigid

d. none of these.

5. When equal and opposite forces applied to a body, tend to elongate it, the stress so produced, is called

a. shear stress

b. compressive stress

c. tensile stress

d. transverse stress.

6. Pick up the wrong statement from the following :

a. the diaphragm is placed between the eyepiece and the objective but nearer to the later

b. the outer component of the objective is a double-convex lens of crown glass

c. the inner component of the objective is a flint glass, convexo-concave

d. all the above.

7. If the radius of a simple curve is 600 m, the maximum length of the chord for calculating offsets, is taken

a. 15 m

b. 20 m

c. 25 m

d. 30 m.

8. If S is the length of a subchord and R is the radius of simple curve, the angle of deflection between its tangent and sub-chord, in minutes, is equal to

a. 573 R/S

b. 171.9 S/R

c. 1718.9 R/S

d. 1718.9 S/R.

9. In horizontal angles, the error due to imperfect levelling of the plate bubble is

a. large when sights are nearly level

b. large for long sights

c. less for steeply inclined sights

d. large for steeply inclined sights.

10. Grid lines are parallel to

a. magnetic meridian of the central point of the grid

b. line representing the central true meridian of the grid

c. geographical equator

d. none of these.

11. A dumpy level was set up at the midpoint between two pegs A and B, 50 m apart and the staff readings at A and B were 1.22 and 1.06. With the level set up a A, the readings at A and B were 1.55 and 1.37. The collimation error per 100 m length of sight is

a. 0.02 m inclined upwards

b. 0.04 m inclined downwards

c. 0.04 m inclined upward

d. none of these.

12. A level when set up 25 m from peg A and 50 m from peg B reads 2.847 on a staff held on A and 3.462 on a staff held on B, keeping bubble at its centre while reading. If the reduced levels of A and B are 283.665 m and 284.295 m respectively, the collimation error per 100 m is

a. 0.015 m

b. 0.030 m

c. 0.045 m

d. 0.060 m

13. Pick up the correct statement from the following:

a. Horn blende mineral is brittle

b. Muscovite is also known as white mica and potashmica

c. Biotite is also known as blackmica

d. All the above.

14. Wrought iron contains carbon upto

a. 0.25%

b. 1.0%

c. 1.5%

d. 2%.

15. Lime stones are generally known as

a. aqueous rocks

b. sedimentary rocks

c. stratified rocks

d. all the above.

16. Steel contains carbon approximately

a. 1.50% to 5.6%

b. 0.05% to 1.75%

c. 0.25%

d. none to these.

17. The softest rock is

a. marble

b. diamond

c. talc

d. quartz.

18. Pick up the correct statement from the following:

a. Cast iron oxidises less

b. Steel oxidises most

c. Wrought iron oxidises moderately

d. All the above.

19. For the construction of R.C.C. slabs, columns, beams, walls, etc. the grade of concrete mix used, is

a. 1 :  : 3

b. 1 : 2 : 4

c. 1 : 1 : 2

d. 1 : 4 : 8.

20. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

21. The commonly used material in the manufacture of cement is

a. sand stone

b. slate

c. lime stone

d. graphite.

22. Slump test is done for

a. clay

b. sand

c. lime

d. concrete.

23. A concrete having a slump of 6.5 cm, is said to be

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

24. The angle of internal friction, is least for

a. angular -grained dense sand

b. round-grained loose sand

c. round-grained loose sand

d. clays.

25. The Mohr's straight theory is based on the following fact :

a. Material fails essentially by shear

b. Ultimate strength of the material is determined by the stress in the plane of slip

c. Failure criterion is independent of the intermediate principal stress

d. All the above.

26. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

27. The void ratio of a soil sample decreases from 1.50 to 1.25 when the pressure is increased from 25 tonnes/m2 to 50 tonnes/m2, the coefficient of compressibility is

a. 0.01

b. 0.02

c. 0.05

d. 0.001

28. You are given a sample of soil containing coarse grains to determine its water content, you will use

a. pycnometer

b. oven-drying method

c. calcium carbide method

d. alcohol method.

29. The phreatic line in an earth dam may be

a. elliptical

b. parabolic

c. a straight line

d. all the above.

30. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

31. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

32. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

33. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

34. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

35. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

36. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

37. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

38. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

39. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

40. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

41. First angle projection method and third angle projection method are used in:

a. axonometric views

b. isometric views

c. orthographic views

d. none of the above

42. The two parts of T-square are....... and .......

a. vertical and horizontal edge

b. straight and slant edge

c. stock and blade

d. blade and batten

43. Free hand drawing is done for

a. generate new idea

b. to make correction

c. both (a) and (b) of above

d. none of the above

44. Number of equal pieces of A4 size paper from A0 is equal to

a. 2

b. 4

c. 8

d. 16

45. The person desires to pay off the amount in 10 equal annual instalments. The amount of each instalment is :

a. Rs 5638

b. Rs 6638

c. Rs 7738

d. None of these

46. Which one of the following is not a construction estimate ?

a. Initial feasibility estimate

b. Conceptual preliminary budget

c. Definite estimate

d. None of these

47. Which one of the following questions is relevant to the construction estimates :

a. Did the estimators precisely evaluate site conditions ?

b. Did the estimators use short cut methods which may be un realistic in their situation ?

c. How much money will the contractor's risk, loosing if he Were to submit bid on the raw estimate of cost.

d. All of these

48. Which one is not concerned with NEC

a. acredition of academic certificate

b. authority of opening engineering college.

c. appointment of teacher

d. all of the above

49. The registered engineer in Nepal Engineering Council should have to renew their registration in:

a. every year

b. every two years

c. every four years

d. none of the above

50. Which of the following is not concerned with the authority of Engineers?

a. good behaviour with client .

b. conscious with his profession

c. divulge the confidential matter

d. express an opinion on engincering matter