***Engineer***

1. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

2. A rolled steel joist is simply supported at its ends and carries a uniformly distributed load which causes a maximum deflection of 10 mm and slope at the ends of 0.002 radian. The length of the joist will be,

a. 12 m

b. 14 m

c. 16 m

d. 18 m

3. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

4. The shear force on a simply supported beam is proportional to

a. sum of the forces

b. sum of the transverse forces

c. algebraic sum of the transverse forces of the section

d. curvature of the neutral axis.

5. If the width of a simply supported beam carrying an isolated load at its centre is doubled, the deflection of the beam at the centre is changed by

a. 1/2

b. 1/8

c. 2

d. 8

6. Under ordinary conditions, the precision of a theodolite traverse is affected by

a. systematic angular errors

b. accidental linear errors

c. systematic linear errors

d. accidental angular errors.

7. An internal focussing type surveying telescope, may be focussed by the movement of

a. objective glass of the telescope

b. convex-lens in the telescope

c. concave lens in the telescope

d. plano-convex lens in the telescope.

8. The least count of a vernier scale is

a. sum of the smallest divisions of main and vernier scales

b. value of one division of the primary scale divided by total number of divisions of vernier scale

c. value of one division of vernier scale divided by total number of divisions of primary scale

d. none of these.

9. Pick up the correct statement from the following :

a. The directions of plumb lines suspended at different points in a survey are not strictly parallel

b. In surveys of small extent, the effect of curvature may be ignored and the level surface of the earth is assumed as horizontal

c. In surveys of large extent, the effect of curvature of the earth must be considered

d. All the above.

10. If d is the distance between equidistant odd ordinates, the Simpson's rule for the areas, is

a. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

b. [h1 + hn x 2(h2 + h4 + ... + hn - 1) + 4(h3 + h5 + ... + hn - 2)]

c. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

d. none of these.

11. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

12. On a diagonal scale, it is possible to read up to

a. one dimension

b. two dimensions

c. three dimensions

d. four dimensions.

13. Index number expressing the relative sizes of both coarse and fine aggregates, is called

a. proportioning of aggregates

b. fineness modulus

c. grading of aggregates

d. none of these.

14. Distemper is

a. a paint consisting of powdered chalk, pigments, and water

b. a water proofing agent

c. a paint consisting of coloured cement and water

d. a drying agent.

15. Percentage content of silica in window glass, is

a. 40 to 45

b. 50 to 55

c. 60 to 65

d. 70 to 75.

16. The initial setting time of hydraulic lime, is

a. 60 minutes

b. 90 minutes

c. 120 minutes

d. 150 minutes.

17. The lime which contains high percentage of calcium oxide, is generally called

a. fat lime

b. rich lime

c. white lime

d. All of these.

18. Pick up the correct statement from the following:

a. Cast iron oxidises less

b. Steel oxidises most

c. Wrought iron oxidises moderately

d. All the above.

19. Pick up the incorrect statement from the following:

a. The axis of the mould is perpendicular to the end faces

b. The internal surface of the mould is kept clean and free from set cement

c. The mould is in the form of a frustum of hexagonal pyramid

d. None of these.

20. Construction joints are generally provided in concrete

a. retaining walls

b. lining of canals

c. lining of tunnels

d. all the above.

21. The factor which affects workability, is

a. shape and size of the aggregates

b. grading and surface textures of the aggregates

c. air entraining agents

d. all the above.

22. Separation of coarse aggregates from mortar during transportation, is known

a. creeping

b. segregation

c. shrinkage

d. none of these.

23. The standard sand now a days used in India, is obtained from

a. Jullundur (Punjab)

b. Hyderabad (Andhra Pradesh)

c. Ennore (Madras)

d. Cuttuck (Orissa).

24. Stoke's law sates that the velocity at which a grain settles out of suspension, the other factors remaining constant, is dependent upon

a. weight of grain

b. size of grain

c. shape and size of grain

d. shape, size and weight of grain.

25. The effective size of particles of soil is denoted by

a. D10

b. D20

c. D30

d. D60

26. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

27. If a soil undergoes a change in shape and volume by application of external loads over it, but recovers its shape and volume immediately after removal of the load, the property of the soil is said to be

a. Resilience of soils

b. Elasticity of soils

c. Compressibility of soils

d. None of these.

28. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

29. The coefficient of curvature for a well graded soil, must be between

a. 0.5 to 1.0

b. 1.0 to 3.0

c. 3.0 to 4.0

d. 4.0 to 5.0

30. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

31. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

32. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

33. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

34. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

35. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

36. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

37. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

38. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

39. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

40. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

41. The most of the texture can be drawn with the help of drawing pen with line thickness

a. 0.1mm

b. 0.5mm

c. 0.8mm

d. 1.0mm

42. The space covered by a drawing in a paper after changing the scale from 1:100 to 1:50 will:

a. increase 400%

b. increase 200%

c. decrease 200

d. decrease 400%

43. To draw or measure angles .......... is used.

a. set-square

b. T-square

c. protractor

d. all of the above

44. Working drawing is prepared not for

a. bill of quantities

b. estimation

c. layout

d. specification

45. Pick up the main purpose of project cost control from the following :

a. To signal immediate warning of uneconomic operations

b. To provide a feed back to the estimator

c. To promote cost consciousness

d. All of these

46. The CRF (ep) is also known as: [CRF(EP) - 8% - 7], where

a. 8% is the rate of interest per year

b. money is borrowed for n = 7 years

c. both (a) and (b)

d. Neither (a) nor (b)

47. Ratio analysis of a construction firm is used for analysis by :

a. firm's management

b. Banks of the firm

c. financial analysts

d. All of these.

48. The minimum qulification required for a general engineer is

a. diploma ine engg.

b. bachelor in engg.

c. master in engg.

d. all of the above

49. Which of the following category of Engineer is not mentioned in Engineering Council rule

a. Charted Engineer

b. General Engineer

c. Professional Engineer

d. Non-Nepali Regd. Engineer

50. The minimum qulification & experience required for the registration of foreigner engineers is

a. bacheclor in engg. and 5 years experience

b. bachelor in engg. and 10yrs experience

c. master in engg. and 5 yrs experience

d. master in engg. and 10yrs experience