***Engineer***

1. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

2. A masonry dam (density = 20, 000 N/m3) 6 m high, one metre wide at the top and 4 m wide at the base, has vertical water face. The minimum stress at the base of the dam when the reservoir is full, will be

a. 75 N/m2

b. 750 N/m2

c. 7500 N/m2

d. 75000 N/m2

3. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

4. If a three hinged parabolic arch carries a uniformly distributed load on its entire span, every section of the arch resists.

a. compressive force

b. tensile force

c. shear force

d. bending moment.

5. The moment diagram for a cantilever whose free end is subjected to a bending moment, will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

6. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

7. For indirect ranging, number of ranging rods required, is

a. 2

b. 3

c. 4

d. 5

8. Pantagraph is used for

a. measuring distances

b. measuring areas

c. enlarging or reducing plans

d. setting out right angles

9. For a curve of radius 100 m and normal chord 10 m, the Rankine's deflection angle, is

a. 0°35'.95

b. 1°25'.53

c. 1°35'.95

d. 2°51'.53.

10. The sag of 50 m tape weighing 4 kg under 5 kg tension is roughly

a. 0.053 m

b. 0.063 m

c. 0.073 m

d. 0.083 m

11. Pick up the correct statement from the following

a. 1 degree of arc corresponds to a displacement ratio of 1:57

b. the angular errors tend to propagate themselves along a traverse as the square root of the number of stations

c. the errors arising from the linear measures tend to be roughly proportional to the lengths of the lines

d. all the above.

12. In horizontal angles, the error due to imperfect levelling of the plate bubble is

a. large when sights are nearly level

b. large for long sights

c. less for steeply inclined sights

d. large for steeply inclined sights.

13. For preparing porcelains, the clay should be

a. sufficiently pure

b. of high degree of tanacity

c. of good plasticity

d. All the above.

14. Pick up the synthetic resin from the following:

a. Phenolic resin

b. Resorcinol resin

c. Urea resin

d. All of these.

15. The thermosetting plastic

a. becomes rigid when moulded at suitable pressure and temperature

b. at 127°C to 177°C permanently set and further application of heat does not soften it

c. charrs at 343°C

d. All the above.

16. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

17. For making fly-ash building bricks, the following mix of fly-ash, sand and lime, is

a. 80 : 13 : 7

b. 70 : 20 : 10

c. 60 : 35 : 5

d. none of these.

18. Refractory bricks are used for

a. retaining walls

b. columns

c. piers

d. combustion chambers.

19. An excess of flaky particles in concrete aggregates

a. increases the quantity of water and sand

b. affects the durability of concrete

c. more than 15% are not desirable

d. all the above.

20. Separation of water or water sand cement from a freshly concrete, is known

a. bleeding

b. creeping

c. segregation

d. flooding

21. The lower water cement ratio in concrete, introduces

a. greater density and smaller permeability

b. improved frost resistance

c. greater wear resistance and improved bond strength

d. all the above.

22. Sand requiring a high water cement ratio, belongs to

a. Zone I

b. Zone II

c. Zone III

d. Zone IV.

23. An ideal ware house, is provided

a. water proof roof

b. few windows which remain generally closed

c. 15 cm thick concrete floor laid on a dry course of soling

d. all the above.

24. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

25. If dry density, water density and specific gravity of solids of a given soil sample are 1.6 g/cc, 1.84 g/cc and 2.56 respectively, the porosity of the soil sample, is

a. 0.375

b. 0.370

c. 0.380

d. 0.390

26. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

27. If a soil undergoes a change in shape and volume by application of external loads over it, but recovers its shape and volume immediately after removal of the load, the property of the soil is said to be

a. Resilience of soils

b. Elasticity of soils

c. Compressibility of soils

d. None of these.

28. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

29. Water content of a soil sample is the difference of the weight of the given sample at the given temperature and the weight determined after drying it for 24 hours at temperature ranging from

a. 90° to 95°C

b. 95° to 100°C

c. 103° to 105°C

d. 105° to 110°C

30. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

31. Optimistic time, most likely time and pessimistic times for the activities of a network in the given figure are written above their arrows. If the contractual obligation time for the project is 75, the latest occurrence time for the event 2, is

a. 20

b. 25

c. 35

d. 15

32. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

33. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

34. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

35. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

36. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

37. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

38. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

39. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

40. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

41. Which of the following theories of mathematics is used by a CAD (computer Aided Design) software in are in creating a perspective design?

a. descriptive geometry

b. linear algebra

c. Bernoulli's theorem

d. vector analysis

42. The selection of suitable scale and allotment of proper space for margin, title block, revision panel, folding marks, etc. on the drawing sheet is known as:

a. formatting

b. layout

c. folding

d. centering

43. In the first angle method of orthographic projection

a. the object comes between the observer and the plane

b. the plane comes between the object and the observer

c. the observer comes between the object and the plane

d. none of the above

44. In case of dimetric projection

a. two sides are not equally inclined

b. two sides are equally inclined

c. two sides are parallel

d. two sides are equal

45. Which one of the following questions is relevant to the construction estimates :

a. Did the estimators precisely evaluate site conditions ?

b. Did the estimators use short cut methods which may be un realistic in their situation ?

c. How much money will the contractor's risk, loosing if he Were to submit bid on the raw estimate of cost.

d. All of these

46. The person desires to pay off the amount in 10 equal annual instalments. The amount of each instalment is :

a. Rs 5638

b. Rs 6638

c. Rs 7738

d. None of these

47. Pick up the correct reason for making conceptual (or preliminary) estimate from the following:

a. To have a check on a definitive cost estimate.

b. To check qoutations from contractors and/or sub contractors.

c. To compute target estimate for the owner while drawings and specifications are in initial stage.

d. All of these.

48. Profession is defined as

a. systematic knowledge through specialized training

b. synonym to job or occupation

c. types of service to needy people

d. all of above

49. Who is the Chairperson of House Rent Fixation Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

50. Engineering shall act in such a manner as to uphold and enhance the honor, and dignity of the engineering profession and shall act with ..... for bribery, fraud, and corruption.

a. a tolerance

b. a penchant

c. zero tolerance

d. a predilection