***Engineer***

1. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

2. Shear strain energy theory for the failure of a material at elastic limit, is due to

a. Guest or Trecas

b. St. Venant

c. Haig

d. Von Mises

3. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

4. n and j are numbers of members and joints in a frame. It contains redundant members if

a. n = 3j - 2

b. n < 2j - 3

c. n < j - 2

d. n > 2j - 3

5. Columns of given length, cross-section and material have different values of buckling loads for different end conditions. The strongest column is one whose

a. both ends are hinged or pin jointed

b. one end is fixed and the other end entirely free

c. both the ends are fixed

d. none of the these.

6. There are two stations A and B. Which of the following statements is correct :

a. the fore bearing of AB is AB

b. the back bearing of AB is BA

c. the fore and back bearings of AB differ by 180°

d. all the above.

7. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum deflection angle to locate its junction point, is

a. 1°43' 08"

b. 1°43' 18"

c. 1°43' 28"

d. 1°43' 38"

8. Cross-staff is used for

a. setting out right angles

b. measuring contour gradient

c. taking levels

d. measuring distances

9. A lemniscate curve will not be transitional throughout, if its deflection angle, is

a. 45°

b. 60°

c. 90°

d. 120°

10. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

11. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

12. While setting a plane table at a station it was found that the error in centering was 30 cm away from the ray of length 40 m drawn from the station. If the scale of the plan is 1 cm = 2 cm, the displacement of the end of the ray in plan from the true position will be

a. 0.02 cm

b. 0.15 cm

c. 02 cm

d. 0.1 cm

13. Apiece of sawn timber whose cross-sectional dimensions exceed 5 cm, in one direction and 20 cm in the other direction, is called a

a. cant

b. deal

c. baulk

d. strip.

14. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

15. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

16. A ferrous metal is

a. cast iron

b. wrought iron

c. steel

d. all the above.

17. The presence of sand in brick earth prevents:

a. cracking of bricks

b. shrinkage of bricks

c. warping of bricks

d. none of these.

18. The portion of a brick cut to form angles other than right angles in plan, is known as

a. queen closer

b. king closer

c. closer

d. squint brick.

19. For the construction of cement concrete floor, the maximum permissible size of aggregate, is

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

20. Strength of concrete with passage of time

a. increases

b. decreases

c. fluctuates

d. remains constant.

21. Workability improved by adding

a. hydrated lime

b. calcium chloride

c. bentonite

d. all the above.

22. Pick up the correct statement from the following:

a. The maximum size of the fine aggregate, is 4.75 mm and minimum 0.075 mm

b. The material having particles of size varying from 0.06 mm to 0.002 mm, is known as silt

c. The material having particles of size less than 0.002 mm, is known as clay

d. All the above.

23. Pick up the correct statement from the following:

a. Water lubricates the mixture of gravel, sand and cement

b. Only a small quantity of water is required for hydration of cement

c. Strength of concrete structure largely depends upon its workability

d. All the above.

24. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

25. A triaxial shear test is prefered to direct shear test, because

a. it can be performed under all three driange conditions with complete control

b. precise measurement of pore pressure and change in volume during test, is not posible

c. stress distribution on the failure plane, is non uniform

d. none of these.

26. Cohesive soils are generally

a. plastic and also compressible

b. elastic and also compressible

c. plastic but incompressible

d. none of these.

27. A soil has bulk density 2.30 g/cm3 and water content 15 per cent, the dry density of the sample, is

a. 1.0 g/cm2

b. 1.5 g/cm3

c. 2.0 g/cm3

d. 2.5 g/cm3

28. According to the Indian Standards the specific gravity is the ratio of the unit weight of soil solids to that of water at a temperature of

a. 17°C

b. 23°C

c. 27°C

d. 30°

29. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

30. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

31. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

32. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

33. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

34. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

35. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

36. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

37. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

38. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

39. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

40. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

41. Why perspective drawing is done by manufacturer?

a. It is easy to express in the market

b. shape is confirmed

c. exact size is not measured

d. all of the above

42. Engineers prefer to make ......... sketches than to pictorial sketches to clear the shape of an object

a. orthographic

b. perspective

c. auto CAD

d. all of the above

43. Working edge of drawing board is kept ........... of the user.

a. left

b. right

c. ahead

d. all of the above

44. Tracing paper is also named as

a. drafting vellum

b. drafting film

c. drafting media

d. drafting line

45. Pick up the correct statement from the following:

a. Uniform series present worth factor =

b. Sinking fund factor =

c. Capital recovery factor =  where letters carry their usual meanings.

d. All of these

46. The person desires to pay off the amount in 10 equal annual instalments. The amount of each instalment is :

a. Rs 5638

b. Rs 6638

c. Rs 7738

d. None of these

47. Pick up the ratio which gives us sufficient information by which to judge the financial condition and performance of the firm, from the following:

a. Financial leverage ratio

b. Activity ratio

c. Portability

d. None of these

48. Engineer shall practices:

a. in favour of client

b. in favour of contractor

c. in favour of him/herself

d. impartial andefficient professional relationship with client, contractor & fellow personnels

49. Engineering ethics is the field of applied ethics and system of moral principles that applt to be

a. practice engineering

b. responsibility engineering

c. management engineering

d. accountability engineering

50. In general, Nepal Engineering Council mecting per year should be held:

a. 2times

b. 3times

c. 4times

d. 5times