***Engineer***

1. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

2. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

3. Stress may be expressed in Newtons

a. per millimetre square (N/mm2)

b. per centimetre square (N/cm2)

c. per metre square (N/m2)

d. All of the above.

4. A simply supported beam carrying a uniformly distributed load over its whole span, is propped at the centre of the span so that the beam is held to the level of the end supports. The reaction of the prop will be

a. 3/8th the distributed load

b. 5/8th the distributed load

c. distributed load.

d. none of these.

5. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

6. The rise and fall method of reduction of levels, provides a check on

a. back sights

b. fore sights

c. intermediate sights

d. all of these.

7. Perpendicular offset from a tangent to the junction of a transition curve and circular curve is equal to

a. shift

b. twice the shift

c. thrice the shift

d. four times the shift.

8. If the radius of a simple curve is R, the length of the chord for calculating offsets by the method of chords produced, should not exceed.

a. R/10

b. R/15

c. R/20

d. R/25.

9. Prismatic compass is considered more accurate than a surveyor's compass, because

a. it is provided with a sliding glass in the object vane

b. its graduations are in whole circle bearings

c. it is provided with a prism to facilitate reading of its graduated circle

d. both (c) and (d).

10. A transit is oriented by setting its vernier A to read the back azimuth of the preceding line. A back sight on the preceding transit station taken and transit is rotated about its vertical axis. The vernier A reads

a. azimuth of the forward line

b. bearing of the. forward line

c. back bearing of the forward line

d. equal to 360°-azimuth of the forward line.

11. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

12. The instrument which is used in plane tabling for obtaining horizontal and vertical distances directly without resorting to chaining, is known as

a. Plane alidade

b. telescopic alidade

c. clinometer

d. tacheometer.

13. Pick up the most favourable condition for the rapid growth of fungus for dry rot from the following:

a. dampness

b. presence of sap

c. stagnant air

d. All the above,

14. Wrought iron is manufactured from pig iron by

a. puddling

b. shingling

c. rolling

d. all the above.

15. Durability of building stone is affected by its

a. texture

b. resistance to atmosphere

c. location in structure

d. all the above.

16. A badly mixed cement concrete results in

a. segregation

b. bleeding

c. honey combing

d. none to these.

17. Index number expressing the relative sizes of both coarse and fine aggregates, is called

a. proportioning of aggregates

b. fineness modulus

c. grading of aggregates

d. none of these.

18. Name the type of cement from the following for canal linings :

a. sulphate resisting cement

b. rapid hardening cement

c. quick setting cement

d. pozzuolana cement.

19. Workability improved by adding

a. foaming agent

b. oily-agent

c. aluminium compound

d. all the above.

20. Internal friction between the ingredients of concrete, is decreased by using

a. fine aggregates

b. rich mix

c. more water and coarse aggregates

d. none of these.

21. According to IS : 382-1963, a good aggregate should be

a. chemically inert

b. sufficiently strong

c. hard and durable

d. all the above.

22. Pick up the correct proportions of chemical ingredients of cement

a. Lime : Silica : Alumina : Iron oxide : 63 : 22 : 6 : 3

b. Silica : Lime : Alumina : Iron oxide : 63 : 22 : 6 : 3

c. Alumina : Silica : Lime : Iron oxide : 63 : 22 : 6 : 3

d. Iron oxide : Alumina : Silica : Lime : 63 : 22 : 6 : 3

23. Pick up the correct statement from the following:

a. According to the petrological characteristics, concrete aggregates are classified as heavy weight, normal weight and light weight

b. According to the shape of the particles, concrete aggregates are classified as rounded irregular, angular and flaky

c. According to the surface texture of the particles, the concrete aggregates are classified as glassy, smooth, granular, rough, crystalline, honey combed and porous

d. All the above.

24. Pick up the correct statement from the following:

a. When water table is above the base of a footing, the dry weight m should be used for soil below water table

b. When water table is located somewhat below the base of a footing, the elastic wedge is partly of moist soil and partly of submerged soil, and a suitable reduction factor is used

c. When water table is just at the base of the footing, no reduction factor is used

d. None of these.

25. If the coefficient of the active pressure Ka is 1/3, the coefficient of passive pressure Kp, is

a. 2/3

b. 1

c. 3/2

d. 3

26. A coarse-grained soil has a voids ratio 0.75, and specific gravity as 2.75. The critical gradient at which quick sand condition occurs, is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

27. 'Talus' is the soil transported by

a. wind

b. water

c. glacier

d. gravitational force.

28. Chemical weathering of soil is caused due to

a. carbonation

b. hydration

c. leaching

d. all the above.

29. Si particles

a. show dilatancy

b. swell when moist

c. possess high strength when dry

d. disintegrate easily.

30. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

31. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

32. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

33. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

34. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

35. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

36. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

37. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

38. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

39. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

40. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

41. In isometric view a circle is seen as :

a. a parabola

b. a hyperbola

c. a cone

d. an ellipse

42. ......... is used to protect the adjoining lines on the drawing when some parts of the line is being erased

a. eraser

b. erasing shield

c. pencil

d. drop compass

43. Tracing paper is also named as

a. drafting vellum

b. drafting film

c. drafting media

d. drafting line

44. The polyhedra  having two equal and similar end bases, parallel to each other and are other faces which may be rectangle, or parallelograms is called ......

a. a prism

b. a pyramid

c. a cylinder

d. a cone

45. If a seller recovers his capital along with accumulated compensating interest not in one single lumpsum payment but in periodical equal payments, over time :

a. Capital Recovery Annuity fs availed

b. Present work Annuity is availed

c. Sinking Fund Annuity is availed

d. Sinking Fund Annuity is availed

46. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

47. The construction manager uses the estimate of the project

a. to advise the architect/engineer regarding design cost parameter especially in value engineering analysis.

b. to develop bids on the project.

c. to control the project during its construction.

d. All of these

48. Which one type of membership is not categorized by NEC

a. general engineer

b. foreign engineer

c. professional engineer

d. supreintendent engineer

49. Code of conduct is not related with

a. making the advice of the enginerr impartial

b. building code

c. ensuring the quality of advice

d. making the advice untained by any personal consideration

50. The minimum and maximum no. of members in the user's committee should be

a. 15&17

b. 17&19

c. 9& 15

d. 7&9