***Engineer***

1. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

2. The forces in the members of simple trusses, may be analysed by

a. graphical method

b. method of joints

c. method of sections

d. all the above.

3. A shaft rotating N.R.M. under a torque T, transmits a power

a.  Newton metres/sec

b.  Newton metres/min

c.  Newton metres/min

d.  Newton metres/sec

4. The effect of arching a beam, is

a. to reduce the bending moment throughout

b. to increase the bending moment throughout

c. nothing on the bending throughout

d. ail the above.

5. The maximum deflection of

a. a fixed ended beam carrying a distributed load over the span is

b. a fixed ended beam carrying a concentrated load at the mid span is

c. a cantilever beam subjected to a moment M at the free end is

d. All the above.

6. For orientation of a plane table with three points A, B and C, Bessel's drill is

a. Align b through a and draw a ray towards c, align a through b and draw a ray towards c, finally align c through the point of intersection of the previously drawn rays

b. Align c through a and draw a ray towards b, align a through c and draw a ray towards b, finally align b through the point of intersection of the previously drawn rays

c. Align c through b and draw a ray towards a, align b through c and draw a ray towards a, finally align a, through the point of intersection of the previously, drawn rays

d. In the first two steps any two of the points may be used and a ray drawn towards the third point, which is sighted through the point of intersection of previously drawn rays in the final step.

7. Hydrographic surveys deal with the mapping of

a. large water bodies

b. heavenly bodies

c. mountaineous region

d. canal system

8. In a closed traverse, sum of south latitudes exceeds the sum of north latitudes and the sum of east departures exceeds the sum of west departures, then, the closing line will lie in

a. north-west quadrant

b. north east quadrant

c. south-east quadrant

d. south-west quadrant.

9. Two concave lenses of 60 cm focal length are cemented on either side of a convex lens of 15 cm focal length. The focal length of the combination is

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

10. The constant vertical distance between two adjacent contours, is called

a. horizontal equivalent

b. vertical equivalent

c. contour interval

d. contour gradient.

11. One of the Lehmann's rules of plane tabling, is

a. location of the instrument station is always distant from each of the three rays from the known points in proportion to their distances

b. when looking in the direction of each of the given points, the instrument station will be on the right side of one and left side of the other ray

c. when the instrument station is outside the circumscribing circle its location is always on the opposite side of the ray to the most distant point as the inter-section of the other two rays

d. none of these.

12. It is more difficult to obtain good results while measuringhorizontal distance by stepping

a. up-hill

b. down-hill

c. in low undulations

d. in plane areas.

13. Stainless steel resists corrosion due to

a. sulphur

b. vanadium

c. chromium

d. maganese.

14. Quick setting cement is produced by adding

a. more amount of gypsum in very fine powdered form

b. aluminium sulphate in very fine powdered form

c. pozzolana in very fine powdered form

d. none of these.

15. Pick up the correct statement from the following:

a. Lime is available by dissolving calcium carbonate in water

b. Lime is available by calcining calcium carbonate at 900°C

c. Lime is nothing but calcium chloride

d. None of these.

16. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

17. Minimum required water cement ratio for a workable concrete, is

a. 0.40

b. 0.50

c. 0.60

d. 1.0.

18. Quick lime

a. reacts with carbon dioxide

b. may be used for white-washing

c. when mixed with water forms slaked lime

d. all the above.

19. Separation of coarse aggregates from mortar during transportation, is known

a. creeping

b. segregation

c. shrinkage

d. none of these.

20. The surface where two successive placements of concrete meet, is known as

a. Expansion joint

b. Construction joint

c. both (a) and (b)

d. both (6) and (c).

21. You are asked to construct a massive dam, the type of cement you will use, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. white cement.

22. For ensuring quality of concrete, use

a. single sized aggegates

b. two sized aggregate

c. graded aggregates

d. coarse aggregates.

23. Pick up the correct statement from the following:

a. Cement should be mixed for at least one minute

b. 10% of water is placed in the rotating drum before adding dry material

c. 10% of water is added after placing the other ingredients in the drum

d. All the above.

24. If the unit weight of sand particles is 2.696 g/cc. and porosity in loose state is 44%, the critical hydraulic gradient for quick sand condition, is

a. 0.92

b. 0.93

c. 0.94

d. 0.95

25. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

26.  The internal molecular attraction of a soil, the cohesion

a. decreases as the moisture content increases

b. increases as the moisture content decreases

c. is more in well compacted clays

d. depends upon the external applied load.

27. Under-reamed piles are generally

a. driven piles

b. bored piles

c. precast piles

d. all the above.

28. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

29. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

30. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

31. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

32. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

33. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

34. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

35. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

36. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

37. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

38. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

39. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

40. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

41. Number of equal pieces of A4 size paper from A0 is equal to

a. 2

b. 4

c. 8

d. 16

42. The space between two sentences should be left equal to

a. height of letter

b. twice the ht. of letter

c. 1.5 times ht. of letter

d. none of the above

43. Engineers prefer to make ......... sketches than to pictorial sketches to clear the shape of an object

a. orthographic

b. perspective

c. auto CAD

d. all of the above

44. Pencil drawings are generally corrected or revised with the help of:

a. razor blade

b. eraser shield

c. ink

d. all of the above

45. Keeping in view, the feasibility order of magnitude, the preliminary, conceptual or budget estimates, are prepared by :

a. construction manager

b. owner himself/herself

c. construction manager

d. None of these

46. Pick up the correct statement regarding financial statement analysis from the following.

a. Final analysis always involves the use of various financial statements i.e., balance sheet and income statement.

b. The balance sheet is the summary of assests, liabilities and owner's equity of business at a point in time.

c. The income statement is the summary of revenues and expenses of a firm over a particular period of time.

d. All the above

47. The wages of supervisors and material handlers are charged as :

a. Over head

b. direct labour cost

c. indirect labour cost

d. None of these

48. The number of members in Nepal Engineers' Association are

a. 5

b. 10

c. 15

d. 20

49. "Fraudulent practice” means

a. offering anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above

50. The general experience required for nomination of chairperson of NEC is:

a. 10 yrs

b. 15 yrs

c. 20 yrs

d. 25 yrs