***Engineer***

1. The ratio of tangential and normal components of a stress on an inclined plane through θ° to the direction of the force, is:

a. cos θ

b. tan θ

c. cos θ

d. sec θ

2. Pick up the correct statement from the following:

a. The shear stress is zero at the outer fibres and maximum at the neutral axis

b. The bending stress at the outer fibres, is known as principal stress

c. The planes of principal stresses are inclined at 45° to the neutral plane

d. All the above.

3. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

4. A shaft turning 150 r.p.m. is subjected to a torque of 150 kgm. Horse power transmitted by the shaft is

a. π

b. 10 π

c. π2

d. 1/π

5. Maximum deflection of a

a. simply supported beam carrying a concentrated load W at mid-span is

b. cantilever beam, carrying a uniformly distributed load over span is

c. simply supported beam carrying a uniformly distributed load over the span is

d. All the above.

6. Centering error of a theodolite produces an error

a. in all angles equally

b. which does not vary with the direction or pointing

c. which varies with the direction of pointing and inversely with the length of sight

d. none of these.

7. The intercept of a staff

a. is maximum if the staff is held truly normal to the line of sight.

b. is minimum if the staff is held truly normal to the line of sight.

c. decreases if the staff is tilted away from normal

d. increases if the staff is tilted towards normal.

8. For true difference in elevations between two points A and B, the level must be set up

a. at any point between A and B

b. at the exact mid point of A and B

c. near the point A

d. near the point B.

9. If h1 and h2 are the differences in level between ground and the formation levels, m is the slope of the sloping sides. D is the distance between the cross sections then, prismoidal correction for a level section is

a. D/3m(h1 - h2)

b. D/6m(h1 - h2)2

c. D/6m(h1 - h2)3

d. D/6m(h1 + h2)2

10. In case of reduction of levels by the height of instrument method,

a. ∑ B.S. - ∑ F.S. = difference in R.L.S of the first station and last station

b. ∑ (R.L. + I + F.S.) - first R.L = ∑ (H.I. + No. of R.L.s.)

c. both (a) and (b) above

d. neither (a) nor (b).

11. Metric chains are generally available in

a. 10 m and 20 m length

b. 15 m and 20 m length

c. 20 m and 30 m length

d. 25 m and 100 m length

12. The tangent to the liquid surface in a level tube, is parallel to the axis of the level tube at

a. every point of the bubble

b. either end of the bubble

c. the mid-point of the bubble

d. no where.

13. Priming consists of

a. one part of white lead, 8 parts of chalk and four parts of twice boiled linseed oil

b. 8 parts of white lead, one part of chalk and four parts of twice boiled linseed oil

c. one part of white lead, 8 parts of chalk and one part of linseed oil

d. none to these.

14. Most commonly used solvent in oil paints, is

a. petroleum

b. spirit

c. coaltar

d. turpentine.

15. For preparing porcelains, the clay should be

a. sufficiently pure

b. of high degree of tanacity

c. of good plasticity

d. All the above.

16. Asbestos

a. is fire-proof and acid proof

b. has sp. gravity equal to 3.10

c. is smooth like glass and silk

d. All the above.

17. Dry rot

a. reduces the strength of timber

b. reduces the timber to powder

c. shrinks the timber

d. spoils the appearance of timber.

18. According to IS 399-1963, the weight of the timber is specified at

a. 8% moisture content

b. 10% moisture content

c. 12% moisture content

d. 14% moisture content.

19. The increased cohesiveness of concrete, makes it

a. less liable to segregation

b. more liable to segregation

c. more liable to bleeding

d. more liable for surface scaling in frosty weather

20. If the effective working time is 7 hours and per batch time of concrete is 3 minutes, the output of a concrete mixer of 150 litre capacity, is

a. 15, 900 litres

b. 16, 900 litres

c. 17, 900 litres

d. 18, 900 litres

21. Pick up the correct statement from the following:

a. Concrete using sand stones, cracks due to excessive shrinkage

b. Very hard and close grained crystallined lime stones are suitable aggregates but provide low strength

c. Broken bricks produce a concrete having good fire resisting qualities

d. All the above.

22. For batching 1:3:6 concrete mix by volume, the ingredients required per bag of 50 kg cement, are:

a. 70 kg of sand and 140 litres of aggregates

b. 105 litres of sand and 140 litres of aggregates

c. 105 litres of sand and 210 litres of aggregates

d. none of these.

23. The ratio of the length to breadth of a wooden float, is

a. 5.5

b. 6.5

c. 7.5

d. 8.5

24. Pick up the correct statement from the following:

a. Kaolinite shows a very little sign of swelling on wetting

b. Kaolinite when wet, becomes moderately plastic because negative electro magnetic charges on platelets attrack water

c. Kaolinite is also called China clay

d. All the above.

25. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

26. If voids ratio is 0.67, water content is 0.188 and specific gravity is 2.68, the degree of saturation of the soil, is

a. 25%

b. 40%

c. 60%

d. 75%

27. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

28. The equation τ = C + σ tan φ is given by

a. Rankine

b. Coulomb

c. Culaman

d. Mohr.

29. A failure wedge develops if a retaining wall

a. moves away from the backfill

b. moves towards the backfill

c. sinks downwards

d. stresses equally by vertical and horizontal forces.

30. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

31. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

32. Pre-tender stage requires

a. selection of site

b. formalization of alignment of work

c. formalization of designs and preparation of estimate

d. all the above.

33. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

34. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

35. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

36. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

37. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

38. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km

39. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

40. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

41. In case of perspective projection, picture plane is placed

a. between eye and object

b. outside of eye towards eye

c. outside of object toward object

d. all of above

42. In case of aligned method of dimensioning, they can be readable form

a. bottoms and right side of drawing sheet

b. bottoms and left side of drawing sheet

c. bottoms and top side of drawing sheet

d. all of the above

43. The equipment mainly used to transfer the measurements is

a. scale

b. divider

c. compass

d. drawing ping

44. The space covered by a drawing in a paper after changing the scale from 1:100 to 1:50 will:

a. increase 400%

b. increase 200%

c. decrease 200

d. decrease 400%

45. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

46. The financial analysis :

a. helps a share holder to compare the expected return on his investment in the firm against the expected return from other alternative investment.

b. helps a bank to know the financial position of the firm for granting a loan to the firm.

c. helps to judge the success of the firm's financial plans.

d. All of these.

47. The financial analysis helps to judge:

a. The operational efficiency of the firm

b. The financial position of the firm.

c. Both (a) and (b)

d. Neither (a) nor (b)

48. NEC does not approach for

a. engineer

b. overseer to engineer

c. minimum degree B.E.

d. all of the above

49. Maximum working hour for a labour per week should be

a. 42hr

b. 45hr

c. 48hr

d. 53hr

50. As per the NEC Act, 2055 the name of engineers

a. general, professional, and foreigner

b. special, local and international wngineer

c. civil, electrical and mechanical engineer.

d. computer, electronics and agriculture engineer