***Engineer***

1. For beams of uniform strength, if depth is constant,

a. width b ∝√ M

b. width b ∝ M

c. width b ∝ 3√ M

d. width b ∝ 2M

2. A steel rod 1 metre long having square cross section is pulled under a tensile load of 8 tonnes. The extension in the rod was 1 mm only. If Esteel = 2 x 106 kg/cm2, the side of the rod, is

a. 1 cm

b. 1.5 cm

c. 2 cm

d. 2.5 cm

3. Pick up the correct statement from the following:

a. The shear stress is zero at the outer fibres and maximum at the neutral axis

b. The bending stress at the outer fibres, is known as principal stress

c. The planes of principal stresses are inclined at 45° to the neutral plane

d. All the above.

4. The point of contraflexture occurs in

a. continuous beams only

b. over hanging beams only

c. all types of beams

d. both (a) and (b).

5. The shape of the bending moment diagram over the length of a beam, carrying a uniformly increasing load, is always

a. linear

b. parabolic

c. cubical

d. circular.

6. A dumpy level was set up at the midpoint between two pegs A and B, 50 m apart and the staff readings at A and B were 1.22 and 1.06. With the level set up a A, the readings at A and B were 1.55 and 1.37. The collimation error per 100 m length of sight is

a. 0.02 m inclined upwards

b. 0.04 m inclined downwards

c. 0.04 m inclined upward

d. none of these.

7. Locating the position of a plane table station with reference to three known points, is known as

a. intersection method

b. radiation method

c. resection method

d. three point problem.

8. Short offsets are measured with

a. an ordinary chain

b. an invar tape

c. a metallic tape

d. a steel tape.

9. In chain surveying tie lines are primarily provided

a. to check the accuracy of the survey

b. to take offsets for detail survey

c. to avoid long offsets from chain lines

d. to increase the number of chain lines.

10. An ideal transition curve is

a. cubic parabola

b. cubic spiral

c. clothoid spiral

d. true spiral.

11. During secular variation of magnetic meridian at different places

a. range of oscillations is constant

b. period of oscillation is constant

c. range and period of oscillation both vary

d. period of oscillation only varies.

12.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

13. Putty is

a. made with finely powdered chalk and linseed oil

b. used for fixing glass panes

c. softened by a solution of pearl ash and quick-lime soaked in water

d. all the above.

14. Depending on the chemical composition and mechanical properties, iron may be classified as

a. cast iron

b. wrought iron

c. steel

d. all the above.

15. Blister steel

a. is full of fissures and cavities

b. can not be forged

c. can be easily welded

d. all the above.

16. Calcination of iron ores is done

a. to remove carbonic acid

b. by roasting in heaps

c. after dressing

d. all the above.

17. Seasoning of timber is essential to remove

a. knots from timber

b. sap from timber

c. twisted fibre from timber

d. roughness of timber.

18. Lime mortar is generally made with

a. fat lime

b. hydraulic lime

c. plain lime

d. none of these.

19. Cement used for normal concrete construction, is obtained by burning a mixture of

a. argilaceous and calcareous materials

b. silicious and catcareous materials

c. silicious, argillaceous and calcareous materials

d. none of these.

20. The rock which is not calcareous, is :

a. macl

b. chalk

c. laterite

d. none of these.

21. Pick up the correct statement from the following:

a. Construction joints in columns are provided at the bottom haunching

b. Construction joints in beams and slabs are provided within middel third

c. Constructionjoints are generally provided in positions subjected to least shear force

d. All the above.

22. For a concrete mix 1:3:6 and water cement ratio 0.6 both by weight, the quantity of water required per bag, is

a. 12 kg

b. 14 kg

c. 16 kg

d. 20 kg

23. Water cement ratio is generally expressed in volume of water required per

a. 20 kg

b. 30 kg

c. 40 kg

d. 50 kg

24. The maximum pressure which a soil can carry without shear failure, is called

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

25. Which one of the following statements is true for Mohr-Coulomb envelope ?

a. The generalised Mohr theory suggests that, though the shear stress depends on the normal stress, the relation is not linear

b. Coulomb and Mohr suggest that a definite relationship exists among the principal stress and the angle of internal friction

c. For an ideal pure friction material, the straight line passes through the origin.

d. All the above.

26. Stoke's law sates that the velocity at which a grain settles out of suspension, the other factors remaining constant, is dependent upon

a. weight of grain

b. size of grain

c. shape and size of grain

d. shape, size and weight of grain.

27. The factor which affects the compaction, is

a. compacting content

b. method of compaction

c. type of soil

d. All the above.

28. If the coefficients of volume change and compressibility of a soil sample are respectively 6.75 x 10-2 and 3 x 10-2, the void ratio of the soil sample, is

a. 1.15

b. 1.20

c. 1.25

d. 1.30

29. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

30. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

31. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

32. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

33. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

34. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

35. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

36. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above

37. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

38. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

39. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

40. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

41. The plane on which perspective is formed is called:

a. horizontal plane

b. picture plane

c. ground plane

d. central plane

42. The two parts of T-square are....... and .......

a. vertical and horizontal edge

b. straight and slant edge

c. stock and blade

d. blade and batten

43. Bisecting a line means, divide a line into..... equal part.

a. two

b. three

c. four

d. none of the above

44. The area occupied by Ao (drawing sheet) is ........... times the area occupied by A4 (drawing sheet)

a. 2

b. 4

c. 8

d. 16

45. Each financial ratio is generally compared by

a. a past ratio calculated from the past financial standard of the firm.

b. a ratio developed by using the projected financial statement of the firm.

c. a ratio of some selected firms most progressive and successful at the point of consideration.

d. All of these

46. The financial analysis :

a. helps a share holder to compare the expected return on his investment in the firm against the expected return from other alternative investment.

b. helps a bank to know the financial position of the firm for granting a loan to the firm.

c. helps to judge the success of the firm's financial plans.

d. All of these.

47. The annuity which refers to a debt payment for recovering the initial amount or capital in equal periodical payments, is known as;

a. Present Worth Annuity

b. Sinking fund annuity

c. Compound annuity

d. Capital recovery annuity

48. For procurement of works up to 2 million, buy which of the following method is to be considered?

a. by sealed tender

b. by direct negotiation

c. by bidding

d. by direct labour or direct negotiation

49. Fundamental principle of Professional Engineering Ethics is:

a. uploading and advancing Engineering Profession

b. maintain high level of ethical conduct

c. both of the above

d. none of the above

50. The registration book for general and professional engineers is written in Nepali language while for foreginer engineer

a. Nepali

b. English

c. Nepali and English

d. French