***Engineer***

1. Pick up the correct statement from the following:

a. Mcg = (M M2 + r2) where letters carry their usual meanings

b. Tcp = m2 + T2 where letters carry their usual meanings

c. The torque which when acting alone would produce maximum shear stress equal to the maximum shear stress caused by the combined bending and torsion, is called equivalent torque

d. All the above.

2. Flat spiral springs

a. are supported at outer end

b. are wound by applying a torque

c. are used in clock-work mechanism

d. all the above.

3.  is the equation of Euler's crippling load if

a. both at the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

4. For the same height, the bottom width for no tension,

a. for triangular section is more than rectangular section

b. for rectangular section is more than triangular section

c. for triangular section is same as that of a rectangular section

d. none of these.

5. Influence lines are drawn for structures

a. of any type

b. statically determinate

c. pin-jointed truss

d. none of these.

6. The sensitiveness of a level tube decreases if

a. radius of curvature of its inner surface is increased

b. diameter of the tube is increased

c. length of the vapour bubble is increased

d. both viscosity and surface tension are increased.

7. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

8. The additional lines which are measured to show the correctness of the chain surveying are called:

a. check clines

b. proof lines

c. tie lines

d. all of these.

9. Subtense tacheometry is generally preferred to if ground is

a. flat

b. undulating

c. mountaineous

d. deserts.

10. Grid lines are parallel to

a. magnetic meridian of the central point of the grid

b. line representing the central true meridian of the grid

c. geographical equator

d. none of these.

11. Centering error of a theodolite produces an error

a. in all angles equally

b. which does not vary with the direction or pointing

c. which varies with the direction of pointing and inversely with the length of sight

d. none of these.

12. In levelling operation

a. when the instrument is being shifted, the staff must not be moved

b. when the staff is being carried forward, the instrument must remain stationary

c. both (a) and (b)

d. neither (a) nor (b).

13. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

14. Bitumen felt

a. is used as water proofing material

b. is used as damp proofing material

c. is made from bitumen and hessian fibres

d. all the above.

15. A rock contains only one mineral. It is called

a. homogeneous

b. non-homogeneous

c. monomineralic

d. polymineralic.

16. Good quality sand is never obtained from

a. river

b. nala

c. sea

d. gravel powder.

17. Forge pig may be converted to wrought iron by

a. rolling

b. pudding

c. shingling

d. refining.

18. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

19. If the depth of moist sand in a cylinder is 15 cm and the depth of the sand when fully inundated with water is 12 cm, the bulking of the moist sand, is

a. 12%

b. 15%

c. 20%

d. 25%.

20. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

21. Pick up the correct statement from the following:

a. Cement should be mixed for at least one minute

b. 10% of water is placed in the rotating drum before adding dry material

c. 10% of water is added after placing the other ingredients in the drum

d. All the above.

22. Cement used for normal concrete construction, is obtained by burning a mixture of

a. argilaceous and calcareous materials

b. silicious and catcareous materials

c. silicious, argillaceous and calcareous materials

d. none of these.

23. Segregation is responsible for

a. porous layers in concrete

b. surface scaling in concrete

c. sand streaks in concrete

d. all the above.

24. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

25. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

26. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

27. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

28. Buoyant unit weight equals the saturated density

a. multiplied by unit weight of water

b. divided by unit weight of water

c. plus unit weight of water

d. minus unit weight of water.

29. The relationship between void ratio (e) and porosity ratio (n) is :

a. n = e(1-e)

b. e = n(1+n)

c. n = e(1+e)

d. e = n/(1 - n)

30. The most popular type of organization used for Civil Engineering Constructions, is

a. line organization

b. line and staff organization

c. functional organization

d. effective organization.

31. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

32. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

33. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

34. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

35. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

36. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

37. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

38. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

39. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

40. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

41. Normally perspective projection is used by ....... to represent the object

a. engineer

b. doctor

c. manufacturer in marketing

d. painter

42. The section obtained when the section plane is inclined to the axis of cone, but parallel to one of the generators of a cone is called ......... ......

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

43. The projection in which the length, breadth and height of an object is shown in one view is known as:

a. pictorial projection

b. isometric projection

c. cavalier projection

d. cabinet projection

44. The best drawing paper used for water colour is :

a. hand made paper (hand made drawing sheet)

b. tracing drawing sheet

c. ordinary drawing sheet

d. ammonia paper

45. Pick up the correct statement from the following:

a. The capital required to get a project started, is called first cost.

b. The costs associated with a new or existing project that remain unaffected by the changes in activity level over the normal range of operation of the project, are called fixed costs.

c. The group of costs that vary proportionately to the changes in the activity level of a new or existing project are called variable costs.

d. All of these

46. Each financial ratio is generally compared by

a. a past ratio calculated from the past financial standard of the firm.

b. a ratio developed by using the projected financial statement of the firm.

c. a ratio of some selected firms most progressive and successful at the point of consideration.

d. All of these

47. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

48. Engineering shall act in such a manner as to uphold and enhance the honor, and dignity of the engineering profession and shall act with ..... for bribery, fraud, and corruption.

a. a tolerance

b. a penchant

c. zero tolerance

d. a predilection

49. The number of members in dispute resolution committee are

a. 2

b. 3

c. 4

d. 5

50. The employer's have right to

a. accept the bid

b. reject the bid

c. both (a) and (b) of above

d. none of the above