***Engineer***

1. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

2. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7

3. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

4. The section modulus of a rectangular light beam 25 metres long is 12.500 cm3. The beam is simply supported at its ends and carries a longitudinal axial tensile load of 10 tonnes in addition to a point load of 4 tonnes at the centre. The maximum stress in the bottom most fibre at the mid span section, is

a. 13.33 kg/cm2 compressive

b. 26.67 kg/cm2 tensile

c. 26.67 kg/cm2 compressive

d. none of these.

5. In a shaft shear stress intensity at a point is not

a. directly proportional to the distance from the axis

b. inversely proportional to the distance from the axis

c. inversely proportional to the polar moment of inertia

d. directly proportional to the applied torque.

6. The bearing of AB is 190° and that of CB is 260° 30'. The included angle ABC, is

a. 80° 30'

b. 99° 30'

c. 70° 30'

d. none of these

7. The Random errors tend to accumulate proportionally to

a. numbers of operations involved

b. reciprocal of operations involved

c. square root of the number of operation involved

d. cube root of the number of operation involved.

8. If L is in kilometres, the curvature correction is

a. 58.2 L2 mm

b. 64.8 L2 mm

c. 74.8 L2 mm

d. 78.4 L2 mm.

9. In horizontal angles, the error due to imperfect levelling of the plate bubble is

a. large when sights are nearly level

b. large for long sights

c. less for steeply inclined sights

d. large for steeply inclined sights.

10. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

11. In reciprocal levelling, the error which is not completely eliminated, is due to

a. earth's curvature

b. non-adjustment of line of collimation

c. refraction

d. non-adjustment of the bubble tube.

12. The line of collimation method of reduction of levels, does not provide a check on

a. intermediate sights

b. fore sights

c. back sights

d. reduced levels.

13. For light and ornamental casting, the most unsuitable pig iron, is

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

14. Pick up the correct statement from the following:

a. soft stones are required for carving

b. light stones are required for arches

c. hard stones are required to stand high pressure

d. All the above.

15. Pick up the correct statement from the following:

a. Horn blende mineral is brittle

b. Muscovite is also known as white mica and potashmica

c. Biotite is also known as blackmica

d. All the above.

16. Initial setting time of cement for asbestos cement products should be not less than

a. 30 minutes

b. 50 minutes

c. 75 minutes

d. 90 minutes.

17. Water paint is a

a. colour wash

b. whiting

c. distemper

d. all the above.

18. The cracks caused by shrinkage of the exterior surface of the wood exposed to atmosphere, are called:

a. radial shakes

b. heart shakes

c. wind cracks

d. twisted fibres.

19. The cement whose strength is a little lower than the ordinary cement during the first three months but attains afterwards the same strength, is known as

a. low-heat Portland cement

b. rapid hardening Portland cement

c. Portland blast slag cement

d. Portland pozzolana cement

20. The strength and quality of concrete, depend upon:

a. surface area of aggregates

b. shape of aggregates

c. surface texture of aggregates

d. all the above.

21. The top diameter, bottom diameter and the height of a slump mould are :

a. 10 cm, 20 cm, 30 cm

b. 10 cm, 30 cm, 20 cm

c. 20 cm, 10 cm, 30 cm

d. 20 cm, 30 cm, 10 cm

22. To hydrate 500kg of cement full water needed, is

a. 110 kg

b. 120 kg

c. 130 kg

d. 140 kg

23. For concreting tunnel linings, tran-portation of concrete is done by

a. wheel borrows

b. containers

c. pumps

d. belt conveyors.

24. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

25. If the cohesive force, (c), is 1.5 t/m2, the density (γ) of the soil is 2.0 t/m3, factor of safety (F) is 1.5 and stability factor (Sn) is 0.05, the safe height of the §lope, is

a. 5 metres

b. 8 metres

c. 10 metres

d. 12 metres

26. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

27. The shear resistance of a soil is con-tituted basically of the following component.

a. The frictional resistance to translocation between the individual soil particles at their contact point

b. To the structural relation to displacement of the soil because of the interlocking of the particles

c. Cohesion and adhesion between the surfaces of the soil particles

d. All the above.

28. The internal molecular attraction of a soil, the cohesion

a. decreases as the moisture content increases

b. increases as the moisture content decreases

c. is more in well compacted clays

d. depends upon the external applied load.

29. The shear strength of a soil

a. is proportional to the cohesion of the soil

b. is generally known as the strength of the soil

c. is proportional to the tangent of the angle of internal friction

d. all the above.

30. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

31. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

32. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

33. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

34. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

35. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

36. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

37. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

38. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

39. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

40. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

41. In orthographic projection which one have all three views same

a. cube

b. circle

c. sphere

d. both (a) and (c) of above

42. Sketching is usually done:

a. with French curves

b. free hand

c. with drafting machine

d. with protractor

43. To prevent warping of warping of the board ........are cleated at its back.

a. packing

b. wooden block

c. batten

d. none of the above

44. Topographic map is used to represent the

a. artificial detail

b. natural detail

c. construction detail

d. all of the above

45. Pick up the correct statement from the following:

a. The gross profit percentage is the average profit margin obtained on goods sold.

b. The relationship of contribution to sales is known as contribution ratio

c. The difference between sales and variable cost of sales, is called contribution.

d. All of these

46. Pick up the correct statement from the following:

a. The ability of a company to meet obligations which are likely to mature in short term, is called liquidity.

b. The liquidity ratio may be defined as a relationship of current liabilities and current assests and advances.

c. The liquidity ratios are used to indicate the financial position of the firm.

d. All of these

47. Pick up the element of the cost from the following:

a. direct material

b. direct labour

c. Over head

d. All of these

48. Who is the Chairperson of Project Co-ordination Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

49. As per the public Procurement Regulation, 2064 of Nepal, what is the maximum age for a technical proposal for consulting services ?

a. 90%

b. 80%

c. 70%

d. 60%

50. Which one type of membership is not categorized by NEC

a. general engineer

b. foreign engineer

c. professional engineer

d. supreintendent engineer