***Engineer***

1. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

2. A steel rod 1 metre long having square cross section is pulled under a tensile load of 8 tonnes. The extension in the rod was 1 mm only. If Esteel = 2 x 106 kg/cm2, the side of the rod, is

a. 1 cm

b. 1.5 cm

c. 2 cm

d. 2.5 cm

3. The force in AD of the truss shown in given figure, is

a. 3.0t compression

b. 0.5t compression

c. 0.5t tension

d. zero.

4. The ratio of the tensile stress developed in the wall of a boiler in the circumferential direction to the tensile stress in the axial direction, is

a. 4

b. 3

c. 2

d. 1

5. In a bar of large length when held vertically and subjected to a load at its lower end, its own-weight produces additional stress. The maximum stress will be

a. at the lower cross-section

b. at the built-in upper cross-section

c. at the central cross-section

d. at every point of the bar.

6. The 'fix' of a plane table from three known points, is good, if

a. middle station is nearest

b. middle station is farthest

c. either the right or left station is nearest

d. none of these.

7. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

8. Pick up the wrong statement from the following :

a. the diaphragm is placed between the eyepiece and the objective but nearer to the later

b. the outer component of the objective is a double-convex lens of crown glass

c. the inner component of the objective is a flint glass, convexo-concave

d. all the above.

9. Ramsden eye-piece consists of

a. two convex lenses short distance apart

b. two concave lenses short distance apart

c. one convex lens and one concave lens short distance apart

d. two plano-convex lenses short distance apart, with the convex surfaces facing each other.

10. If the length of a chain line along a slope of θ° is l, the required slope correction is

a. 2l cos2 θ/2

b. 2l sin2 θ/2

c. l tan2 θ/2

d. l cos2 θ/2.

11. True meridians are generally preferred to magnetic meridians because

a. these converge to a point

b. these change due to change in time

c. these remain constant.

d. None of these.

12. The diaphragm of a stadia theodolite is fitted with two additional

a. horizontal hairs

b. vertical hairs

c. horizontal and two vertical hairs

d. none of these.

13. The timber having maximum resistance against white ants, is obtained from

a. chir

b. shisham

c. sal

d. teak.

14. The compound of Portland cement which contributes to the strength after two to three years is

a. Tricalcium silicate

b. Di-calcium silicate

c. Tricalcium aluminate

d. Tetracalcium alumino ferrite.

15. The presence of original rounded surface on the manufactured piece of timber, is called

a. Wane

b. Torn grain

c. Diagonal grain

d. Chipmark.

16. The steel used for the manufacture of rails, is

a. Bessemer steel

b. mild steel

c. cast steel

d. stainless steel.

17. Pick up the correct statement from the following:

a. Catalysts are added to assist and accelerate the hardening of resin,

b. The fillers are inert materials and they impart strength and hardness

c. Fibrous fillers increase thermal resistance

d. All the above.

18. The percentage of water for normal consistency, is

a. 10% to 25%

b. 15% to 25%

c. 20% to 30%

d. 25% to 35%.

19. The diameter of the Vicat plunger is 10 mm and its length varies from

a. 30 mm to 40 mm

b. 40 mm to 50 mm

c. 50 mm to 60 mm

d. none of these.

20. Strength of concrete with passage of time

a. increases

b. decreases

c. fluctuates

d. remains constant.

21. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

22. Pick up the correct statement from the following:

a. The quantity of water required for concreting, depends upon the grading of aggregate amd method of compaction

b. 10% excess of water reduces the strength of concrete by 15%

c. 30% excess of water reduces the strength of concrete by 50%

d. All the above.

23. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

24. The direct shear test suffers from the following disadvantage:

a. Pore water pressure cannot be measured

b. Shear stress on the failure plane is not uniform.

c. The area under the shear and vertical loads does not remain constant throughout the test

d. All the above.

25. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

26. Pick up the correct statement from the following:

a. Silty clayloam contains highest percentage of silt

b. Stiff boulder clay offers maximum shear strength

c. Soft chalk carries least safe load

d. All the above.

27. The maximum load carried by a pile, when it continues to sink without further increase of load, is known as

a. ultimate load carrying capacity

b. ultimate bearing capacity

c. ultimate bearing resistant

d. all the above.

28. A coarse-grained soil has a voids ratio 0.75, and specific gravity as 2.75. The critical gradient at which quick sand condition occurs, is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

29. If a soil undergoes a change in shape and volume by application of external loads over it, but recovers its shape and volume immediately after removal of the load, the property of the soil is said to be

a. Resilience of soils

b. Elasticity of soils

c. Compressibility of soils

d. None of these.

30. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

31. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

32. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

33. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

34. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

35. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

36. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

37. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

38. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

39. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

40. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

41. As per IŞI recommended size of drawing board is/are

a. B0 (1000x 1500m)

b. B1 (700x 1000m)

c. B2 (500x700m)

d. all of the above

42. The angle between isometric axes is

a. 30°

b. 60°

c. 90°

d. 120°

43. For dimensioning and lettering normally following pencils are used

a. H & HB

b. 2H & HB

c. H&2HB

d. 2H & 2HB

44. Third angle projection is the traditional ............. system.

a. American

b. British

c. Indian

d. Nepali

45. In the cash-flow diagram shown in the given figure

a. Equal deposits of Rs 3000 per year (A) are made, starting now.

b. The rate of interest is 10% per yearaccount

c. The amount accumulated after the seventh deposit is to be computed

d. All of these

46. If P is principal amount, i is the rate of interest per annum and n is the number of periods in years, the compound amount factor (CAF) is :

a. (1 + i)n

b. (1 + i)(1/2n)

c. (n + i)

d. None of these

47. Pick up the correct statement from the following:

a. The ratios which show profitability in relation to sales and those which show profitability in relation to investment, are called profitability ratios.

b. The ratio of gross profit and net sales, is called profitability in relation to sales ratio.

c. The ratio of net profit after taxes to total assests is known as profitability in relation to investment ratio

d. All of these

48. In Nepal, which of the following act contains the provisions on health and safety of the workers?

a. contract act

b. industry act

c. labour act

d. all of the above

49. Who is the chairperson of Central Labour Advisory Board?

a. Secretory of ministry of labour

b. Specialist of ministry of labour

c. Director General, Department of labour

d. Labour minister/ satate minister

50. In general, Nepal Engineering Council mecting per year should be held:

a. 2times

b. 3times

c. 4times

d. 5times