***Engineer***

1. The ratio of shear stress and shear strain of an elastic material, is

a. Shear Modulus

b. Young's Modulus

c. Modulus of Elasticity

d. both (a) and (b).

2. Pick up the correct statement from the following:

a. Hoop strain of the walls of a cylinder due to liquid is

b. Longitudinal strain in the walls of a cylinder due to liquid is

c. Volumetric change in the cylinderdue to liquid is

d. All the above.

3. A spring of mean radius 40 mm contains 8 action coils of steel (N = 80000 N/mm2), 4 mm in diameter. The clearance between the coils being 1 mm when unloaded, the minimum compressive load to remove the clearance, is

a. 25 N

b. 30 N

c. 35 N

d. 40 N

4. The region of the cross-section of a column in which compressive load may be applied without producing any tensile stress, is known as the core of the cross-section. In circular columns the radius of the core, is

a. one-third of the radius

b. one-quarter of the radius

c. one-fifth of the radius

d. one-sixth of the radius.

5. A cantilever beam rectangular in cross-section is subjected to an isolated load at its free end. If the width of the beam is doubled, the deflection of the free end will be changed in the ratio of

a. 1/8

b. 1/2

c. 2

d. 3

6. Measuring with a 30 m chain, 0.01 m too short, introduces

a. positive compensating error

b. negative compensating error

c. positive cumulative error

d. negative cumulative error.

7. If a tacheometer is fitted with an anal-latic lens

a. additive constant is 100, multiplying constant is zero

b. multiplying constant is 100, additive constant is zero

c. both multiplying and additive constants are 100

d. both multiplying and additive constants are 50.

8. The angle between two plane mirrors of optical square, is

a. 30°

b. 45°

c. 60°

d. 90°.

9. The whole circle bearing of a line is 290°. Its reduced bearing is

a. N 20° E

b. N 20° W

c. N 70° W

d. S 70° E

10. One of the Lehmann's rules of plane tabling, is

a. location of the instrument station is always distant from each of the three rays from the known points in proportion to their distances

b. when looking in the direction of each of the given points, the instrument station will be on the right side of one and left side of the other ray

c. when the instrument station is outside the circumscribing circle its location is always on the opposite side of the ray to the most distant point as the inter-section of the other two rays

d. none of these.

11. From any point on the surface with a given inclination

a. only one contour gradient is possible

b. two contour gradients are possible

c. indefinite contour gradients are possible

d. all the above.

12. Pick up the correct specification of Ramsden eyepiece from the following :

a. the curved surfaces of plano-convex lenses face each other

b. the two lenses are separated by a distance equal to 2/3 of the focal length of either lens.

c. the distance between the diaphragm and the front lens of the eyepiece is kept equal to 1/4 th of the focal length of a lens so that rays from a point on the diaphragm enter the eye as a parallel beam

d. all the above.

13. Softer variety of steel may be obtained by

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

14. Refractory bricks resist

a. high temperature

b. chemical action

c. dampness

d. all the above.

15. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

16. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

17. The process of manufacturing steel by heating short lengths of wrought iron bars mixed with charcoal in fire clay crucibles and collecting the molten iron into moulds, is known as

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

18. The curved swellings from the growth of layers or wounds left after branches are cut off in an irregular manner, are known as

a. knots

b. rindgalls

c. burls

d. none of these.

19. If 20 kg of coarse aggregate is sieved through 80 mm, 40 mm, 20 mm, 10 mm, 4.75 mm, 2.36 mm, 1.18 mm, 600 micron, 300 micron and 150 micron standard sieves and the weights retained are 0 kg, 2 kg, 8 kg, 6 kg, 4 kg respectively, the fineness modulus of the aggregate, is

a. 7.35

b. 7.40

c. 7.45

d. none of these.

20. In the method of voids for determination of the quantity of cement paste, it is assumed that

a. Voids in fine aggregates are filled by the cement paste

b. Volume of fine aggregates is equal to total voids in coarse aggregates plus 10% extra

c. Volume of cement paste required is equal to total volume of voids in fine aggregates plus 15% extra

d. All the above.

21. Pick up the correct statement from the following:

a. The specific gravity of aggregate is important for the determination of the moisture content

b. The absorption and porosity of an aggregate influence the property of the concrete

c. A highly absorptive aggregate reduces the workability of concrete considerably

d. All the above.

22. Slump test of concrete is a measure ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

23. Horizontal construction joints in concrete walls are generally provided at

a. soffit level

b. window sill level

c. floor level

d. all the above.

24. A saturated soil sample has water content of 40% and specific gravity of soil particles 2.7. The void ratio of the soil, is

a. 0.4

b. 0.52

c. 1.08

d. none of these.

25. According to the Indian Standards the specific gravity is the ratio of the unit weight of soil solids to that of water at a temperature of

a. 17°C

b. 23°C

c. 27°C

d. 30°

26. A plane inclined at an angle φ to the horizontal at which the soil is expected to stay in the absence of any lateral support, is known as

a. natural slope line

b. repose line

c. the φ line

d. all the above.

27. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the degree of saturation, is

a. 95.4%

b. 95.5%

c. 95.6%

d. 95.7%

28. 'Talus' is the soil transported by

a. wind

b. water

c. glacier

d. gravitational force.

29. Pick up the correct statement from the following:

a. Illite bond is stronger than montmorillonite bond

b. Illites do not swell when wet

c. Illites are composed of two silica tetrahedralsheets with a central octahedral sheet

d. All the above.

30. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

31. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

32. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

33. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

34. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

35. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

36. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

37. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

38. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

39. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

40. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

41. "Perspective Drawing" is an approximate representation on a flat surface of an image.......

a. as it is perceived by the eye

b. as it is drawn on computer

c. of a free hand drawing

d. as it is felt by the hands

42. Inking pen is used for drawing -- in ink

a. writing

b. curve

c. straight line

d. none of the above

43. In drafting machine, which one does not follow :

a. levelling machine

b. T-squared

c. set square

d. divider

44. In engineering drawing, auxiliary views are used to show details of

a. incline surface

b. horizontal surface

c. vertical surface

d. none of the above

45. Which one of the following questions is relevant to the construction estimates :

a. Did the estimators precisely evaluate site conditions ?

b. Did the estimators use short cut methods which may be un realistic in their situation ?

c. How much money will the contractor's risk, loosing if he Were to submit bid on the raw estimate of cost.

d. All of these

46. Pick up the correct statement from the following:

a. An annuity is a series of equal payments occurring at equal period of time.

b. Annuity is called an equal payment or uniform payment series.

c. An annuity may have periods of time of any length but should always be of equal length.

d. All the above.

47. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

48. As per the Public Procurement Regulation of Nepal, 2064, What is the maximum consulting services that can be procured directly by a public agency?

a. Rs. 200,000/-

b. Rs. 300,000/-

c. Rs. 500,000/-

d. none of the above

49. Labour act was introduced /founded in

a. 2038

b. 2048

c. 2043

d. 2053

50. What percent of interest is taken for the advance amount paid to the contractor by employer?

a. 1% of advance amount

b. 2% of advance amount

c. 3% of advance amount

d. none of the above