***Engineer***

1. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

2. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

3. A steel rod of sectional area 250 sq. mm connects two parallel walls 5 m apart. The nuts at the ends were tightened when the rod was heated to 100°C. If αsteel = 0.000012/C°, Esteel = 0.2 MN/mm2, the tensile force developed at a temperature of 50°C, is

a. 80 N/mm2

b. 100 N/mm2

c. 120 N/mm2

d. 150 N/mm2

4. The shape of the bending moment diagram over the length of a beam, having no external load, is always

a. linear

b. parabolic

c. cubical

d. circular.

5. Beams of uniform strength are preferred to those of uniform section because these are economical for

a. large spans

b. heavy weights

c. light weights

d. short spans.

6. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

7. Probable accidental error in precise levelling as recommended by International Geodetic Association, should not exceed (where k is in kilometers.)

a. ± 0.5 k mm

b. ± 1 k mm

c. ± 2 k mm

d. ± 5 k mm.

8. Removal of parallax, may be achieved by focussing

a. the objective

b. the eye-piece

c. the objective and the eye-piece

d. none of these.

9. If the whole circle bearing of a line is 270°, its reduced bearing is

a. N 90° W

b. S 90° W

c. W 90°

d. 90° W.

10. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

11. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

12. If the rate of gain of radial acceleration is 0.3 m per sec3 and full centrifugal ratio is developed. On the curve the ratio of the length of the transition curve of same radius on road and railway, is

a. 2.828

b. 3.828

c. 1.828

d. 0.828.

13. If the furnace is provided with insufficient fuel at low temperatures, the type of pig iron produced, is called

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

14. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

15. Pick up the correct statement from the following:

a. Solder material is an alloy which melts at a temperature above 400°C

b. Brazing is done at temperature above 600°C to 1100°C

c. Brazing joint is stronger than the solder joint

d. all the above.

16. In the cement the compound quickest to react with water, is

a. Tricalcium aluminate

b. Tetra-calcium alumino-ferrite

c. Tricalcium silicate

d. Dicalcium silicate.

17. The most commonly used synthetic abrasive is

a. aluminium carbide

b. boric acid

c. silicon

d. All of these.

18. Pick up the correct statement from the following:

a. Lime is available by dissolving calcium carbonate in water

b. Lime is available by calcining calcium carbonate at 900°C

c. Lime is nothing but calcium chloride

d. None of these.

19. Too wet concrete may cause

a. excessive laitance

b. segregation

c. lower density

d. all the above.

20. Workability of concrete mix with low water cement ratio is determined by

a. slump test

b. compaction factor test

c. flexural strength test

d. none of these.

21. Pick up the correct statement from the following:

a. Concrete using sand stones, cracks due to excessive shrinkage

b. Very hard and close grained crystallined lime stones are suitable aggregates but provide low strength

c. Broken bricks produce a concrete having good fire resisting qualities

d. All the above.

22. The preliminary test is repeated if the difference of compressive strength of three test specimens, exceeds

a. 8 kg/cm2

b. 10 kg/cm2

c. 12 kg/cm2

d. 15 kg/cm2

23. A concrete using an air entrained cement

a. has more resistance to weathering

b. is more plastic and workable

c. is free from segregation and bleeding

d. all the above.

24. A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

25. Pick up the correct statement applicable to plate load test

a. At the centre of the test pit, a hole is dug out whose size is kept equal to the size of the test plate

b. Bottom level of the hole dug at the centre of the test pit, is kept at the level of the actual formation

c. Ratio of the depth of the hole to the width of the test plate is the same as the ratio of the depth of the actual formation to the width of the formation

d. All the above.

26. Pick up the correct statement from the following:

a. If the ratio of depth to width is more than 2, it is deep foundation

b. If the ratio of the length to width is between 1 and 2, it is spread foundation

c. If the length is large as compared to width, it is a strip foundation

d. All the above.

27. The bearing capacity of a soil depends upon

a. shape of the particles

b. cohesive properties of particles

c. internal frictional resistance of particles

d. all the above.

28. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

29. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

30. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

31. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

32. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

33. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

34. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

35. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

36. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet

37. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

38. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

39. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

40. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

41. Circle and arcs of circles are drawn by means of a .......

a. lengthening bar

b. divider

c. compass

d. T-square

42. In engineering drawing, auxiliary views are used to show details of

a. incline surface

b. horizontal surface

c. vertical surface

d. none of the above

43. The working edge of T-square is used to

a. draw horizontal line

b. as a base of set square

c. draw horizontal parallel line

d. all of the above

44. The angle between isometric axes is

a. 30°

b. 60°

c. 90°

d. 120°

45. The estimate based on a detailed quantity survey and furnishes the most accurate and reliable estimate possible is known as

a. Conceptual estimate

b. Definitive estimate

c. Probabilistic estimate

d. None of these

46. Pick up the correct statement from the following:

a. Both the numerator and denominator of financial ratios come directly from the balance sheet.

b. Income statement ratios compare one 'flow' item from the income statement to another flow item from the income statement.

c. Income statement ratios compare a flow item from the income statement to another flow item form the income statement

d. All of these

47. The wages of supervisors and material handlers are charged as :

a. Over head

b. direct labour cost

c. indirect labour cost

d. None of these

48. Engineering ethics is the field of applied ethics and system of moral principles that applt to be

a. practice engineering

b. responsibility engineering

c. management engineering

d. accountability engineering

49. Which one is correct with respect to values, morals and ethics?

a. values are the rules

b. morals are the personal character

c. ethics are professional sstandards

d. all of the above

50. The sequence of priority in making ethical decision is (from top to bottom)

a. society, profession organization, individual

b. profession, organization, individual, society.

c. individual, organization, profession, society

d. profession, individual, organization society