***Engineer***

1. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

2. Pick up the correct statement from the following:

a. The shear stress is zero at the outer fibres and maximum at the neutral axis

b. The bending stress at the outer fibres, is known as principal stress

c. The planes of principal stresses are inclined at 45° to the neutral plane

d. All the above.

3. A truss containing j joints and m members, will be a simple truss if

a. m = 2j - 3

b. j = 2m - 3

c. m = 3j - 2

d. j = 3m - 2

4. Failure of riverted joints is due to

a. Tearing of plates between rivets

b. Shearing of rivets

c. Crushing of rivets

d. All the above.

5. Columns of given length, cross-section and material have different values of buckling loads for different end conditions. The strongest column is one whose

a. both ends are hinged or pin jointed

b. one end is fixed and the other end entirely free

c. both the ends are fixed

d. none of the these.

6. An ideal vertical curve to join two gradients, is

a. parabolic

b. elliptical

c. hyperbolic

d. none of these.

7. A uniform slope was measured by the method of stepping. If the difference in level between two points is 1.8 m and the slope distance between them is 15 m, the error is approximately equal to

a. cumulative, + 0.11 m

b. compensating, ± 0.11 m

c. cumulative, - 0.11 m

d. none of these

8. The first reading from a level station is

a. foresight

b. intermediate sight

c. back-sight

d. any sight.

9. Mistakes which may produce a very serious effect upon the final results arise due to

a. in attention

b. in experience

c. carelessness

d. all of these.

10. Pick up the correct statement from the following:

a. the theodolite in which telescope can be rotated in vertical plane is called a transit

b. when the vertical circle is to the left of the telescope during observation, it is called to be in left face

c. when the vertical circle is to the right of the telescope during observation, it is called to be in right face

d. all the above.

11. In case of a double line river, contours are

a. stopped at the banks of the river

b. stopped at the edge of the river

c. drawn across the water

d. drawn by parabolic curves having their vertex at the centre of the water.

12. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum perpendicular offset for the transition curve, is

a. 1.70 m

b. 2.70 m

c. 3.70 m

d. 4.70 m

13. Snowcrete is one of the patent forms of

a. distempers

b. water proof cement paints

c. enamel paints

d. cellulose paints.

14. Based on flow quality, the sequence of pipes is

a. A.C. pipes, G.I. pipes, C.I. pipes, PVC pipes

b. C.I. pipes, G.I. pipes, A.C. pipes, PVC pipes

c. C.I. pipes, G.I. pipes, PVC pipes, A.C. pipes

d. None of these.

15. Stones used for ornamental work must be

a. soft

b. hard

c. light

d. heavy.

16. The lime which contains mainly calcium oxide and slacks with water, is

a. quick lime

b. hydraulic lime

c. poor lime

d. none of these.

17. Pick up the correct statement from the following:

a. Catalysts are added to assist and accelerate the hardening of resin,

b. The fillers are inert materials and they impart strength and hardness

c. Fibrous fillers increase thermal resistance

d. All the above.

18. Duco is one of the patent forms of

a. plastic paints

b. bituminous paints

c. aluminium paints

d. cellulose paints.

19. The commercial name of white and coloured cement in India, is

a. rainbow cement

b. silvicrete

c. snowcem

d. all the above.

20. If a grading curve is horizontal bet-wen the portions of 20 mm I.S. Sieve and 4.75 mm I.S. Sieve, the graded aggregates do not contain

a. 20 mm particles

b. 10 mm particles

c. 4.75 mm particles

d. all the above.

21. Pick up the correct statement from the following:

a. The concrete cured at a temperature below 23°C, gains strength up to 28 days

b. The concrete does not set at freezing point

c. The strength of concrete increases with its age

d. All the above.

22. The diameter of the Vicat plunger is 10 mm and its length varies from

a. 30 mm to 40 mm

b. 40 mm to 50 mm

c. 50 mm to 60 mm

d. none of these.

23. The aggregate impact value of the aggregate used in

a. building concrete is less than 45

b. road pavement concrete is less than 30

c. runway concrete is less than 30

d. all the above.

24. The quantity of seepage of water through soils is proportional to

a. coefficient of permeability of soil

b. total head loss through the soil

c. neither (a) nor (b)

d. both (a) and (b).

25. Stoke's law does not hold good if the size of particle is smaller than

a. 0.0002 mm

b. 0.002 mm

c. 0.02 mm

d. 0.2 mm

26. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

27. The plasticity index is the numerical difference between

a. liquid limit and plastic limit

b. plastic limit and shrinkage limit

c. liquid limit and shrinkage limit

d. none of these.

28. The ratio of settlement at any time 't' to the final settlement, is known as

a. co-efficient of consolidation

b. degree of consolidation

c. consolidation index

d. consolidation of undisturbed soil.

29. If there is no impervious boundary at the bottom of a hydraulic structure, stream lines tend to follow :

a. a straight line

b. a parabola

c. a semi-ellipse

d. a semi-circle.

30. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

31. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

32. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

33. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

34. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

35. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

36. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

37. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

38. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

39. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

40. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

41. The main purpose of as built drawing is

a. maintenance of service work '

b. dimensioning

c. profession

d. to award tender

42. Working edge of drawing board is kept ........... of the user.

a. left

b. right

c. ahead

d. all of the above

43. In engineering drawing, auxiliary views are used to show details of

a. incline surface

b. horizontal surface

c. vertical surface

d. none of the above

44. "Perspective Drawing" is an approximate representation on a flat surface of an image.......

a. as it is perceived by the eye

b. as it is drawn on computer

c. of a free hand drawing

d. as it is felt by the hands

45. Pick up the correct statement from the following:

a. The manifestation of the time value of money is termed as interest.

b. Interest on borrowing = present amount owed - original loan

c. The original investment (or loan) is referred to as principal.

d. All of these.

46. Liquidity ratios are used :

a. to measure a firms ability to meet short-cut obligations.

b. to compare short term obligations to short-term resources available to meet these obligations.

c. to obtain much insight into the present cash solvency of the firm and the firm's ability to remain solvent in the event of adversity.

d. All of these

47. Keeping in view, the feasibility order of magnitude, the preliminary, conceptual or budget estimates, are prepared by :

a. construction manager

b. owner himself/herself

c. construction manager

d. None of these

48. Which one is not concerned with NEC

a. acredition of academic certificate

b. authority of opening engineering college.

c. appointment of teacher

d. all of the above

49. The Nepal Engineers' Association was established in

a. 2014 BS

b. 2024 BS

c. 2034 BS

d. 2044 BS

50. As per the prevalling Public Procurement Acts & Regulations of Nepal, provision of price adjustments is normally to inculpated in civil works contract having project duration ?

a. more than 12 months

b. more than 15 months

c. more than 18 months

d. more than 24 months