***Engineer***

1. A steel bar 20 mm in diameter simply-supported at its ends over a total span of 40 cm carries a load at its centre. If the maximum stress induced in the bar is limited to N/mm2, the bending strain energy stored in the bar, is

a. 411 N mm

b. 511 N mm

c. 611 Nmm

d. 711 N mm

2. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

3. The load on a spring per unit deflection, is called

a. stiffness

b. proof resilience

c. proof stress

d. proof load.

4. A cantilever carrying a uniformly distributed load W over its full length is propped at its free end such that it is at the level of the fixed end. The bending moment will be zero at its free end also at

a. fixed point of the cantilever

b. 1/4th length from free end

c. 3/4th length from free end

d. half length from free end.

5. Along the principal plan subjected to maximum principal stress

a. maximum shear stress acts

b. minimum shear stress acts

c. no shear stress acts

d. none of these.

6. The magnetic meridian at any point, is the direction indicated by a freely suspended

a. magnetic needle

b. and properly balanced magnetic needle

c. properly balanced and uninfluenced by local attractive force

d. magnetic needle over an iron pivot.

7. Pick up the correct statement from the following :

a. the eyepiece plays no part in defining the line of sight

b. the diaphragm plays no part in defining the line of sight

c. the optical centre of the objective plays no part in defining the line of sight

d. none of these.

8. A transit is oriented by setting its vernier A to read the back azimuth of the preceding line. A back sight on the preceding transit station taken and transit is rotated about its vertical axis. The vernier A reads

a. azimuth of the forward line

b. bearing of the. forward line

c. back bearing of the forward line

d. equal to 360°-azimuth of the forward line.

9. The orthographical projection of a traverse leg upon the reference meridian, is known as

a. departure of leg

b. latitude to the leg

c. co-ordinate of the leg

d. bearing of the leg.

10. Mistakes which may produce a very serious effect upon the final results arise due to

a. in attention

b. in experience

c. carelessness

d. all of these.

11. If R is the radius of the main curve, θ the angle of deflection, S the shift and L the length of the transition curve, then, total tangent length of the curve, is

a. (R + S) tan θ/2 - L/2

b. (R + S) tan θ/2 + L/2

c. (R - S) tan θ/2 + L/2

d. (R - S) cos θ/2 + L/2

12. In chain surveying field work is limited to

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. all the above.

13. Snowcrete is one of the patent forms of

a. distempers

b. water proof cement paints

c. enamel paints

d. cellulose paints.

14. Seasoning of timber is done for

a. increasing moisture content

b. decreasing moisture content

c. increasing strength of timber

d. none to these.

15. Black marble is generally found in the district of

a. Jaipur

b. Jabalpur

c. Jaisalmer

d. Pune.

16. Mastic asphalt is generally used for

a. damp proof course

b. water proof layer

c. partition walls

d. both (a) and (b).

17. Second class bricks

a. are of dark brown colour

b. produce a metallic sound when struck

c. are well burnt

d. are under burnt.

18. Dorry's testing machine is used for

a. crushing test of stone

b. hardness test of stone

c. impact test of stone

d. water absorption test.

19. For construction of structures in sea water, the cement generally preferred to, is

a. Portland-pozzolana cement

b. quick setting cement

c. low heat Portland cement

d. rapid hardening cement

20. The void ratio of

a. single size coarse aggregate is roughly 0.45.

b. graded coarse aggregate is roughly 0.040

c. fine aggregate is roughly 0.45

d. all the above.

21. Grading of sand causes great variation in

a. strength of concrete

b. durability of concrete

c. handing and placing of concrete

d. all the above.

22. The process of hardening the concrete by keeping its surface moist is known

a. wetting

b. curing

c. compacting

d. none of these.

23. Pick up the incorrect statement from the following:

a. Tricalcium silicate (C3S) generates more heat of hydration

b. Tricalcium silicate (C3S) develops early strength

c. Tricalcium silicate (C3S) has more resistance to sulphate attack

d. None of these.

24. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

25. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

26. Pick up the cohesive soil from the following:

a. Red earth

b. Clay

c. Black cotton soil

d. Compacted ground.

27. The specific yield of soil depends upon

a. compaction of stratum

b. distribution of pores

c. shape and size of particles

d. all the above.

28. The compressibility of clays, is caused due to:

a. expulsion of double layer water from in between the grains

b. sliping of particles to new positions of greater density

c. bending of particles as elastic sheets

d. all the above.

29. The critical exist gradient of seepage water in soils, is

a. inversely proportional to the specific gravity

b. directly proportional to the specific gravity

c. inversely proportional to the voids ratio

d. none of these.

30. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

31. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

32. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

33. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

34. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

35. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

36. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

37. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

38. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

39. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

40. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

41. Pencil drawings are generally corrected or revised with the help of:

a. razor blade

b. eraser shield

c. ink

d. all of the above

42. In orthographic projection, projector lines are

a. vertical lines

b. inclined lines

c. horizontal lines

d. none of above

43. Which of the following is not a drawing hand tool?

a. alidade

b. straight edge

c. french curve

d. set squares

44. Third angle projection is the traditional ............. system.

a. American

b. British

c. Indian

d. Nepali

45. Pick up the element of the cost from the following:

a. direct material

b. direct labour

c. Over head

d. All of these

46. Annuities involve:

a. all payments of equal amount

b. payment at equal time intervals

c. payments at the end of periods.

d. All of these

47. Renu Bala deposits Rs 1200 now, Rs 800 two years from now and Rs 1000 five years from now. If the savings bank's rate of interest in 5%, she will receive an amount of Rs X 10 years from now, where X is

a. Rs 3415

b. Rs 4225

c. Rs 4413

d. Rs 4826

48. The document announcing to prospective bidder that the owner is ready to receive bids is called

a. notice to bidders

b. instruction to bidders

c. bill of quantities

d. intension to acceptance

49. The Nepal Engineers' Association was established in

a. 2014 BS

b. 2024 BS

c. 2034 BS

d. 2044 BS

50. The general experience required for nomination of chairperson of NEC is:

a. 10 yrs

b. 15 yrs

c. 20 yrs

d. 25 yrs