***Engineer***

1. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

2. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

3. Shear strain energy theory for the failure of a material at elastic limit, is due to

a. Guest or Trecas

b. St. Venant

c. Haig

d. Von Mises.

4. A beam is said to be of uniform strength, if

a. deflection is same throughout the beam

b. bending stress is same throughout the beam

c. shear stress is same throughout the beam

d. none of these.

5. The ratio of the effective length of a column and minimum radius of gyration of its cross-sectional area, is known

a. buckling factor

b. slenderness ratio

c. crippling factor

d. none of these.

6. Pick up the correct statement from the following :

a. the angle between the true meridian and the magnetic meridian is called magnetic variation

b. one end of the magnetic needle supported at its centre of gravity tends to dip down towards the. nearer magnetic pole of the earth

c. the magnet properly pivoted is balanced by means of riding weight movable along the needle

d. all the above.

7. The bubble tube is nearly filled with

a. alcohol or chloroform

b. a liquid which is very mobile

c. a liquid having low freezing point

d. all the above.

8. With usual notations, the expression  represents

a. centrifugal force

b. centrifugal ratio

c. super elevation

d. radial acceleration.

9. The magnetic meridian at any point, is the direction indicated by a freely suspended

a. magnetic needle

b. and properly balanced magnetic needle

c. properly balanced and uninfluenced by local attractive force

d. magnetic needle over an iron pivot.

10. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

11. The magnetic bearing of a line is 32° and the magnetic declination is 10° 15' W. The true bearing is

a. 21° 45'

b. 42° 15'

c. 42° 15' W

d. 21° 45' W

12. If S is the length of a subchord and R is the radius of simple curve, the angle of deflection between its tangent and sub-chord, in minutes, is equal to

a. 573 R/S

b. 171.9 S/R

c. 1718.9 R/S

d. 1718.9 S/R.

13. For obtaining vinyl chloride acetate, the method used, is

a. addition polymerization

b. condensation polymerization

c. co-polymerization

d. none of these.

14. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

15. Pick up the correct statement from the following:

a. Plastics have generally low melting point

b. The coefficient of thermal expansion of plastics is about three times than that of steel

c. The acoustical boards prepared by impregnating fibre-glass with phenolic resins has absorption coefficient of about 0.67

d. All the above.

16. The proportions of charcoal, saltpetre and sulphur in gun powder by weight, are respectively:

a. 15, 75, 10

b. 75, 10, 15

c. 10, 15, 75

d. 10, 75, 15.

17. Based on its dry weight, a freshly felled tree may contain water

a. 25%

b. 50%

c. 75%

d. 100%.

18. Shingle is

a. decomposed laterite

b. crushed granite

c. water bound pebbles

d. air weathered rock.

19. The aggregate impact value of the aggregate used in

a. building concrete is less than 45

b. road pavement concrete is less than 30

c. runway concrete is less than 30

d. all the above.

20. If the various concrete ingredients i.e. cement, sand and aggregates are in the ratio of 1:3:6, the grade of concrete, is

a. M 100

b. M 150

c. M 200

d. M 250

21. For an ordinary Portland cement

a. soundness varies from 5 to 10 mm

b. initial setting time is not less than 30 minutes

c. compressive stress after 7 days, is not less than 175 kg/cm2

d. all the above.

22. Curing

a. reduces the shrinkage of concrete

b. preserves the properties of concrete

c. prevents the loss of water by evaporation

d. all of the above.

23. According to I.S. : 456, the number of grades of concrete mixes, is

a. 4

b. 5

c. 6

d. 7

24. The shearing strength of a cohesion-less soil depends upon

a. dry density

b. rate of loading

c. confining pressure

d. nature of loading.

25. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

26. When drainage is permitted under initially applied normal stress only and full primarily consolidation is allowed to take place, the test is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

27. A soil not fully consolidated under the existing over-burden pressure, is called

a. pre-consolidated

b. normally consolidated

c. under-consolidated

d. none of these.

28. A decrease in water content results in a reduction of the volume of a soil in

a. liquid state

b. plastic state

c. semi solid state

d. all of these.

29. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

30. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

31. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

32. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

33. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

34. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

35. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

36. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

37. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

38. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

39. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

40. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

41. The techniques for free hand sketch to drawing circle is

a. fixing a fixed point and arc

b. fixing the perimeter

c. fixing the radius only

d. none of the above

42. The shape of scales are

a. flat and triangular

b. flat and rectangular

c. flat and square

d. all of the above

43. In engineering drawing, auxiliary views are used to show details of

a. incline surface

b. horizontal surface

c. vertical surface

d. none of the above

44. The angle between two perpendicular line is

a. 0°

b. 45°

c. 90°

d. 180°

45. The annuity which refers to a debt payment for recovering the initial amount or capital in equal periodical payments, is known as;

a. Present Worth Annuity

b. Sinking fund annuity

c. Compound annuity

d. Capital recovery annuity

46. Pick up the correct statement from the following:

a. The capital required to get a project started, is called first cost.

b. The costs associated with a new or existing project that remain unaffected by the changes in activity level over the normal range of operation of the project, are called fixed costs.

c. The group of costs that vary proportionately to the changes in the activity level of a new or existing project are called variable costs.

d. All of these

47. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

48. For the registration of general engineer tha amount of fee required is:

a. NRS 2000/-

b. NRS 3000/-

c. NRS 3500/-

d. NRS 4000/-

49. Professional code of conduct issued by Nepal Engineering Council has

a. 6 articles

b. 7 articles

c. 8 articles

d. 9 articles

50. Ethics is the science which deals with

a. rightness of professional

b. wrong of professional

c. both (a) and (b) of above

d. none of the above