***Engineer***

1. The locus of reaction of a two hinged semi-circular arch, is

a. straight line

b. parabola

c. circle

d. hyperbola.

2. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these

3. The normal component of a force inclined through θ° is obtained by multiplying the force by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

4. A three-hinged arch is said to be :

a. statically determinate structure

b. statically indeterminate structure

c. a bent beam

d. none of these.

5. The stress in the wall of a cylinder in a direction normal to its longitudinal axis, due to a force acting along the circumference, is known as

a. longitudinal stress

b. hoop stress

c. circumferential stress

d. ultimate stress.

6. The distance between terminal points computed from a subsidiary traverse run between them, is generally known, as

a. traverse leg

b. a base

c. traverse base

d. all the above.

7. In a precision traverse, included angles are measured by setting the vernier

a. to read zero exactly on back station

b. to read 5° exactly on back station

c. some where near zero and reading both verniers on back station

d. all the above.

8. If θ is the slope of the ground and l is the measured distance, the correction is

a. 2l sin2 θ/2

b. 2l cos2 θ/2

c. 2l tan2 θ/2

d. 2l cot2 θ/2.

9. Ranging is an operation of

a. reconnaissance

b. judging the distance

c. determination of slope

d. establishing intermediate points between terminals.

10. The method of reversal

a. is usually directed to examine whether a certain part is truly parallel or perpendicular to another

b. makes the erroneous relationship between parts evident

c. both (a) and (b)

d. neither (a) nor (b)..

11. If a tacheometer is fitted with an anal-latic lens

a. additive constant is 100, multiplying constant is zero

b. multiplying constant is 100, additive constant is zero

c. both multiplying and additive constants are 100

d. both multiplying and additive constants are 50.

12. The angle of intersection of a curve is the angle between

a. back tangent and forward tangent

b. prolongation of back tangent and forward tangent

c. forward tangent and long chord

d. back tangent and long chord.

13. To retard the initial setting time of cement, the compound responsible, is

a. Tricalcium silicate

b. Gypsum

c. Di-calcium silicate

d. Tri calcium aluminate.

14. Quick lime (or caustic lime)

a. is obtained by the calcination of pure lime stone

b. has great affinity to moisture

c. is amorphous

d. All the above.

15. The clay to be used for manufacturing bricks for a large project, is dugout and allowed to weather throughout

a. the monsoon

b. the winter

c. the summer

d. none of these.

16. The pigment used in paints for corrosive resistance, is

a. ferrous oxide

b. zinc white

c. red lead

d. gypsum.

17. Seasoning of timber is essential to remove

a. knots from timber

b. sap from timber

c. twisted fibre from timber

d. roughness of timber.

18. Pick up the correct statement from the following:

a. The low voltage porcelain is prepared by wet process

b. The high voltage porcelain is prepared by dry process

c. The low voltage porcelain is prepared by dry process

d. None of the above.

19. Transport of concrete by pumps, is done for a distance of

a. 200 m

b. 300 m

c. 400 m

d. none of these.

20. To obtain a very high strength concrete, use very fine grained

a. Granite

b. Magnetite

c. Barite

d. Volcanic scoria.

21. For the construction of thin R.C.C. structures, the type of cement to be avoided, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. sulphate resisting cement.

22. A flaky aggregate is said to be elongated if its length is

a. twice the mean size

b. thrice the mean size

c. four times the mean size

d. five times the mean size.

23. The lower water cement ratio in concrete, introduces

a. greater density and smaller permeability

b. improved frost resistance

c. greater wear resistance and improved bond strength

d. all the above.

24. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

25. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

26. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

27. For a base failure of a slope, depth factor

a. Df = 1

b. Df < 1

c. Df > 1

d. none of these.

28. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

29. A soil has bulk density 2.30 g/cm3 and water content 15 per cent, the dry density of the sample, is

a. 1.0 g/cm2

b. 1.5 g/cm3

c. 2.0 g/cm3

d. 2.5 g/cm3

30. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

31. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

32. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

33. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

34. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

35. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

36. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

37. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

38. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

39. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m.

40. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

41. To draw or measure angles .......... is used.

a. set-square

b. T-square

c. protractor

d. all of the above

42. Topographic map is used to represent the

a. artificial detail

b. natural detail

c. construction detail

d. all of the above

43. The shades and shadows are :

a. the same

b. the different

c. the result of object on the ground

d. the result of light on the ground

44. The scale should never be used as a ........ for drawing straight lines.

a. set-square

b. working edge

c. straight edge

d. none of the above

45. The sunk costs include :

a. a past expenditure

b. an unrecovered balance

c. an invested capital that cannot be retreived

d. All of these

46. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

47. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

48. Provision for professional code of conduct of engineers in Nepal is made in detail in:

a. NEC Act

b. NEC Rules

c. NEC bye-laws

d. NEC ordience

49. During construction price variation clause in a contract applies to

a. increase in cost of important materials only

b. variation in the cost of material labour and fuel

c. variation in total cost of the project estimated as lump sum figure

d. the rate of inflation

50. The space required for living the worker is 15m with considering the maximum height of room above floor level.

a. 3m

b. 3.5m

c. 4m

d. 5 m