***Engineer***

1. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

2. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

3. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

4. A column is said to be of medium size if its slenderness ratio is between

a. 32 and 120

b. 120 and 160

c. 160 and 180

d. 180 and 200

5. For structural analysis, Maxwell's reciprocal theorem can be applied to :

a. plastic structures

b. elastic structures

c. symmetrical structures

d. all the above.

6. The orthographical projection of a traverse leg upon the reference meridian, is known as

a. departure of leg

b. latitude to the leg

c. co-ordinate of the leg

d. bearing of the leg.

7. The bearing of lines OA and OB are 16° 10' and 332° 18', the value of the included angle BOA is

a. 316° 10'

b. 158° 28'

c. 348° 08'

d. 43°  52'

8. Correction per chain length of 100 links along a slope of α radians, is

a. 100 α2

b. 100 α

c. 100 α3

d. 100 α-1.

9. For true difference in elevations between two points A and B, the level must be set up

a. at any point between A and B

b. at the exact mid point of A and B

c. near the point A

d. near the point B.

10. Pick up the correct statement from the following :

a. to locate a gross error in bearing that may exist in controlled theodolite traverse, we may plot the traverse from each end. The traverse station having the same coordinates by each route is the one where the error lies

b. to locate a gross error in bearing, in a controlled traverse, we plot the traverse and the station through which perpendicular to sector of the closing line passes is the station at which the error was made

c. to locate a gross error due to taping in a controlled traverse, we plot the traverse to a convenient scale. The bearing of the closing error will be approximately the same as that of the leg in which the gross error consists

d. all the above.

11. While measuring with a metallic tape of 30 m length pull should be applied

a. 1 kg

b. 2 kg

c. 3 kg

d. 4 kg

12. The difference in the lengths of an arc and its subtended chord on the earth surface for a distance of 18.2 km, is only

a. 1 cm

b. 5 cm

c. 10 cm

d. 100 cm.

13. Mastic asphalt is normally used for

a. sound insulation

b. water proofing

c. fire proofing

d. none to these.

14. The commonly used colour pigment in paints, is

a. carbon black

b. iron oxide

c. lamp black

d. all the above.

15. The cracks which extend from bark towards the sap wood in the cross section of a tree, are called

a. radial shakes

b. star shakes

c. heart shakes

d. cup shakes.

16. Pick up the correct statement from the following:

a. For mass concrete structures subjected to wetting and drying, the water ratio should be 0.55

b. For thin structures which remain continuously under water, the water-cement ratio by weight should be 0.55

c. For massive concrete structures which remain continuously under water, the water cement ratio by weight should be 0.65

d. All the above.

17. Lacquer paints

a. are less durable as compared to enamel paints

b. consist of resin and nitro-cellulose

c. contain alcohol as thinner

d. all the above.

18. Cement paints usually

a. contain 5% to 10% colour pigments

b. are prepared with white cement

c. contain 5% sodium chloride

d. all the above.

19. Concrete mainly consists of

a. aggregates

b. admixture

c. water

d. all the above.

20. Internal friction between the ingredients of concrete, is decreased by using

a. fine aggregates

b. rich mix

c. more water and coarse aggregates

d. none of these.

21. Curing a concrete for long period ensures better

a. strength

b. water resistance

c. water tightness and durability

d. all the above.

22. The ratio of the length to breadth of a wooden float, is

a. 5.5

b. 6.5

c. 7.5

d. 8.5

23. For given workability the grading requiring the least amount of water is one that gives

a. greatest surface area for the given cement and aggregates

b. least surface area for the given cement and aggregates

c. least weight for the given cement and aggregates

d. greatest weight for the given cement and aggregates

24. The shear strength of a soil

a. is proportional to the cohesion of the soil

b. is generally known as the strength of the soil

c. is proportional to the tangent of the angle of internal friction

d. all the above.

25. The void ratio of a soil sample decreases from 1.50 to 1.25 when the pressure is increased from 25 tonnes/m2 to 50 tonnes/m2, the coefficient of compressibility is

a. 0.01

b. 0.02

c. 0.05

d. 0.001

26. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

27. A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

28. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

29. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

30. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

31. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

32. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

33. For the network shown in the given figure, the expected time for the activity

a. 1-2 is 4

b. 2-3 is 7

c. 3-4 is 8

d. all the above.

34. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

35. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

36. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

37. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

38. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

39. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

40. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

41. Free hand sketching can be done only when, one has

a. proper proficiency

b. good practice

c. proper patience et

d. all of the above

42. The plane on which perspective is formed is called:

a. horizontal plane

b. picture plane

c. ground plane

d. central plane

43. Engineers prefer to make ......... sketches than to pictorial sketches to clear the shape of an object

a. orthographic

b. perspective

c. auto CAD

d. all of the above

44. The drawing prepared by a pencil and without use of drawing instrument is called

a. isometric drawing

b. orthographic projection

c. cadastral

d. free hand sketching

45. The construction manager uses the estimate of the project

a. to advise the architect/engineer regarding design cost parameter especially in value engineering analysis.

b. to develop bids on the project.

c. to control the project during its construction.

d. All of these

46. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

47. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. None of these

48. The purpose of bid evaluation is to determine

a. lowest bid

b. responsive bid

c. substantially responsive bid

d. lowest evaluated substantially responsive bid

49. Ethics as a normative science needs code of conduct and guidelines to maintaining high level of standard of good behavior in the society. This is applicable to :

a. Lawers

b. doctors

c. Engineers

d. all of the above

50. "Corrupt practice" means

a. offering/receiving anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above