***Engineer***

1. Pick up the correct statement from the following:

a. In a loaded beam, the moment at which the entire section of the beam becomes fully plastic, is called plastic moment

b. In a fully plastic stage of the beam, the neutral axis divides the section in two sections of equal area

c. The ratio of plastic moment to the yield moment, is called shape factor

d. All the above.

2. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

3. Principal planes are subjected to

a. normal stresses only

b. tangential stresses only

c. normal stresses as well as tangential stresses

d. none of these.

4. A diagram which shows the variations of the axial load for all sections of the span of a beam, is called

a. shear force diagram

b. thrust diagram

c. stress diagram

d. none of these

5. Reactions at the supports of a structure can be determined by equating the algebraic sum of

a. horizontal forces to zero

b. vertical forces to zero

c. moment about any point to zero

d. all the above.

6. Which of the following introduces an error of about 1 in 1000 if 20 m chain is used

a. one end of the chain 0.9 m off the line

b. one end of chain 0.9 m higher than the other

c. middle of the chain 0.45 m off the line

d. all the above.

7. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

8. A level when set up 25 m from peg A and 50 m from peg B reads 2.847 on a staff held on A and 3.462 on a staff held on B, keeping bubble at its centre while reading. If the reduced levels of A and B are 283.665 m and 284.295 m respectively, the collimation error per 100 m is

a. 0.015 m

b. 0.030 m

c. 0.045 m

d. 0.060 m

9. If the declination of the needle is 10° W

a. each of the whole circle reckoning has to be micros by 10°

b. in the quadrantal method, the correction is positive in the 1st and 3rd quadrants

c. in the quadrantal method, the corrections is negative in 2nd and 4th quadrants

d. all the above.

10. Perpendicularity of an offset may be judged by eye, if the length of the offset is

a. 5 m

b. 10 m

c. 15 m

d. 20 m.

11. When the bubble of the level tube of a level, remains central

a. line of sight is horizontal

b. axis of the telescope is horizontal

c. line of collimation is horizontal

d. geometrical axis of the telescope is horizontal.

12. The radius of a simple circular curve is 300 m and length of its specified chord is 30 m. The degree of the curve is

a. 5.73°

b. 5.37°

c. 3.57°

d. 3.75°.

13. During smelting process, the combination of fuel in the furnace

a. forms carbon dioxide

b. carbon dioxide with carbon forms carbon mono-oxide

c. carbon mono-oxide reacts with Fe2O3 to form iron and liberates CO2

d. all the above.

14. Porcelain is used as :

a. electric insulators

b. storage vessels

c. reactor chambers

d. All of the above.

15. In a rock calcium carbonate predominates. State whether it is :

a. Sillicious rock

b. Argillaceous rock

c. Calcareous rock

d. None of these.

16. Whitworth compressed steel is obtained when molten steel is subjected to a pressure of

a. 9 kg/mm2

b. 13 kg/mm2

c. 15 kg/mm2

d. 10 kg/mm2.

17. Turpentine oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

18. Plastic

a. consists of natural or synthetic binders

b. finished products are rigid and stable at normal temperature

c. is capable of flow when necessary heat and pressure are applied

d. All the above.

19. Pick up the correct statement from the following i.

a. Higher workability indicates deficiency of sand

b. If the concrete mix is dry, the slump is zero

c. Concrete mix having zero slump, is unsuitable for high strength.

d. All the above.

20. The bulk density of aggregates, is generally expressed as

a. kg/cubic metre

b. kg/litre

c. g/cm3

d. none of these.

21. Le-Chatelier's apparatus is used for testing

a. soundness of cement

b. hardness of cement

c. strength of cement

d. durability of cement.

22. Curing

a. reduces the shrinkage of concrete

b. preserves the properties of concrete

c. prevents the loss of water by evaporation

d. all of the above.

23. Curing a concrete for long period ensures better

a. strength

b. water resistance

c. water tightness and durability

d. all the above.

24. On wetting, cohesive soils,

a. loose permeability

b. gain shear strength

c. loose elasticity

d. decrease their shear strength.

25. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

26. Under-reamed piles are generally

a. driven piles

b. bored piles

c. precast piles

d. all the above.

27. A phreatic line is defined as the line within a dam section below which there are

a. positive hydrostatic pressure

b. negative hydrostatic pressure

c. negative equipotential lines

d. none of these.

28. Through a point in a loaded soil, the principal stress is maximum on

a. minor principal plane

b. intermediate principal plane

c. major principal plane

d. none of these.

29. Pick up the clay soil group which does not swell when wet from the following :

a. Kaolinite group

b. Illite group

c. Vermiculite group

d. Montrorillonite group.

30. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

31. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

32. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

33. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

34. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

35. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

36. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

37. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

38. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

39. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

40. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

41. Which is not pictorial projection?

a. oblique projection

b. orthographic projection

c. trignometric

d. axonometric projection

42. The projection of any feature on the horizontal plane is called

a. plan

b. elevation

c. map

d. all of the above

43. Which pencil is used for thinner line?

a. H

b. B

c. 2B

d. 2H

44. In isometric view a circle is seen as :

a. a parabola

b. a hyperbola

c. a cone

d. an ellipse

45. A project construction cost estimate includes:

a. the labour and material cost

b. the equipment and over head cost

c. the profit of the contractor

d. All of these

46. A construction estimate is used

a. to judge tentatively or approximate value of the project

b. to produce a statement of the approximate cost

c. to decide an approximation of the value of the project and not the exact cost.

d. None of these

47. If a seller recovers his capital along with accumulated compensating interest not in one single lump sum payment but in periodical equal payments, over time :

a. Capital Recovery Annuity fs availed

b. Present work Annuity is availed

c. Sinking Fund Annuity is availed

d. Sinking Fund Annuity is availed

48. Engineering profession needs cooperation with the profession of:

a. medical professional

b. accountant

c. lawyer

d. all of the above

49. Which one type of membership is not categorized by NEC

a. general engineer

b. foreign engineer

c. professional engineer

d. supreintendent engineer

50. Ethics is the science which deals with

a. rightness of professional

b. wrong of professional

c. both (a) and (b) of above

d. none of the above