***Engineer***

1. Pick up the correct statement from the following:

a. In a loaded beam, the moment at which the entire section of the beam becomes fully plastic, is called plastic moment

b. In a fully plastic stage of the beam, the neutral axis divides the section in two sections of equal area

c. The ratio of plastic moment to the yield moment, is called shape factor

d. All the above.

2. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

3. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

4. The section modulus of a rectangular light beam 25 metres long is 12.500 cm3. The beam is simply supported at its ends and carries a longitudinal axial tensile load of 10 tonnes in addition to a point load of 4 tonnes at the centre. The maximum stress in the bottom most fibre at the mid span section, is

a. 13.33 kg/cm2 compressive

b. 26.67 kg/cm2 tensile

c. 26.67 kg/cm2 compressive

d. none of these.

5. The type of butt joints in common use, is :

a. single inverted V-butt joint

b. double V-butt joint

c. double U-butt joint

d. single V-butt joint.

6. Pick up the correct statement from the following :

a. the principal axes of both the lenses coincide the optical axis of the telescope

b. the lens nearer the object to be viewed is convex and is called objective

c. the lens nearer the eye is called eyepiece

d. all the above.

7. The bearing of lines OA and OB are 16° 10' and 332° 18', the value of the included angle BOA is

a. 316° 10'

b. 158° 28'

c. 348° 08'

d. 43°  52'

8. In precision theodolite traverse if included angles are read twice and the mean reading accepted using both verniers having a least count of 30". Assuming the instrument to be in perfect adjustment, linear measurements correct to 6 mm per 30 metre tape duly corrected for temperature, slope and sag, the angular error of closure not to exceed (where n is the number of traverse legs)

a. 50" n

b. 30" n

c. 60" n.

9. Correction per chain length of 100 links along a slope of α radians, is

a. 100 α2

b. 100 α

c. 100 α3

d. 100 α-1.

10. Keeping the instrument height as 1  m, length of staff 4 m, the up gradient of the ground 1 in 10, the sight on the up slope must be less than

a. 25 cm

b. 20 m

c. 45 m

d. 10 m

11. Pick up the correct statement from the following :

a. the apparent error on reversal is twice the actual error

b. the correction may be made equal to half the observed discrepancy.

c. the good results may be obtained from a defective instrument by reversing and taking the mean of two erroneous results

d. all the above.

12. The bearings of the lines AB and BC are 146° 30' and 68° 30'. The included angle ABC is

a. 102°

b. 78°

c. 45°

d. none of these.

13. Pig iron is manufactured from the ores by

a. dressing

b. calcination and roasting

c. smelting

d. all the above.

14. In stone masonry, stones (stratified rocks) are so placed that the direction of pressure to the plane of bedding is

a. right angles

b. 45°

c. 60°

d. parallel

15. Gniess is obtained from

a. igneous rocks

b. metamorphic rocks

c. sedimentary rocks

d. sedimentary metamorphic rocks.

16. According to ISI, bitumen is classified into

a. 4 grades

b. 6 grades

c. 8 grades.

d. 10 grades.

17. Pick up the correct statement from the following:

a. Impurities float on the molten iron as slag

b. The slag contains lime about 45%

c. The molten iron is made to run, in a long channel formed in sand called 'sow'

d. All the above.

18. The colour of statuary marble used for sculptor's work, is

a. blue

b. white

c. green

d. yellow

19. Workability of concrete mix with low water cement ratio is determined by

a. slump test

b. compaction factor test

c. flexural strength test

d. none of these.

20. Strength of concrete with passage of time

a. increases

b. decreases

c. fluctuates

d. remains constant.

21. Proper proportioning of concrete, ensures

a. desired durability

b. water tightness of the structure

c. resistance to water

d. all the above.

22. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

23. To ensure constant moisture content in aggregates

a. height of each aggregate pile should not exceed 1.50 m

b. aggregate pile should be left for 24 hours before aggregates are used

c. conical heaps of aggregates should be avoided to prevent moisture variation

d. all the above.

24. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

25. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

26. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

27. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

28. The Westergaard analysis is used for

a. sandy soils

b. cohesive soils

c. stratified soils

d. clayey soils.

29. The angle of internal friction of clays, is usually

a. 0° to 5°

b. 5° to 20°

c. 20° to 30°

d. 30° to 45°

30. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

31. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

32. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

33. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

34. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

35. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

36. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

37. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

38. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above

39. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

40. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

41. In isometric view a circle is seen as :

a. a parabola

b. a hyperbola

c. a cone

d. an ellipse

42. The main objective of writing letters/alphabets on the drawing is to make the drawing

a. more fantastic

b. more informative

c. more expensive

d. all of the above

43. The isometric length of the edge of the cube is

a. more than actual length

b. qual to actual length

c. less than actual length

d. all of the above

44. In full sectional view, the object is imagined to be cut off

a. one third

b. one half

c. one fourth

d. one fifth

45. Annuities involve:

a. all payments of equal amount

b. payment at equal time intervals

c. payments at the end of periods.

d. All of these

46. The person desires to pay off the amount in 10 equal annual instalments. The amount of each instalment is :

a. Rs 5638

b. Rs 6638

c. Rs 7738

d. None of these

47. Present worth Annuity (PWA) is generally known as

a. Premium annuities

b. Income annuities

c. Future annuities

d. All of these

48. For the registration of general engineer tha amount of fee required is:

a. NRS 2000/-

b. NRS 3000/-

c. NRS 3500/-

d. NRS 4000/-

49. Provision for professional code of conduct of engineers in Nepal is made in detail in:

a. NEC Act

b. NEC Rules

c. NEC bye-laws

d. NEC ordience

50. As per the public Procurement Regulation, 2064 of Nepal, what is the maximum age for a technical proposal for consulting services ?

a. 90%

b. 80%

c. 70%

d. 60%