***Engineer***

1. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

2. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

3. For calculating the allowable stress of long columns  is the empirical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula

4. The areas of cross-section of a square beam and a circular beam subjected to equal bending moments, are same.

a. square beam is more economical

b. both the beams are equally strong

c. both the beams are equally economical

d. none of these.

5. Pick up the correct statement from the following :

a. A ductile material has large plastic zone

b. A brittle material has no plastic zone

c. A rigid material has no plastic zone

d. All the above.

6. The sensitiveness of a level tube decreases if

a. radius of curvature of its inner surface is increased

b. diameter of the tube is increased

c. length of the vapour bubble is increased

d. both viscosity and surface tension are increased.

7. The angle of intersection of a contour and a ridge line, is

a. 30°

b. 45°

c. 60°

d. 90°.

8. In a theodolite

a. the telescope axis is perpendicular to transit axis

b. the axis of rotation is perpendicular to transit axis

c. the telescope axis, the transit axis and the rotation axis pass through the centre of theodolite

d. all the above.

9. Diurnal variation of magnetic declination is

a. greater at equator than nearer the poles

b. less at equator than nearer the poles

c. less in summer than in winter

d. same at all latitudes and during different months.

10. In levelling operation

a. if first reading is more than second, it represents a rise

b. if  first reading is less than second, it represents a fall

c. if  second reading is less than first, it represents a fall

d. both (b) and (c).

11. A well conditioned triangle has no angle less than

a. 20°

b. 30°

c. 45°

d. 60°.

12. The radius of curvature of the arc of the bubble tube is generally kept

a. 10 m

b. 25 m

c. 50 m

d. 100 m

13. Fibre boards can be

a. distempered

b. painted

c. painted and distempered

d. used for furniture.

14. The stones obtained by blasting are used as

a. ballast in railways

b. aggregates for concrete

c. roadmetal

d. All the above.

15. The portion of a brick cut to form angles other than right angles in plan, is known as

a. queen closer

b. king closer

c. closer

d. squint brick.

16. Water paint is a

a. colour wash

b. whiting

c. distemper

d. all the above.

17. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

18. For obtaining vinyl chloride acetate, the method used, is

a. addition polymerization

b. condensation polymerization

c. co-polymerization

d. none of these.

19. Batching error means inaccuracy in the quantity of

a. aggregates

b. cement

c. water

d. all the above.

20. Pick up the incorrect statement from the following:

a. A rich mix of concrete possesses higher strength than that a lean mix of desired workability with excessive equantity of water

b. The strength of concrete decreases as the water cement ratio increases

c. If the water cement ratio is less than 0.45, the concrete is not workable and causes honey-combed structure

d. Good compaction by mechanical vibrtions, increases the strength of concrete

21. Pick up the correct statement from the following:

a. Gypsum (calcium sulphate) acts as an accelerator

b. Gypsum (calcium sulphate) acts as a retarder

c. Calcium chloride acts as an accelerator

d. Both (c) and (d).

22. Pick up the incorrect statement from the following:

a. Tricalcium silicate (C3S) generates more heat of hydration

b. Tricalcium silicate (C3S) develops early strength

c. Tricalcium silicate (C3S) has more resistance to sulphate attack

d. None of these.

23. The rock which is not calcareous, is :

a. macl

b. chalk

c. laterite

d. none of these.

24. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

25. Pick up the correct statement from the following:

a. The dry density reduces by addition of water after attaining optimum moisture content

b. The line joining the peak of three moisture content graphs obtained by using three compactive energies, is called line of optimus

c. Well graded coarse grained soils can be compacted to a very high density as compared to fine grained soils

d. All the above. .

26. Compression of soil occurs rapidly if voids are filled with

a. air

b. water

c. partly with air and partly with water

d. none of these.

27. The expression  ρω is used for

a. dry density

b. bulk density

c. degree of saturation

d. optimum water content.

28. The maximum pressure which a soil can carry without shear failure, is called

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

29. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

30. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

31. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

32. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

33. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

34. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

35. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

36. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

37. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

38. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

39. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

40. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

41. In orthographic projection, projector lines are

a. vertical lines

b. inclined lines

c. horizontal lines

d. none of above

42. Angles in multiples of 15 degree are constructed by the combined use of

a. T-square and set-square

b. set-square and scale

c. scale and french curve

d. french curve and erasing shield

43. In a topographic drawing the symbols representing natural features are drawn by free hand while artificial works are drawn by

a. free hand

b. mechanically

c. both (a) and (b) of above

d. none of the above

44. Axonometric drawing is a drawing like

a. parallel to the plane

b. perpendicular to the plane

c. inclined to the plane

d. all of the above

45. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

46. The sunk costs include :

a. a past expenditure

b. an unrecovered balance

c. an invested capital that cannot be retreived

d. All of these

47. Pick up the correct statement from the following:

a. An annuity is a series of equal payments occurring at equal period of time.

b. Annuity is called an equal payment or uniform payment series.

c. An annuity may have periods of time of any length but should always be of equal length.

d. All the above.

48. The space required for living the worker is 15m with considering the maximum height of room above floor level.

a. 3m

b. 3.5m

c. 4m

d. 5 m

49. Provision for professional code of conduct of engineers in Nepal is made in detail in:

a. NEC Act

b. NEC Rules

c. NEC bye-laws

d. NEC ordience

50. Procurement of services means:

a. buying logistics

b. hiring consultant

c. renting house

d. none of the above