***Engineer***

1. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

2. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration

3. Maximum principal stress theory for the failure of a material at elastic point, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

4. The materials which have the same elastic properties in all directions, are called

a. isotropic

b. brittle

c. homogeneous

d. hard.

5. The region of the cross-section of a column in which compressive load may be applied without producing any tensile stress, is known as the core of the cross-section. In circular columns the radius of the core, is

a. one-third of the radius

b. one-quarter of the radius

c. one-fifth of the radius

d. one-sixth of the radius.

6. The defect of a lens whereby rays of white light proceeding from a point get dispersed into their components and conveyed to various foci, forming a blurred and coloured image is known as

a. chromatic aberration

b. spherical aberration

c. astigmatism

d. coma.

7. If whole circle bearing of a line is 120°, its reduced bearing is

a. S 20° E

b. S 60° E

c. N 120° E

d. N 60° E.

8. Geodetic surveying is undertaken

a. for developing the science of geodesy

b. making use of most accurate instruments and methods of observation

c. for determination of accurate positions on the earth's surface of system of control points

d. all the above.

9. In an internal focusing telescope

a. the objective is at a fixed distance from the diaphragm

b. the focusing is done by the sliding of a divergent lens.

c. the focusing divergent lens is situated at about the middle of the tube

d. all the above.

10. Bergchrund is a topograhical feature in

a. plains

b. water bodies

c. hills

d. glaciated region

11. Tilt of the staff in stadia tacheometry increases the intercept if it is

a. away from the telescope pointing down hill

b. towards the telescope pointing up-hill

c. away from the telescope pointing up-hill

d. none of these.

12. Surveys which are carried out to depict mountains, rivers, water bodies, wooded areas and other cultural details, are known as

a. city surveys

b. topographical surveys

c. guide map surveys

d. plane surveys.

13. Bullet proof glass is made of thick glass sheet sandwiched by a layer of

a. steel

b. stainless steel

c. high test plastic

d. chromium plate.

14. Mastic asphalt is

a. water proof

b. fire proof

c. elastic

d. all the above.

15. Pick up the correct statement from the following:

a. Lime is available by dissolving calcium carbonate in water

b. Lime is available by calcining calcium carbonate at 900°C

c. Lime is nothing but calcium chloride

d. None of these.

16. A piece of timber whose thickness and width are respectively 5 cm and 10 cm is called

a. slate

b. plank

c. board

d. strip.

17. Lime stones are generally known as

a. aqueous rocks

b. sedimentary rocks

c. stratified rocks

d. all the above.

18. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

19. Joints in concrete structures, are provided

a. to reduce the tensile stresses likely to be developed due to evaporation of water

b. to minimise the change in the dimensions of the slab

c. to minimise the necessary cracking

d. all the above.

20. Non-uniform compaction may cause the concrete

a. porous

b. non-homogeneous

c. reduced strength

d. all the above.

21. For the construction of thin R.C.C. structures, the type of cement to be avoided, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. sulphate resisting cement.

22. Sand requiring a high water cement ratio, belongs to

a. Zone I

b. Zone II

c. Zone III

d. Zone IV.

23. For preparing ordinary concrete, the quantity of water used, is

a. 10% by weight of aggregates plus 10% of weight of cement

b. 5% by weight of aggregates plus 30% of weight of cement

c. 30% by weight of aggregates plus 10% of weight of cement

d. none of these.

24. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

25. Pick up the correct statement from the following:

a. A soil having pH value more than 7 is an acidic soil

b. A soil having pH value less than 7 is an acidic soil

c. A soil having pH value more than 7 is an alkaline soil

d. A soil containing chemicals for the manufacture of portland cement is preferred.

26. Pick up the incorrect definition from the following:

a. The rotation of soil particles into stable state while remoulding, is known as the thiostropy of soil

b. The water content at which a soil changes from the liquid state to solid state, is known liquid limit of the soil

c. The water content at which a soil flows, is known plastic limit of the soil

d. None of these.

27. A critical hydraulic gradient may occur when

a. flow is in upward direction

b. seepage pressure is in upward direction

c. effective pressure is zero

d. all the above.

28. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

29. A soil mass coated with a thin layer of paraffin weighs 460 g. When immersed, it displaces 299 cc of water. The weight of paraffin is 10 g. If specific gravity of solids is 2.5 and that of paraffin 0.9, the void ratio of soil, is

a. 0.55

b. 0.60

c. 0.65

d. 0.70

30. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

31. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

32. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

33. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

34. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

35. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

36. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

37. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

38. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

39. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

40. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

41. As per IŞI recommended size of drawing board is/are

a. B0 (1000x 1500m)

b. B1 (700x 1000m)

c. B2 (500x700m)

d. all of the above

42. Free hand sketch is normally used for

a. concept

b. final

c. discussion

d. none of the above

43. Find the RF for scale 1cm= 25 km

a. 1/250

b. 1/2500

c. 1/25000

d. 1/2500000

44. Perspective drawing is done for

a. not for civil engineers

b. show details of market products

c. both (a) and (b) of above

d. none of the above

45. Pick up the correct statement from the following:

a. Uniform series present worth factor =

b. Sinking fund factor =

c. Capital recovery factor =  where letters carry their usual meanings.

d. All of these

46. The financial analysis helps to judge:

a. The operational efficiency of the firm

b. The financial position of the firm.

c. Both (a) and (b)

d. Neither (a) nor (b)

47. The estimate based on a detailed quantity survey and furnishes the most accurate and reliable estimate possible is known as

a. Conceptual estimate

b. Definitive estimate

c. Probabilistic estimate

d. None of these

48. In Nepal, which of the following act contains the provisions on health and safety of the workers?

a. contract act

b. industry act

c. labour act

d. all of the above

49. Which one is not concerned with the NEC

a. licensing (Registration) of Engineers

b. accreditation of certificates of academic qualifications

c. recognition of the academic institution

d. authority of opening engineering college

50. Five members are elected from Nepal Engineers Association members to represent in Nepal Engineering Council every:

a. 2 years

b. 3 years

c. 4years

d. 5years