***Engineer***

1. Flat spiral springs

a. are supported at outer end

b. are wound by applying a torque

c. are used in clock-work mechanism

d. all the above.

2. Pick up the correct statement from the following:

a. Hoop strain of the walls of a cylinder due to liquid is

b. Longitudinal strain in the walls of a cylinder due to liquid is

c. Volumetric change in the cylinderdue to liquid is

d. All the above.

3. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

4. A joint of a frame is subjected to three tensile force P, Q and R equally inclined to each other. If P is 10 tonnes, the other forces will be

a. R + 10 tonnes and Q = zero

b. Q + R = 10 tonnes

c. Q - R = zero

d. Q and R each is equal to 10 tonnes.

5. A reinforced concrete beam is assumed to be made of

a. homogeneous material

b. heterogeneous material

c. isotropic material

d. none of these.

6. If + 0.8% grade meets - 0.7% grade and the rate of change of grade for 30 m distance is 0.05, the length of the vertical curve will be

a. 700 m

b. 800 m

c. 900 m

d. 1000 m

7. Contours of different elevations may cross each other only in the case of

a. an over hanging cliff

b. a vertical cliff

c. a saddle

d. an inclined plane.

8. Volume of the earth work may be calculated by

a. end areas

b. Prismoidal formula

c. Trapezoidal

d. all the above.

9. In chain surveying tie lines are primarily provided

a. to check the accuracy of the survey

b. to take offsets for detail survey

c. to avoid long offsets from chain lines

d. to increase the number of chain lines.

10. Rankine's deflection angle in minutes is obtained by multiplying the length of the chord by

a. degree of the curve

b. square of the degree of the curve

c. inverse of the degree of the curve

d. none of these.

11. In quadrantal bearing system, back bearing of a line may be obtained from its forward bearing, by

a. adding 180°, if the given bearing is less than 180°

b. subtracting 180°, if the given bearing, is more than 180°

c. changing the cardinal points, i.e. substituting N for S and E for W and vice-versa

d. none of these.

12. The 10 mm markings on a levelling staff placed at 20 m are separated by

a.  radian

b.  radian

c.  radian

d.  radian.

13. Portland pozzolana cement possesses

a. lower heat of hydration

b. lower shrinkage on drying

c. water tightness

d. all the above.

14. Pick up the correct statement from the following:

a. During rusting, first ferrous bicarbonates are formed

b. On further oxidation ferrous bicarbonates get converted to ferric bicarbonates

c. Ultimately hydrated ferric oxide is formed during rusting and carbon dioxide gets liberated

d. All the above.

15. The minimum compressive strength of Ilnd class bricks should be

a. 75 kg/cm2

b. 90 kg/cm2

c. 100 kg/cm2

d. 120 kg/cm2

16. The usual percentages of clay and metal in cermet are :

a. 50%, 50%

b. 60%, 40%

c. 70%, 30%

d. 80%, 20%.

17. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

18. In stone masonry, stones (stratified rocks) are so placed that the direction of pressure to the plane of bedding is

a. right angles

b. 45°

c. 60°

d. parallel

19. The increased cohesiveness of concrete, makes it

a. less liable to segregation

b. more liable to segregation

c. more liable to bleeding

d. more liable for surface scaling in frosty weather

20. If the various concrete ingredients i.e. cement, sand and aggregates are in the ratio of 1:3:6, the grade of concrete, is

a. M 100

b. M 150

c. M 200

d. M 250

21. To obtain cement dry powder, lime stones and shales or their slurry, is burnt in a rotary kiln at a temperature between

a. 1200° and 1300°C

b. 1300° and 1400°C

c. 1400° and 1500°C

d. 1500° and 1600°C

22. Pick up the incorrect statement from the following:

a. The axis of the mould is perpendicular to the end faces

b. The internal surface of the mould is kept clean and free from set cement

c. The mould is in the form of a frustum of hexagonal pyramid

d. None of these.

23. Pick up the correct proportions of chemical ingredients of cement

a. Lime : Silica : Alumina : Iron oxide : 63 : 22 : 6 : 3

b. Silica : Lime : Alumina : Iron oxide : 63 : 22 : 6 : 3

c. Alumina : Silica : Lime : Iron oxide : 63 : 22 : 6 : 3

d. Iron oxide : Alumina : Silica : Lime : 63 : 22 : 6 : 3

24. An unsaturated 100 cm3 sample of soil weighs 190 g. If its dried weight is 160 g, water content of the soil, is

a. 0.188

b. 0.288

c. 0.388

d. 0.488

25. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

26. The lateral earth pressure on a retaining wall

a. proportional to the depth of the soil

b. proportional to the square of the depth of the soil

c. proportional to the internal friction of the soil

d. none of these.

27. Pick up the correct statement from the following:

a. Failure plane carries maximum shear stress

b. Failure plane does not carry maximum shear stress

c. Failure plane carries shear stress equal to maximum shear stress

d. None of these.

28. The Westergaard analysis is used for

a. sandy soils

b. cohesive soils

c. stratified soils

d. clayey soils.

29. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

30. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

31. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

32. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

33. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

34. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

35. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

36. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

37. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

38. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

39. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

40. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

41. The space between two sentences should be left equal to

a. height of letter

b. twice the ht. of letter

c. 1.5 times ht. of letter

d. none of the above

42. In case of aligned method of dimensioning, they can be readable form

a. bottoms and right side of drawing sheet

b. bottoms and left side of drawing sheet

c. bottoms and top side of drawing sheet

d. all of the above

43. Inking pen is used for drawing -- in ink

a. writing

b. curve

c. straight line

d. none of the above

44. The drawing prepared by a pencil and without use of drawing instrument is called

a. isometric drawing

b. orthographic projection

c. cadastral

d. free hand sketching

45. The CRF (ep) is also known as: [CRF(EP) - 8% - 7], where

a. 8% is the rate of interest per year

b. money is borrowed for n = 7 years

c. both (a) and (b)

d. Neither (a) nor (b)

46. Annuities involve:

a. all payments of equal amount

b. payment at equal time intervals

c. payments at the end of periods.

d. All of these

47. Liquidity ratios are used :

a. to measure a firms ability to meet short-cut obligations.

b. to compare short term obligations to short-term resources available to meet these obligations.

c. to obtain much insight into the present cash solvency of the firm and the firm's ability to remain solvent in the event of adversity.

d. All of these

48. Engineering profession needs cooperation with the profession of:

a. medical professional

b. accountant

c. lawyer

d. all of the above

49. "Corrupt practice" means

a. offering/receiving anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above

50. Engineering ethics is the field of applied ethics and system of moral principles that applt to be

a. practice engineering

b. responsibility engineering

c. management engineering

d. accountability engineering