***Engineer***

1. A cantilever of length 2 cm and depth 10 cm tapers in plan from a width 24 cm to zero at its free end. If the modulus of elasticity of the material is 0.2 x 106 N/mm2, the deflection of the free end, is

a. 3 mm

b. 4 mm

c. 5 mm

d. 6 mm.

2. A truss containing j joints and m members, will be a simple truss if

a. m = 2j - 3

b. j = 2m - 3

c. m = 3j - 2

d. j = 3m - 2

3. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above.

4. In a beam, the neutral plane

a. may be its centre

b. passes through the C.G. of the area of cross-section

c. does not change during deformation

d. none of these.

5. A member is balanced at its end by two inclined members carrying equal forces. For equilibrium the angle between the inclined bars must be

a. 45°

b. 60°

c. 90°

d. 120°

6. While surveying a plot of land by plane tabling, the field observations

a. and plotting proceed simultaneously

b. and plotting do not proceed simultaneously

c. and recorded in field books to be plotted later

d. all the above.

7. Straight, parallel and widely spaced contours represent

a. a steep surface

b. a flat surface

c. an inclined plane surface

d. curved surface.

8. If the declination of the needle is 10° W

a. each of the whole circle reckoning has to be micros by 10°

b. in the quadrantal method, the correction is positive in the 1st and 3rd quadrants

c. in the quadrantal method, the corrections is negative in 2nd and 4th quadrants

d. all the above.

9. The line of collimation method of reduction of levels, does not provide a check on

a. intermediate sights

b. fore sights

c. back sights

d. reduced levels.

10. The ratio of the linear displacement at the end of a line, subtended by an arc of one second to the length of the line, is

a. 1:206 300

b. 1:3440

c. 1:57

d. 1:100.

11. In tacheometrical observations, vertical staff holding is generally preferred to normal staffing, due to

a. ease of reduction of observations

b. facility of holding

c. minimum effect of careless holding on the result

d. none of these.

12. Mistakes which may produce a very serious effect upon the final results arise due to

a. in attention

b. in experience

c. carelessness

d. all of these.

13. For a good building stone, its specific gravity should the greater than

a. 1.5

b. 1.7

c. 2.2

d. 2.7.

14. Dorry's testing machine is used for

a. crushing test of stone

b. hardness test of stone

c. impact test of stone

d. water absorption test.

15. Pick up the correct statement from the following:

a. Steel produced by open hearth process is milder than that obtained by the bessemer process

b. Engineers prefer open hearth steel for structural purpose as it is more homogenous

c. Basic Bessemer process is suitable for converting poor ore containing a large proportion of sulphur and phosphorus into steel

d. all the above.

16. The thermosetting plastic

a. becomes rigid when moulded at suitable pressure and temperature

b. at 127°C to 177°C permanently set and further application of heat does not soften it

c. charrs at 343°C

d. All the above.

17. Pick up the correct statement from the following:

a. The distinct plane of division along which a stone can easily be split, is called natural bed of stone

b. The natural bed of sedimentary rocks is along the planes of stratification

c. The natural bed of igneous rocks is not defined

d. All the above.

18. Rapid hardening cement attains early strength due to

a. larger proportion of lime grounded finer than normal cement

b. lesser proportion of lime grounded coarser than normal cement

c. lesser proportion of lime grounded finer than normal cement

d. larger proportien of lime grounded coarser than normal cement

19. Pozzolanic properties exist in

a. fly ash

b. pumicite

c. diatomaceous clay

d. all the above.

20. The light weight aggregates are obtained from

a. sedimentary rocks

b. metamorphic rocks

c. igneous rocks

d. volcanic source.

21. If a grading curve is horizontal bet-wen the portions of 20 mm I.S. Sieve and 4.75 mm I.S. Sieve, the graded aggregates do not contain

a. 20 mm particles

b. 10 mm particles

c. 4.75 mm particles

d. all the above.

22. The strength and quality of concrete, depend upon:

a. surface area of aggregates

b. shape of aggregates

c. surface texture of aggregates

d. all the above.

23. A concrete having a slump of 6.5 cm, is said to be

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

24. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

25. If the unit weight of sand particles is 2.696 g/cc. and porosity in loose state is 44%, the critical hydraulic gradient for quick sand condition, is

a. 0.92

b. 0.93

c. 0.94

d. 0.95

26. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

27. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

28. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

29. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

30. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

31. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

32. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

33. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

34. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

35. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

36. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

37. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

38. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

39. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

40. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

41. In case of unidirectional method of dimensioning, they can be read from

a. bottom side of drawing sheet only

b. right side of drawing sheet only

c. top side of drawing sheet only

d. left side of drawing sheet only

42. In general structural drawings would typically include following information:

a. north point

b. plan, section and elevation

c. notes on specification

d. all of the above

43. Axonometric drawing is a drawing like

a. parallel to the plane

b. perpendicular to the plane

c. inclined to the plane

d. all of the above

44. In full sectional view, the object is imagined to be cut off

a. one third

b. one half

c. one fourth

d. one fifth

45. The construction manager uses the estimate of the project

a. to advise the architect/engineer regarding design cost parameter especially in value engineering analysis.

b. to develop bids on the project.

c. to control the project during its construction.

d. All of these

46. The financial analysis :

a. helps a share holder to compare the expected return on his investment in the firm against the expected return from other alternative investment.

b. helps a bank to know the financial position of the firm for granting a loan to the firm.

c. helps to judge the success of the firm's financial plans.

d. All of these.

47. Pick up the correct statement from the following:

a. Engineering economy is a collection of mathematical techniques which simplify economic comparisons

b. Engineering economy is a decision assistance tool by which one method will be chosen as the most economically one.

c. For understanding the engineering economy, one should be able to classify the basic terminology and fundamental concepts of economy.

d. All of these.

48. According to the Public Procurement Regulation, 2064, Who approves the prequalification criteria ?

a. concerned project chief

b. concerned department chief

c. concerned secretary

d. concerned minister

49. What percent of interest is taken for the advance amount paid to the contractor by employer?

a. 1% of advance amount

b. 2% of advance amount

c. 3% of advance amount

d. none of the above

50. The time between ordering of the goods or materials and receiving and placing them into use at the point of need is called

a. order-interval

b. lead time

c. usage time

d. delayed time