***Engineer***

1. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7

2. If ΣH and ΣV are the algebraic sums of the forces resolved horizontally and vertically respectively, and ΣM is the algebraic sum of the moments of forces about any point, for the equilibrium of the body acted upon

a. ΣH = 0

b. ΣV = 0

c. ΣM = 0

d. all the above.

3. A steel bar 5 m x 50 mm is loaded with 250, 000 N. If the modulus of elasticity of the material is 0.2 MN/mm2 and Poisson's ratio is 0.25, the change in the volume of the bar is :

a. 1.125 cm3

b. 2.125 cm3

c. 3.125 cm3

d. 4.125 cm2

4. Failure of riverted joints is due to

a. Tearing of plates between rivets

b. Shearing of rivets

c. Crushing of rivets

d. All the above.

5. The moment diagram for a cantilever carrying linearly varying load from zero at its free end and to maximum at the fixed end will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

6. It θ is the probable error of an observed bearing of a line of length l, the error over the whole length of the traverse of n lines of length l is

a. l n

b.  n

c. θ

d.  θ n

7. The bearing of AB is 190° and that of CB is 260° 30'. The included angle ABC, is

a. 80° 30'

b. 99° 30'

c. 70° 30'

d. none of these

8. If vertical angles of inclined sights do not exceed 10° and non-verticality of the staff remains within 1°, stadia system of tacheometric observations are made on

a. staff normal

b. staff vertical

c. staff normal as well as vertical

d. none of these.

9. To avoid large centering error with very short legs, observations are generally made

a. to chain pins

b. by using optical system for centering the theodolite

c. to a target fixed on theodolite tripod on which theodolite may be fitted easily

d. all the above.

10. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

11. Pick up the correct statement from the following :

a. the apparent error on reversal is twice the actual error

b. the correction may be made equal to half the observed discrepancy.

c. the good results may be obtained from a defective instrument by reversing and taking the mean of two erroneous results

d. all the above.

12. Profile levelling is usually done for determining

a. contours of an area

b. capacity of a reservoir

c. elevations along a straight line

d. boundaries of property

13. Mastic asphalt is

a. acid resisting material

b. non-corrosive material

c. corrosive material

d. heating-resisting material.

14. The rocks in which argil (or clay) predominates, are called

a. sillicious rocks

b. argillaceous rocks

c. calcareous rocks

d. igneous rocks.

15. Bessemer process is used for the manufacture of

a. Pig iron

b. cast iron

c. Wrought iron

d. Steel.

16. Acrylic is the name of

a. cellulose resin

b. alkyd resin

c. methyl metha crylate

d. cumarone-indene.

17. Index number expressing the relative sizes of both coarse and fine aggregates, is called

a. proportioning of aggregates

b. fineness modulus

c. grading of aggregates

d. none of these.

18. Clay and silt content in a good brick earth must be at least

a. 50%

b. 40%

c. 30%

d. 25%

19. The increased cohesiveness of concrete, makes it

a. less liable to segregation

b. more liable to segregation

c. more liable to bleeding

d. more liable for surface scaling in frosty weather

20. Hardening of cement occurs at

a. rapid rate during the first few days and afterwards it continues to increase at a decreased rate

b. slow rate during the first few days and afterwards it continues to increase at a rapid rate

c. uniform rate throughout its age

d. none of these.

21. Transport of concrete by pumps, is done for a distance of

a. 200 m

b. 300 m

c. 400 m

d. none of these.

22. The compaction of concrete, improves

a. density

b. strength

c. durability

d. all the above.

23. Pick up the correct statement from the following:

a. Expansion joints are provided to accommodate thermal expansion

b. Construction joints are provided to control shrinkage cracks

c. Expansion joints need not be provided in foundation concrete

d. All the above.

24. The maximum shear stress occurs on the filament which makes an angle with the horizontal plane equal to

a. 30°

b. 45°

c. 60°

d. 90°

25. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

26. The water content of soil is defined as the ratio of

a. volume of water to volume of given soil

b. volume of water to volume of voids in soil

c. weight of water to weight of air in voids

d. weight of water to weight of solids of given mass of soil.

27. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

28. A pile is being driven with a drop hammer weighing 1800 kg and having a free fall of 1.00 m. If the penetration with last blow is 5 mm, the load carrying capacity of the pile, -according to the Engineering News formula, is

a. 100 tonnes

b. 50 tonnes

c. 20 tonnes

d. 10 tonnes.

29. Maximum size of clay particles, is :

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm

d. 0.08 mm

30. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

31. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

32. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

33. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

34. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

35. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

36. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

37. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

38. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

39. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

40. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

41. Drawing is the .................of engineers.

a. language

b. tools

c. machine

d. all of the above

42. In drafting machine, which one does not follow :

a. levelling machine

b. T-squared

c. set square

d. divider

43. Which pencil is used for thinner line?

a. H

b. B

c. 2B

d. 2H

44. The maps which are on sufficient large scale to enable the individual features shown on the map to be identified on the ground by their shape and position are called :

a. topographical maps

b. geographical maps

c. geological maps

d. site plan

45. Pick up the correct statement from the following:

a. The ratios which show profitability in relation to sales and those which show profitability in relation to investment, are called profitability ratios.

b. The ratio of gross profit and net sales, is called profitability in relation to sales ratio.

c. The ratio of net profit after taxes to total assests is known as profitability in relation to investment ratio

d. All of these

46. Pick up the correct statement from the following:

a. The gross profit percentage is the average profit margin obtained on goods sold.

b. The relationship of contribution to sales is known as contribution ratio

c. The difference between sales and variable cost of sales, is called contribution.

d. All of these

47. Which method is adopted to develop an approximate or conceptual estimate for perimeter works for buildings from the following:

a. Cost per function method

b. Cost per square metre method

c. Cost per cubic metre method

d. Cost per linear metre method

48. "Corrupt practice" means

a. offering/receiving anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above

49. Which of the following category of Engineer is not mentioned in Engineering Council rule

a. Charted Engineer

b. General Engineer

c. Professional Engineer

d. Non-Nepali Regd. Engineer

50. For the fulfillment of his/her professional duties Engineer should:

a. Paramount the health of the public

b. paramount the safety of the public .

c. welfare of the public

d. all of the above