***Engineer***

1. In plastic analysis, the shape factor for a triangular section, is

a. 1.5

b. 1.34

c. 2.34

d. 2.5

2. Maximum shear stress theory for the failure of a material at the elastic limit, is known

a. Guest's or Trecas' theory

b. St. Venant's theory

c. Rankine's theory

d. Haig's theory

3. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm.

4. Beams of uniform strength are preferred to those of uniform section because these are economical for

a. large spans

b. heavy weights

c. light weights

d. short spans.

5. In a loaded beam, the point of con-traflexture occurs at a section where

a. bending moment is zero or changes sign

b. bending moment is maximum

c. shearing force is maximum

d. shearing force is minimum.

6. Pick up the correct statement from the following :

a. the eyepiece plays no part in defining the line of sight

b. the diaphragm plays no part in defining the line of sight

c. the optical centre of the objective plays no part in defining the line of sight

d. all of these.

7. The most reliable method of plotting a theodolite traverse, is

a. by consecutive co-ordinates of each station

b. by independent co-ordinates of each station

c. by plotting included angles and scaling off each traverse leg

d. by the tangent method of plotting.

8. The length of a traverse leg may be obtained by multiplying the latitude and

a. secant of its reduced bearing

b. sine of its reduced bearing

c. cosine of its reduced bearing

d. tangent of its reduced bearing.

9. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

10. In setting up a plane table at any station

a. levelling is done first

b. centering is done first

c. both levelling and centering are done simultaneously

d. orientation is done first.

11. The power of a lens

a. is reciprocal of its focal length

b. is positive if it is a convex lens

c. is negative if it is a concave lens

d. is measured in diopter.

12. One of the tacheometric constants is additive, the other constant, is

a. subtractive constant

b. multiplying constant

c. dividing constant

d. indicative constant.

13. Based on flow quality, the sequence of pipes is

a. A.C. pipes, G.I. pipes, C.I. pipes, PVC pipes

b. C.I. pipes, G.I. pipes, A.C. pipes, PVC pipes

c. C.I. pipes, G.I. pipes, PVC pipes, A.C. pipes

d. None of these.

14. Pick up the correct statement from the following: Method of sawing timber

a. tangentially to annual rings, is known as tangential method.

b. in four quarters such that each board cuts annual rings at angles not less than 45°, is known as quarter sawing method.

c. cut out of quarter logs, parallel to the medullary rays and perpendicular to annual rings, is known as radial sawing.

d. all the above.

15. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

16. Which one of the following polymers is obtained from condensation polymerization?

a. phenol formaldehyde

b. carbamide

c. melamine-formaldehyde

d. all of these.

17. Rapid hardening cement attains early strength due to

a. larger proportion of lime grounded finer than normal cement

b. lesser proportion of lime grounded coarser than normal cement

c. lesser proportion of lime grounded finer than normal cement

d. larger proportien of lime grounded coarser than normal cement

18. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

19. Workability improved by adding

a. hydrated lime

b. calcium chloride

c. bentonite

d. all the above.

20. For the construction of cement con-certe dams, the maximum permissible size of the aggregates, is

a. 40 mm

b. 50 mm

c. 60 mm

d. 70 mm

21. Pick up the incorrect statement from the following:

a. Admixtures accelerate hydration

b. Admixtures make concrete water proof

c. Admixtures make concrete acid proof

d. Admixtures give high strength

22. On a grading curve, the gap grading is represented by

a. a horizontal line

b. a vertical line

c. N.W. inclined line

d. N.E. inclined line

23. Pick up the incorrect statement from the following:

a. With passage of time, the strength of cement increases

b. With passage of time, the strength of cement decreases

c. After a period of 24 months, the strength of cement reduces to 50%

d. The concrete made with storage deteriorated cement, gains strength with time

24. The ratio of volume of air voids to the volume of total voids, is known as

a. air content

b. percentage air voids

c. percentage voids

d. porosity.

25. When drainage is permitted under initially applied normal stress only and full primarily consolidation is allowed to take place, the test is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

26. The ratio of the weight of given volume of soil solids to the weight of an equal volume of distilled water at the given temperature, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

27. The ratio of the volume of water present in a given soil mass to the total volume of its voids, is known

a. porosity

b. void ratio

c. percentage voids

d. degree of saturation.

28. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

29. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

30. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

31. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

32. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

33. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

34. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

35. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

36. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

37. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

38. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

39. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

40. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

41. During drawing stock of T-square is placed

a. adjoining the working edge of the board

b. adjoining the sliding edge of the board

c. surface of the board,

d. none of the above

42. The paper size of A, is

a. 297 x 420

b. 210 x 297

c. 420x 850

d. none of the above

43. "Perspective Drawing" is an approximate representation on a flat surface of an image.......

a. as it is perceived by the eye

b. as it is drawn on computer

c. of a free hand drawing

d. as it is felt by the hands

44. The isometric length of the edge of the cube is

a. more than actual length

b. qual to actual length

c. less than actual length

d. all of the above

45. Pick up the correct statement from the following:

a. The gross profit percentage is the average profit margin obtained on goods sold.

b. The relationship of contribution to sales is known as contribution ratio

c. The difference between sales and variable cost of sales, is called contribution.

d. All of these

46. Probabilistic estimating of a construction project includes:

a. Labour

b. Productivity

c. wage scale

d. All of these

47. The ratio of current assests to current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

48. Engineering ethics is the field of applied ethics and system of moral principles that applt to be

a. practice engineering

b. responsibility engineering

c. management engineering

d. accountability engineering

49. Workers are not allowed to work contiunously more than

a. 3hr

b. 5hr

c. 7hr

d. 4hr

50. The document announcing to prospective bidder that the owner is ready to receive bids is called

a. notice to bidders

b. instruction to bidders

c. bill of quantities

d. intension to acceptance