***Engineer***

1. The forces in the members of simple trusses, may be analysed by

a. graphical method

b. method of joints

c. method of sections

d. all the above.

2. A compound bar consists of two bars of equal length. Steel bar cross-section is 3500 mm2 and that of brass bar is 3000 mm2. These are subjected to a compressive load 100,000 N. If Eb = 0.2 MN/mm2 and Eb = 0.1 MN/mm2, the stresses developed are:

a. σb = 10 N/mm2, σs = 20 N/mm2

b. σb = 8 N/mm2, σs = 16 N/mm2

c. σb = 6 N/mm2, σs = 12 N/mm2

d. σb = 5 N/mm2, σs = 10 N/mm2

3. Pick up the correct statement from the following:

a. For a uniformly distributed load, B.M. curve is a parabola

b. For a load varying linearly, the shear force curve is a parabola

c. For a load varying linearly, the B.M. curve is a cubic parabola

d. All the above.

4. For the same height, the bottom width for no tension,

a. for triangular section is more than rectangular section

b. for rectangular section is more than triangular section

c. for triangular section is same as that of a rectangular section

d. none of these.

5. If a three hinged parabolic arch carries a uniformly distributed load on its entire span, every section of the arch resists.

a. compressive force

b. tensile force

c. shear force

d. bending moment.

6. Horizontal distances obtained tacheometerically are corrected for

a. slope correction

b. temperature correction

c. refraction and curvature correction

d. all the above.

7. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

8. Prismatic compass is considered more accurate than a surveyor's compass, because

a. it is provided with a sliding glass in the object vane

b. its graduations are in whole circle bearings

c. it is provided with a prism to facilitate reading of its graduated circle

d. both (c) and (d).

9. If a linear traverse follows a sharp curve round a large lake where it is difficult to have long legs, the accuracy of the traverse may be improved by

a. taking short legs

b. making repeated observations of angular and linear measurements

c. making a subsidiary traverse to determine the length of a long leg

d. all the above.

10. To avoid large centering error with very short legs, observations are generally made

a. to chain pins

b. by using optical system for centering the theodolite

c. to a target fixed on theodolite tripod on which theodolite may be fitted easily

d. all the above.

11. If whole circle bearing of a line is 120°, its reduced bearing is

a. S 20° E

b. S 60° E

c. N 120° E

d. N 60° E.

12. Tacheometric formula for horizontal distances using horizontal sights can also suitable be employed for inclined sights through θ by multiplying

a. the constants by cos2 θ

b. the constants by cos θ

c. the constants by sin θ

d. the multiplying constant by cos2 θ and additive constant by cos θ.

13. The commonly used base for iron and steel work, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

14. Dextrin is

a. starch glue

b. albumin glue

c. rubber based adhesive

d. none to these.

15. The process of decarbonising the pig iron completely and then adding proper percentage of carbon for manufacturing steel, is called

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

16. Jhumb bricks are

a. under burnt

b. over burnt

c. kutcha

d. none of these.

17. For construction of structures under water, the type of lime used, is

a. hydraulic lime

b. fat lime

c. quick lime

d. pure lime

18. In stone masonry, stones (stratified rocks) are so placed that the direction of pressure to the plane of bedding is

a. right angles

b. 45°

c. 60°

d. parallel

19. Setting time of cement increases by adding

a. gypsum

b. hydrogen peroxide

c. calcium chloride

d. sodium oxide

20. Hydration of cement is due to chemical action of water with

a. Tricalcium silicate and dicalcium silicate

b. Dicalcium silicate and tricalcium aluminate

c. Tricalcium aluminate and tricalcium alumino ferrite

d. All the above.

21. Under normal conditions using an ordinary cement, the period of removal of the form work, is :

a. 14 days for bottom slabs of spans 4.6 m and more

b. 21 days for bottom beams over 6 m spans

c. 2 days for vertical sides of columns

d. all the above.

22. The mixture of different ingredients of cement, is burnt at

a. 1200°C

b. 1400°C

c. 1600°C

d. 1800°C

23. The type of aggregates not suitable for high strength concrete and for pavements subjected to tension, is

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregate

24. When a cohesionless soil attains quick condition, it looses

a. shear strength

b. bearing capacity

c. both (a) and (b)

d. neither (a) nor (6).

25. If the specific gravity of a soil particle of 0.05 cm diameter is 2.67, its terminal velocity while settling in distilled water of viscosity, 0.01 poise, is

a. 0.2225 cm/sec

b. 0.2250 cm/sec

c. 0.2275 cm/sec

d. 0.2300 cm/sec

26. Pick up the cohesive soil from the following:

a. Red earth

b. Clay

c. Black cotton soil

d. Compacted ground.

27. The pressure that builds up in pore water due to load increment on the soil, is termed

a. excess pore pressure

b. excess hydrostatic pressure

c. hydrodynamic pressure

d. all the above.

28. Water content of a soil sample is the difference of the weight of the given sample at the given temperature and the weight determined after drying it for 24 hours at temperature ranging from

a. 90° to 95°C

b. 95° to 100°C

c. 103° to 105°C

d. 105° to 110°C

29. Pick up the correct statement from the following:

a. When water table is above the base of a footing, the dry weight m should be used for soil below water table

b. When water table is located somewhat below the base of a footing, the elastic wedge is partly of moist soil and partly of submerged soil, and a suitable reduction factor is used

c. When water table is just at the base of the footing, no reduction factor is used

d. None of these.

30. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

31. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

32. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

33. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

34. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

35. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

36. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

37. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

38. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

39. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

40. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

41. The drawing for municipality is done at scale

a. 1" = 8"

b. 1:50

c. 1:100

d. both (a) and (c)

42. Free hand sketch is normally used for

a. concept

b. final

c. discussion

d. none of the above

43. Inking pen is used for drawing -- in ink

a. writing

b. curve

c. straight line

d. none of the above

44. The purpose of making the drawing board with seasoned soft wood

a. easy to carry

b. for fixing of drawing paper with the help of drawing pin

c. to match with T-square

d. all of the above

45. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

46. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. All of these

47. Pick up the correct statement from the following:

a. Both the numerator and denominator of financial ratios come directly from the balance sheet.

b. Income statement ratios compare one 'flow' item from the income statement to another flow item from the income statement.

c. Income statement ratios compare a flow item from the income statement to another flow item form the income statement

d. All of these

48. Profession is defined as

a. systematic knowledge through specialized training

b. synonym to job or occupation

c. types of service to needy people

d. all of above

49. The minimum qulification required for a general engineer is

a. diploma ine engg.

b. bachelor in engg.

c. master in engg.

d. all of the above

50. Ethics is the science which deals with

a. rightness of professional

b. wrong of professional

c. both (a) and (b) of above

d. none of the above