***Engineer***

1. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

2. The ratio of maximum and average shear stresses on a rectangular section, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

3. The strain energy stored in a spring when subjected to greatest load without being permanently distorted, is called

a. stiffness

b. proof resilience

c. proof stress

d. proof load.

4. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

5. The shear stress at any section of a shaft is maximum

a. at a distance r/2 from the centre

b. at the top of the surface

c. at a distance 3/4 r from the centre

d. none of these.

6. Pick up the correct statement from the following :

a. the tangent screw enables to give small movement under conditions of smooth and positive control

b. standing on the tripod is the levelling head or trib arch

c. the levelling screws are used to tilt the instrument so that its rotation axis is truly vertical

d. all the above.

7. Pick up the correct statement from the following :

a. the angle between the true meridian and the magnetic meridian is called magnetic variation

b. one end of the magnetic needle supported at its centre of gravity tends to dip down towards the. nearer magnetic pole of the earth

c. the magnet properly pivoted is balanced by means of riding weight movable along the needle

d. all the above.

8. In precision theodolite traverse if included angles are read twice and the mean reading accepted using both verniers having a least count of 30". Assuming the instrument to be in perfect adjustment, linear measurements correct to 6 mm per 30 metre tape duly corrected for temperature, slope and sag, the angular error of closure not to exceed (where n is the number of traverse legs)

a. 50" n

b. 30" n

c. 60" n.

9. The method of reversal

a. is usually directed to examine whether a certain part is truly parallel or perpendicular to another

b. makes the erroneous relationship between parts evident

c. both (a) and (b)

d. neither (a) nor (b).

10. It is more difficult to obtain good results while measuringhorizontal distance by stepping

a. up-hill

b. down-hill

c. in low undulations

d. in plane areas.

11. The angle of intersection of a curve is the angle between

a. back tangent and forward tangent

b. prolongation of back tangent and forward tangent

c. forward tangent and long chord

d. back tangent and long chord.

12. If the angular measurements of a traverse are more precise than its linear measurements, balancing of the traverse, is done by

a. Bowditch's rule

b. Transit rule

c. Empirical rule

d. all of the above.

13. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

14. Slump test for concrete is carried out, to determine

a. strength

b. durability

c. workability

d. water content.

15. The coefficient of hardness of stones used in road work should be greater than

a. 10

b. 12

c. 15

d. 17.

16. Bitumen completely dissolves in

a. chloroform

b. benzol

c. coaltar

d. All of these.

17. Percentage content of silica in window glass, is

a. 40 to 45

b. 50 to 55

c. 60 to 65

d. 70 to 75.

18. Seasoning of timber is essential to remove

a. knots from timber

b. sap from timber

c. twisted fibre from timber

d. roughness of timber.

19. For concreting tunnel linings, tran-portation of concrete is done by

a. wheel borrows

b. containers

c. pumps

d. belt conveyors.

20. The lower water cement ratio in concrete, introduces

a. greater density and smaller permeability

b. improved frost resistance

c. greater wear resistance and improved bond strength

d. all the above.

21. Di-calcium silicate (C2S)

a. generates less heat of hydration

b. hardens rapidly

c. provides less ultimate strength to cement

d. has less resistance to sulphate attack.

22. The minimum percentage of chemical ingredient of cement is that of

a. magnesium oxide

b. iron oxide

c. alumina

d. lime

23. Pozzolana cement is used with confidence for construction of

a. massive foundations

b. abutments

c. R.C.C. structures

d. all the above.

24. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

25. The reduction in volume of soil due to squeezing out of water from the voids, is termed

a. primary consolidation

b. primary compression

c. primary time effect

d. all the above.

26. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

27. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

28. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

29. Tergazhi's theory of one dimensional consolidation assumes

a. water and soil particles are incompressible

b. deformation of the soil, is entirely due to change in volume

c. Darcey's law for the velocity of flow of water through soil, is perfectly valid

d. all the above.

30. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

31. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

32. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

33. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

34. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

35. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

36. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

37. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

38. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

39. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

40. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

41. Horizontal lines are drawn

a. left to right

b. right to left

c. top to bottom

d. both (a) and (b) above

42. The techniques for free hand sketch to drawing circle is

a. fixing a fixed point and arc

b. fixing the perimeter

c. fixing the radius only

d. none of the above

43. Which of the following regarding scales in drawing is NOT true?

a. the name of the scale (e.g. scale 1:10) should be mentioned below the scale

b. a diagonal scale consists of a primary scale and a venirer

c. comparative scales may be plain or diagonal

d. when an unusual scale is used it is constructed on the drawing sheet

44. The main purpose of as built drawing is

a. maintenance of service work '

b. dimensioning

c. profession

d. to award tender

45. Pick up the ratio which gives us sufficient information by which to judge the financial condition and performance of the firm, from the following:

a. Financial leverage ratio

b. Activity ratio

c. Portability

d. None of these

46. The construction estimate of a project is used by :

a. the owner of the facility

b. the consulting architect/engineer

c. the contractor of the project

d. All of these

47. In a cash flow series :

a. uniform gradient signifies that an income or disbursement changes by the same amount in each interest period.

b. Either an increase or a decrease in the amount of a cash flow is called the gradient.

c. The gradient in the cash flow may be positive or negative.

d. All of these

48. As per the prevalling Public Procurement Acts & Regulations of Nepal, provision of price adjustments is normally to inculpated in civil works contract having project duration ?

a. more than 12 months

b. more than 15 months

c. more than 18 months

d. more than 24 months

49. Engineering shall act in such a manner as to uphold and enhance the honor, and dignity of the engineering profession and shall act with ..... for bribery, fraud, and corruption.

a. a tolerance

b. a penchant

c. zero tolerance

d. a predilection

50. The number of members in dispute resolution committee are

a. 2

b. 3

c. 4

d. 5