***Engineer***

1. The ratio of shear stress and shear strain of an elastic material, is

a. Shear Modulus

b. Young's Modulus

c. Modulus of Elasticity

d. both (a) and (b).

2. A steel rod of sectional area 250 sq. mm connects two parallel walls 5 m apart. The nuts at the ends were tightened when the rod was heated to 100°C. If αsteel = 0.000012/C°, Esteel = 0.2 MN/mm2, the tensile force developed at a temperature of 50°C, is

a. 80 N/mm2

b. 100 N/mm2

c. 120 N/mm2

d. 150 N/mm2

3. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

4. The stress in the wall of a cylinder in a direction normal to its longitudinal axis, due to a force acting along the circumference, is known as

a. longitudinal stress

b. hoop stress

c. circumferential stress

d. ultimate stress.

5. If the stress in each cross-section of a pillar is equal to its working stress, it is called

a. body of equal

b. body of equal section

c. body of equal strength

d. none of these.

6. The ratio of the angles subtended at the eye, by the virtual image and the object, is known as telescope's

a. resolving power

b. brightness

c. field of view

d. magnification.

7. Simpson's rule for calculating areas states that the area enclosed by a curvilinear figure divided into an even number of strips of equal width, is equal to

a. one third the width of a strip, multiplied by the sum of two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

b. one third the width of a strip, multiplied by the sum of two extreme offsets, four times the sum of the remaining odd offsets, and twice the sum of the even offsets

c. one sixth the width of a strip, multiplied by the sum of the two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

d. none of these

8. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

9. If L is in kilometres, the curvature correction is

a. 58.2 L2 mm

b. 64.8 L2 mm

c. 74.8 L2 mm

d. 78.4 L2 mm.

10. For true difference in elevations between two points A and B, the level must be set up

a. at any point between A and B

b. at the exact mid point of A and B

c. near the point A

d. near the point B.

11. The true meridian of a place is the line in which earth's surface is intersected by a plane through

a. east and west points

b. zenith and nadir points

c. north and south geographical poles

d. north and south magnetic poles.

12. To avoid large centering error with very short legs, observations are generally made

a. to chain pins

b. by using optical system for centering the theodolite

c. to a target fixed on theodolite tripod on which theodolite may be fitted easily

d. all the above.

13. Wrought iron is manufactured from pig iron by

a. puddling

b. shingling

c. rolling

d. all the above.

14. Asbestos is

a. an incombustible fire proof material

b. an organic substance

c. a bad insulator for sound and heat

d. all the above.

15. The lime which contains mainly calcium oxide and slacks with water, is

a. quick lime

b. hydraulic lime

c. poor lime

d. none of these.

16. Second class bricks

a. are of dark brown colour

b. produce a metallic sound when struck

c. are well burnt

d. are under burnt.

17. Mastic asphalt is

a. acid resisting material

b. non-corrosive material

c. corrosive material

d. heating-resisting material.

18. Priming consists of

a. one part of white lead, 8 parts of chalk and four parts of twice boiled linseed oil

b. 8 parts of white lead, one part of chalk and four parts of twice boiled linseed oil

c. one part of white lead, 8 parts of chalk and one part of linseed oil

d. none to these.

19. The strength and quality of concrete, depend upon:

a. surface area of aggregates

b. shape of aggregates

c. surface texture of aggregates

d. all the above.

20. The most useless aggregate is one whose surface texture is

a. smooth

b. granular

c. glassy

d. honey combed and porous.

21. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

22. The impurity of mixing water which affects the setting time and strength of concrete, is

a. sodium chlorides

b. sodium carbonates and bicarbonates

c. calcium chorides

d. calcium bicarbonates

23. Wp and Wf are the weights of a cylinder containing partially compacted and fully compacted concrete. If the compaction factor  is 0.95, the workability of concrete is

a. very low

b. low

c. high

d. none of these.

24. Pick up the incorrect definition from the following:

a. The rotation of soil particles into stable state while remoulding, is known as the thiostropy of soil

b. The water content at which a soil changes from the liquid state to solid state, is known liquid limit of the soil

c. The water content at which a soil flows, is known plastic limit of the soil

d. None of these.

25. Pick up the correct statement from the following:

a. If the ratio of depth to width is more than 2, it is deep foundation

b. If the ratio of the length to width is between 1 and 2, it is spread foundation

c. If the length is large as compared to width, it is a strip foundation

d. All the above.

26. The specific gravity of sands, is approximately

a. 2.0

b. 2.2

c. 2.4

d. 2.6

27. The maximum pressure which a soil can carry without shear failure, is called

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

28. If there is no impervious boundary at the bottom of a hydraulic structure, stream lines tend to follow :

a. a straight line

b. a parabola

c. a semi-ellipse

d. a semi-circle.

29. The soil which contains finest grain particles, is

a. coarse sand

b. fine sand

c. silt

d. clay.

30. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

31. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

32. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

33. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

34. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

35. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

36. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

37. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

38. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

39. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

40. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

41. Working edge of drawing board is kept ........... of the user.

a. left

b. right

c. ahead

d. all of the above

42. The purpose of sectional view is to show the

a. surface of the object

b. internal of the object

c. shape of the object

d. none of the above

43. To remove unnecessary lines ....... is used

a. duster

b. chalk

c. sand box

d. eraser

44. Ground plane is

a. horizontal plane

b. vertical plane

c. inclined plane

d. none of the above

45. Pick up the correct statement from the following:

a. The ratios which show profitability in relation to sales and those which show profitability in relation to investment, are called profitability ratios.

b. The ratio of gross profit and net sales, is called profitability in relation to sales ratio.

c. The ratio of net profit after taxes to total assests is known as profitability in relation to investment ratio

d. All of these

46. Pick up the method used for project evaluation and selection in capital budgetting from the following:

a. Internal ratio of return

b. Net present worth

c. Profitability index

d. All the above

47. Which one of the following is not a construction estimate ?

a. Initial feasibility estimate

b. Conceptual preliminary budget

c. Definite estimate

d. None of these

48. Which is the professional body of architectural and engineering firms is Nepal?

a. Society of Nepalese Architects (SONA)

b. Society of Public Health Engineers (SOPHEN)

c. Sociality of Consulting Engineering and Architectural firms

d. None of above

49. NEC publishes the name of registered engineers each year as well as every forth year. The name of foreigner engineers publish

a. each year

b. every forth year

c. both (a) and (b)

d. none of the above

50. For the supply of any goods as per aggrement, who is responsible for the all taxes, duties, licence fees and other such levies imposed by the GoN.

a. CDO

b. employer

c. supplier

d. consultant