***Engineer***

1. The locus of reaction of a two hinged semi-circular arch, is

a. straight line

b. parabola

c. circle

d. hyperbola.

2. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

3. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

4. A long vertical member, subjected to an axial compressive load, is called

a. a column

b. a strut

c. a tie

d. a stanchion

5. The property by which a body returns to its original shape after removal of the force, is called

a. plasticity

b. elasticity

c. ductility

d. malleability.

6. Two concave lenses of 60 cm focal length are cemented on either side of a convex lens of 15 cm focal length. The focal length of the combination is

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

7. Ranging in chain survey means

a. establishing an intermediate point on the line

b. determining the distance between end points

c. determining the offset distance

d. none of these

8. Which one of the following statements is correct ?

a. when the axes of rotation of the graduated circle and the verniers are not coincident, the instrument possesses eccentricity

b. the mean of the readings of the two verniers gives correct reading free from the eccentricity

c. one vernier may be used if the readings of two verniers differ by a constant

d. all the above.

9. For taking offsets with an optical square on the right hand side of the chain line, it is held

a. by right hand upside down

b. by left hand upright

c. by right hand upright

d. by left hand up side down.

10. Perpendicularity of an offset may be judged by eye, if the length of the offset is

a. 5 m

b. 10 m

c. 15 m

d. 20 m.

11. Back bearing of a line is equal to

a. Fore bearing ± 90°

b. Fore bearing ± 180°

c. Fore bearing ± 360°

d. Fore bearing ± 270°

12. Chain surveying is well adopted for

a. small areas in open ground

b. small areas with crowded details

c. large areas with simple details

d. large areas with difficult details.

13. The compound of Portland cement which reacts immediately with water and also sets first is

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

14. Pick up the correct statement from the following:

a. Plastics have generally low melting point

b. The coefficient of thermal expansion of plastics is about three times than that of steel

c. The acoustical boards prepared by impregnating fibre-glass with phenolic resins has absorption coefficient of about 0.67

d. All the above.

15. The cement becomes unsound by the presence of excess

a. sulphur

b. magnesia

c. lime

d. All of these.

16. The most durable varnish is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

17. In   the   method   of   condensation polymerization,

a. low-molecular substances are removed from the high molecular substance

b. the reaction proceeds with an evolution of ammonia

c. the reaction proceeds with an evolution of hydrogen chloride

d. all the above.

18. The most important constituent of varnish, is

a. drier

b. solvent

c. resin

d. all the above.

19. An excess of flaky particles in concrete aggregates

a. increases the quantity of water and sand

b. affects the durability of concrete

c. more than 15% are not desirable

d. all the above.

20. The top diameter, bottom diameter and the height of a slump mould are :

a. 10 cm, 20 cm, 30 cm

b. 10 cm, 30 cm, 20 cm

c. 20 cm, 10 cm, 30 cm

d. 20 cm, 30 cm, 10 cm

21. Pick up the incorrect statement from the following:

a. Cement bags should preferably be piled on wooden planks

b. Cement bags should be placed such that bags of one layer does not touch the bags of the adjacent layer

c. Width and height of the pile should not exceed 3 m and 2.70 m respectively

d. None of these.

22. Pick up the correct statement from the following:

a. Sand obtained from flooded pits, need not be washed before use

b. Sea shore sand contains chlorides which cause effloresance

c. The chloride in sea shore sand and shingle may cause corrosion of reinforcement if the concrete is porous

d. All the above.

23. If the slump of a concrete mix is 60 mm, its workability is

a. low

b. medium

c. high

d. none of these.

24. For a clay slope of height of 10 m, the stability number is 0.05, γ = 2.0 t/m3, C = 2.5 t/m2, the critical height of the slope of the soil, is

a. 4.0 m

b. 12.5 m

c. 25.0 m

d. 15.0 m

25. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

26. The ratio of the volume of water present in a given soil mass to the total volume of its voids, is known

a. porosity

b. void ratio

c. percentage voids

d. degree of saturation.

27. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

28. The ratio of the volume of voids to the total volume of the given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

29. If water content of a soil is 40%, G is 2.70 and void ratio is 1.35, the degree of saturation is

a. 75%

b. 80%

c. 85%

d. 90%

30. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

31. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant.

32. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

33. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

34. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

35. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

36. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

37. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

38. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

39. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

40. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

41. Axonometric projection is a special types of

a. orthographic projection

b. perspective projection

c. isometric projection

d. multi view projective

42. The maps which are on sufficient large scale to enable the individual features shown on the map to be identified on the ground by their shape and position are called :

a. topographical maps

b. geographical maps

c. geological maps

d. site plan

43. In orthographic projection which one have all three views same

a. cube

b. circle

c. sphere

d. both (a) and (c) of above

44. When a pyramid or cone is cut by a cutting plane parallel to its base, the remaining portion thus obtained after removing the top portion is called :..

a. a prism .

b. frustum

c. a cylinder

d. a cone

45. The more critical (or severe) test of the firm's liquidity can be judged by :

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. Pick up the correct statement from the following:

a. Engineering economy is a collection of mathematical techniques which simplify economic comparisons

b. Engineering economy is a decision assistance tool by which one method will be chosen as the most economically one.

c. For understanding the engineering economy, one should be able to classify the basic terminology and fundamental concepts of economy.

d. All of these.

47. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

48. Workers are not allowed to work contiunously more than

a. 3hr

b. 5hr

c. 7hr

d. 4hr

49. Who is the chairperson of Central Labour Advisory Board?

a. Secretory of ministry of labour

b. Specialist of ministry of labour

c. Director General, Department of labour

d. Labour minister/ satate minister

50. Who is the Chairperson of Construction Material Rate fixation Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer