***Engineer***

1. Maximum shear stress theory for the failure of a material at the elastic limit, is known

a. Guest's or Trecas' theory

b. St. Venant's theory

c. Rankine's theory

d. Haig's theory.

2. The ratio of tangential and normal components of a stress on an inclined plane through θ° to the direction of the force, is :

a. cos θ

b. tan θ

c. cos θ

d. sec θ

3. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

4. If the stress in each cross-section of a pillar is equal to its working stress, it is called

a. body of equal

b. body of equal section

c. body of equal strength

d. none of these.

5. Every material obeys the Hooke's law within its

a. elastic limit

b. plastic point

c. limit of proportionality

d. none of these.

6. Permanent adjustments of a level are

a. 2 in number

b. 3 in number

c. 4 in number

d. 6 in number

7. The bearing of AB is 190° and that of CB is 260° 30'. The included angle ABC, is

a. 80° 30'

b. 99° 30'

c. 70° 30'

d. none of these

8. The method generally preferred to for contouring an undulating area, is

a. chain surveying

b. plane table surveying

c. tacheometrical surveying

d. compass surveying.

9. A dumpy level is set up with its eye-piece vertically over a peg A. The height from the top of peg A to the centre of the eye-piece is 1.540 m and the reading on peg B is 0.705 m. The level is then setup over B. The height of the eye-piece above peg B is 1.490 m and a reading on A is 2.195 m. The difference in level between A and B is

a. 3.030 m

b. 0.770 m

c. 0.785 m

d. 1.770 m.

10. Magnetic declination at any place

a. remains constant

b. does not remain constant

c. fluctuates

d. changes abruptly.

11. Whole circle bearing of a line is preferred to a quadrantal bearing merely because

a. bearing is not completely specified by an angle

b. bearing is completely specified by an angle

c. Sign of the correction of magnetic declination is different in different quadrants

d. its trigonometrical values may be extracted from ordinary tables easily.

12. In a closed traverse, sum of south latitudes exceeds the sum of north latitudes and the sum of east departures exceeds the sum of west departures, then, the closing line will lie in

a. north-west quadrant

b. north east quadrant

c. south-east quadrant

d. south-west quadrant.

13. Cement paints usually

a. contain 5% to 10% colour pigments

b. are prepared with white cement

c. contain 5% sodium chloride

d. all the above.

14. In stone masonry, stones (stratified rocks) are so placed that the direction of pressure to the plane of bedding is

a. right angles

b. 45°

c. 60°

d. parallel

15. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

16. Polymerization helps to improve the property of

a. strength

b. rigidity

c. elasticity

d. all of these.

17. Pozzolana (or surkhi) is used in lime

a. to impart hydraulicity

b. to prevent shrinkage

c. to decrease the cost of construction

d. to decrease the setting time.

18. Pick up the correct statement from the following:

a. Blistering may be cured by applying water paint finished with oil paint dried with a little copal varnish

b. Cracked paints may be cured by removing paint and giving a fresh coat of paint

c. Crawling paints may be cured by sand preparing the surface and giving a fresh coat with plenty of turps

d. All the above.

19. The process of hardening the concrete by keeping its surface moist is known

a. wetting

b. curing

c. compacting

d. none of these.

20. To hydrate 500kg of cement full water needed, is

a. 110 kg

b. 120 kg

c. 130 kg

d. 140 kg

21. Le-Chatelier's apparatus is used for testing

a. soundness of cement

b. hardness of cement

c. strength of cement

d. durability of cement.

22. Addition of pozzolana to ordinary port land cement, causes

a. reduction in chemical action with sulphates

b. increase in shrinkage

c. reduction bleeding

d. all the above.

23. For preparing a test-specimen, it is necessary

a. to mix coarse aggregates

b. to mix water to the cement, fine aggregates and coarse aggregates

c. to oil inner surface of the mould and the base plate, before placing concrete

d. all the above.

24. The Mohr's straight theory is based on the following fact :

a. Material fails essentially by shear

b. Ultimate strength of the material is determined by the stress in the plane of slip

c. Failure criterion is independent of the intermediate principal stress

d. All the above.

25. Tergazhi's theory of one dimensional consolidation assumes

a. coefficient of permeability is constant

b. excess pore water drains out only in the vertical direction

c. time lag in consolidation is due entirely to permeability

d. all the above.

26. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

27. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

28. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

29. The ultimate Settlement of a soil is directly proportional to:

a. compressive index

b. void ratio

c. both (a) and (b)

d. none of these.

30. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

31. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

32. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

33. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

34. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

35. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

36. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm.

37. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these

38. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

39. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

40. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

41. Bisecting a line means, divide a line into..... equal part.

a. two

b. three

c. four

d. none of the above

42. The unseen/inner edge of an object is represented by

a. solid line

b. dotted line

c. hatch line

d. none of the above

43. To draw very light line which pencil is used

a. B

b. HB

c. H

d. 2H

44. First angle projection method and third angle projection method are used in:

a. axonometric views

b. isometric views

c. orthographic views

d. none of the above

45. Liquidity ratios are used :

a. to measure a firms ability to meet short-cut obligations.

b. to compare short term obligations to short-term resources available to meet these obligations.

c. to obtain much insight into the present cash solvency of the firm and the firm's ability to remain solvent in the event of adversity.

d. All of these

46. The person desires to pay off the amount in 10 equal annual instalments. The amount of each instalment is :

a. Rs 5638

b. Rs 6638

c. Rs 7738

d. None of these

47. If P is principal amount, i is the rate of interest and n is the number of periods in years, then the interest factor is :

a. (1 + ni)

b. (ni - 1)

c. ni

d. None of these

48. The variation exceeding 15 % of the contract amount is approved by

a. gazetted 2nd class

b. gazetted 1st class

c. deaprtment head

d. none of the above

49. Engineer shall practices:

a. in favour of client

b. in favour of contractor

c. in favour of him/herself

d. impartial andefficient professional relationship with client, contractor & fellow personnels

50. The minimum and maximum no. of members in the user's committee should be

a. 15&17

b. 17&19

c. 9& 15

d. 7&9